| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0121-v1 |
| --- | --- |

| Title: | EE2-related: on GL-CCCM improvement (test 1.12a) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Karam Naser, Franck Galpin, Edouard Francois (InterDigital) Ramin G. Youvalari, Pekka Astola Jani Lainema (Nokia) | Email: | philippe.bordes@interdigital.com, karam.naser@interdigital.com, franck.galpin@interdigital.com, edouard.francois@interdigital.com ramin.ghaznavi-youvalari@nokia.com, pekka.astola@nokia.com, jani.lainema@nokia.com |
| Source: | InterDigital, Nokia |  |  |

_____________________________

# Abstract

In EE2 test 1.12a, a gradient and location based convolutional cross-component model (GL-CCCM) for chroma prediction is explored to improve compression efficiency of ECM, where three types of templates for deriving the CC-models are defined: full template, top-only template, and left-only template. It is proposed to extend the template selection to six. Reported PSNR-Y,U,V BD-rate variations of this proposal compared to ECM7.0 anchor are: -0.06%, -0.99%, -1.06% for AI configuration, -xx%, -xx%, -xx% for RA configuration.

# Introduction

In CCCM mode of ECM-7.0, 7-tap convolutional filter coefficients are derived for each chroma block separately using regression based MSE minimization (i.e., the same solver as CCCM) on reference samples in the PU’s neighborhood (template). Three types of templates for deriving the CC-models are defined: full template, top-only template, and left-only template (LM_CHROMA_IDX, MDLM_T_IDX, MDLM_L_IDX)[1].

Also, similarly to CCLM, there is an option of using a single model or multi-model variant of CCCM. The multi-model variant uses two models, one model derived for samples above the average luma reference value and another model for the rest of the samples (following the spirit of the CCLM design). Single or multi model is signaled jointly with template using {LM_CHROMA_IDX, MDLM_T_IDX, MDLM_L_IDX} or {MMLM_CHROMA_IDX, MMLM_T_IDX, MMLM_L_IDX} respectively.

In EE2 test 1.12a [2], an additional CCCM mode (GL-CCCM) based on a gradient and location based convolutional cross-component model for chroma prediction is explored to improve compression efficiency of ECM [3]. As for regular CCCM, 7-tap convolutional filter coefficients are derived for each chroma block separately using regression based MSE minimization. When it comes to signalling, GL-CCCM is considered a sub-mode of CCCM. That is, the GL-CCCM flag is only signalled when original CCCM flag is true.

# Proposed method

To provide more diversity in the selection of CCCM-models, it is proposed to extend the number of templates to 6 (Figure 1). When one of the 3 extended templates is selected, the model is derived selecting the samples of the template with reconstructed luma value within the range of the reconstructed current CU luma values. If CCCM mode is selected, similarly to GL-CCCM, an EXT-CCCM flag is coded to indicate whether regular or extended template is used.

Figure 1: extended templates used for the derivation of CCCM model.

# Experimental results

The proposed method is tested on top of EE2-1.12a software under the common test conditions [4] in AI and RA configurations.

Table 1: Performance results of EE2-1.12a and proposal in AI.

| AI | EE2-test-1.12a |  |  |  |  | EE2-test-1.12a + proposal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.72% | -0.99% | 102% | 102% | -0.10% | -1.09% | -1.55% | 102% | 101% |
| Class A2 | -0.03% | -0.81% | -0.78% | 103% | 100% | -0.04% | -1.42% | -1.45% | 105% | 102% |
| Class B | -0.03% | -0.67% | -0.78% | 99% | 98% | -0.05% | -1.22% | -1.26% | 101% | 99% |
| Class C | -0.03% | -0.28% | -0.41% | 101% | 102% | -0.05% | -0.55% | -0.64% | 103% | 101% |
| Class E | -0.05% | -0.34% | 0.10% | 102% | 99% | -0.07% | -0.65% | -0.38% | 102% | 98% |
| Overall | -0.04% | -0.56% | -0.59% | 101% | 100% | -0.06% | -0.99% | -1.06% | 102% | 100% |
| Class D | -0.01% | -0.48% | -0.30% | 103% | 103% | -0.01% | -0.70% | -0.41% | 102% | 103% |
| Class F | -0.05% | -0.41% | -0.53% | 103% | 101% | -0.10% | -0.64% | -1.00% | 101% | 100% |
| Class TGM | 0.01% | -0.07% | -0.05% | 101% | 99% | -0.22% | -0.45% | -0.46% | 101% | 99% |

Table 2: Performance results of EE2-1.12a and proposal in RA.

| RA | EE2-test-1.12a |  |  |  |  | EE2-test-1.12a + proposal |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.37% | -0.43% | 99% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | -0.28% | -0.25% | 100% | 97% |  |  |  |  |  |
| Class B | 0.00% | -0.53% | -0.56% | 104% | 102% | -0.02% | -0.72% | -1.05% | 100% | 100% |
| Class C | 0.00% | -0.33% | -0.11% | 101% | 101% | -0.02% | -0.33% | -0.28% | 103% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.40% | -0.35% | 101% | 100% |  |  |  |  |  |
| Class D | 0.01% | -0.10% | -0.25% | 103% | 104% | -0.04% | -0.31% | 0.05% | 102% | 102% |
| Class F | -0.11% | -0.59% | -0.40% | 102% | 102% | -0.12% | -0.56% | -0.54% | 102% | 99% |
| Class TGM | 0.05% | -0.09% | -0.01% | 99% | 96% | -0.04% | -0.25% | -0.07% | 98% | 95% |

# Conclusion

This contribution proposes extension of template number used for CCCM. The results show that the gain of CCCM can be further improved using the proposed method combined with GL-CCCM method. It is suggested to study the proposed method in the next EE round.

# References

- Y. Chang, V. Seregin, M. Karczewicz, “EE2-1.13: On CCCM improvement,” JVET-AB0143, 28th Meeting, Mainz, DE, 20–28 October 2022.

- V. Seregin, J. Chen, G. Li, K. Naser, J. Ström, F. Wang, M. Wiken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AB2024, Oct. 2022.

- R. Youvalari, P. Astola, J. Lainema, “Non-EE2: Gradient and location based convolutional cross-component model (GL-CCCM) for intra prediction,” JVET-AB0119, 28th Meeting, Mainz, DE, 20–28 October 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).