| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11-20 January 2023 | Document: JVET-AC0122-v2 |
| --- | --- |

| Title: | AHG9: Attenuation Map Information SEI for reducing energy consumption of displays |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olivier LE MEUR, Claire-Hélène DEMARTY, Franck AUMONT, Erik REINHARD, Laurent BLONDE, Zoubida AMEUR, Edouard FRANCOIS | Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

The energy consumption of a display represents a large part of the total energy consumption when consuming a video content.

Light emitting display technologies allow for controlling the consumption at the pixel level through a pixel-wise reduction of the luminance.

This contribution proposes the signaling of new metadata for enabling the attenuation of each sample’s values (of each color component) of a decoded video at the receiver side. The reported benefits are to allow an intelligent reduction of each pixel luminance value individually so as to reduce the energy consumption, while maintaining or controlling the quality of experience for the end user.

This contribution proposes to transmit the Attenuation Map through an additional type of auxiliary picture. Its further use at the player or display side is guided by the addition of a new SEI message.

# Introduction

Reducing the energy consumption of electronic devices has become a requirement not only for electronic devices manufacturers but also for limiting, as much as possible, the environmental impact and for contributing to the emergence of a sustainable display industry. In the panel of electronic devices, displays are among the most important sources of energy consumption.

Organic Light Emitting Diode (OLED) displays are finding increasingly widespread use because of numerous advantages compared to transmissive displays such as Thin-Film Transistor Liquid Crystal Displays (TFT-LCDs). Rather than using a uniform or non-uniform backlight, OLED displays, as well as mini LEDS, are composed of directly emissive image pixels which allow controlling the image luminance, and therefore energy consumption, in a pixel wise manner. The energy consumption of backlit displays is largely determined by the intensity of the backlight.

Attenuating the luminance of an image in a pixel-based or region-based manner before displaying it is already done directly in some displays. However, doing luminance adjustment fully at the player or display side can lead to the following issues:

- The modified video will not necessarily respect the content creator intent because, if done at the player or display side, some areas might be impacted by the adjustment independently of the content creator control and the quality of experience could be degraded. On the contrary, if prepared during the content creation, content creators can indicate which areas should not be affected by the processing and/or they can assess that the processing is compliant with the quality of experience.

- Most of the processing needed to reduce the luminance of an image or a video can be done independently of any information that is provided only at the player or display side. Therefore, factorizing such video processing operations at the encoder side will allow to reduce their number, compared to doing them multiple times after the decoding (number of decodings multiplied by number of devices receiving the same content).

Thus, this contribution argues for the signaling of metadata for assisting with attenuating of the decoded video at the receiver side and enabling the use case by proposing a new type of auxiliary pictures and associated Attenuation Map Information (AMI) SEI message. The metadata is compatible with OLED and mini-LED displays, and may drive a backlight scaling mechanism for compatibility with backlit LCD displays.

# Motivations

## General

The main motivation for the proposal of a new auxiliary picture type is to convey pixel-wise information that allows the reduction of the energy consumption associated with a video content at the player or display side.

The proposal is further motivated by the fact that the new auxiliary picture can coexist with other types of auxiliary pictures. For example, it is possible to have an Attenuation Map and a Depth Picture conveyed at the same time.

## Energy reduction through the use of an Attenuation Map (proposed use case)

A correlation exists between the consumed energy and the OLED display intensity. In the case of RGB OLED displays, this correlation is linear [1] as shown in Figure 1.

Figure 1: Linearity between energy consumed and display intensity (luminance) (courtesy of [1] ).

From this relation, it is obvious that reducing the luminance of an image will reduce the energy needed at display time for OLED displays. Other display types such as backlit LCD screens show a similar correlation between brightness and energy consumption (see Figure 2).

Figure 2 : Power vs Backlight level (courtesy of [1])

In the literature [2] and in some already available devices (e.g., smartphones or TVs) a global dimming is already applied depending for example on the ambient light (e.g., Samsung QN90B NeoQLED), which results in a global and uniform reduction of the luminance regardless of the content present in an image.

The purpose of the proposed Attenuation Map is to give pixel-wise information of how the luminance can be attenuated, to specifically adapt to the image content and to the intent of the content creator. Such pixel-wise information will ensure that the quality of experience is better controlled than in the case of a uniform reduction.

Some works already exist in the literature that propose the construction of such an Attenuation Map [3] [2], through learning of content-specific pixel-wise maps that balance luminance reduction (hence energy reduction) with contrast enhancement.

Error! Reference source not fond. Table 1 shows measures of luminance reduction for different reduction modes and for our independent implementation of the method proposed in [3].

Table 1 - Power reduction statistics based on our independent implementation of the method [3]. Power measurements have been made on a SONY OLED KD - 55AF9 display (55'', HD). The power consumption of the device is 60 Watts when a black image is displayed.

|  |  | Power consumption (Watt) and reduction (%) |  |  |
| --- | --- | --- | --- | --- |
| Image | Original | Mode 1 | Mode 2 | Mode 3 |
| Kimono | 76.4 W | 75.8 W (3% decrease) PSNR=44.4dB SSIM=0.99 | 74.4 W (12% decrease) PSNR=30.00dB SSIM=0.98 | 73.3 W (19% decrease) PSNR=25.05dB SSIM=0.97 |
| Ritual Dance | 104.7 W | 104 W (2% decrease) PSNR=38.14dB SSIM=0.99 | 101 W (8% decrease) PSNR=26.49dB SSIM=0.99 | 95 W (22% decrease) PSNR=18.72dB SSIM=0.97 |

For transmissive pixel displays, such as backlit displays, it is also possible to use the Attenuation Map to provide guidance to a backlight scaling algorithm. An example of such a backlight scaling consists in computing the maximum luminance (or luma) values before and after applying the Attenuation Map on the corresponding image.  The ratio between these maxima determines the percentage by which the backlight can be dimmed. The image to which the Attenuation Map is applied, is rescaled to its original value and is then used to set the pixel values of the LED panel. This latter form of backlight scaling allows either a stronger reduction of energy or a smaller reduction in visual quality than conventional backlight scaling methods, due to the pixel-wise nature of the Attenuation Map.

# Proposal: new Attenuation Map Information SEI message

Add new clauses 8.X.1

8.X.1 Attenuation Map Information SEI message syntax

| attenuation_map_info ( payloadSize ) { | Descriptor |
| --- | --- |
| ami_cancel_flag | u(1) |
| if ( !ami_cancel_flag ) { |  |
| ami_display_model | u(4) |
| ami_global_flag | u(1) |
| ami_map_approximation_model | u(4) |
| ami_map_number | u(4) |
| for ( i=0; i<ami_map_number; i++ ) { |  |
| ami_layer_id[ i ] | u(8) |
| ami_ols_number[ i ] | u(4) |
| for ( j=0;j<ami_ols_number[ i ]; j++ ) { |  |
| ami_ols_id[ i ][ j ] | u(8) |
| } |  |
| ami_energy_reduction_rate[ i ] | u(8) |
| if ( !ami_global_flag or ( i = = 0 ) ) { |  |
| ami_attenuation_use_idc[ i ] | u(4) |
| ami_attenuation_comp_idc[ i ] | u(4) |
| ami_preprocessing_flag[ i ] | u(1) |
| if( ami_preprocessing_flag[ i ] ) { |  |
| ami_preprocessing_type_idc[ i ] | u(4) |
| } |  |
| ami_preprocessing_scale_idc[ i ] | u(8) |
| ami_backlight_scaling_idc[i] | u(4) |
| } |  |
| } |  |
| } |  |
| } |  |

# 8.X.2 Attenuation Map Information SEI message semantics

Note: In a preferred mode, the Attenuation Map shall be applied on the sample values of the decoded primary picture(s), i.e., just after the decoding process.

The Attenuation Map Information (AMI) SEI message provides information about the interpretation of the Attenuation Map sample values coded in auxiliary pictures of type AUX_ATTENUATION (in the following, picAMI) and the post-processing intended to be applied to the one or more associated primary pictures of the CVS.

Note: the association of auxiliary pictures to primary pictures are specified in the SDI SEI message (see ISO/IEC 23002-7).

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to 3 for at least one value of i, no picture in the CVS shall be associated with a AMI SEI message.

When an access unit (AU) contains both an SDI SEI message with sdi_aux_id[ i ] equal to 3 for at least one value of i and a AMI SEI message, the SDI SEI message shall precede the AMI SEI message in decoding order.

When an AU contains a picAMI in a layer, with nuh_layer_id equal to nuhLayerIdAMI, that is indicated as an Attenuation Map auxiliary layer by an SDI SEI message, the Attenuation Map sample values of picAMI persist in output order until one or more of the following conditions are true:

–	The next picture, in output order, with nuh_layer_id equal to nuhLayerIdAMI is output.

–	A CLVS containing the auxiliary picture picAMI ends.

–	The bitstream ends.

–	A CLVS of any associated primary layer of the auxiliary picture layer with nuh_layer_id equal to nuhLayerIdAMI ends.

The following semantics apply separately to each nuh_layer_id targetLayerId among the nuh_layer_id values to which the AMI SEI message applies.

ami_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous Attenuation Map Information SEI message in output order. ami_cancel_flag equal to 0 indicates that Attenuation Map Information parameters follow.

ami_map_number specifies the number of auxiliary pictures of type AUX_ATTENUATION in the CVS.

ami_display_model is a bit field mask which indicates the display models on which the Attenuation Map sample values of the auxiliary picture picAMI may be used.

Table 2 - Interpretation of the bits of ami_display_model

| Bit number | Display model |
| --- | --- |
| 0 | Backlit pixel |
| 1 | Emissive pixel |
| 2..3 | Reserved for future types |

For example, ami_display_model=11 means the Attenuation Map Information can be used for both “Backlit” and “Emissive” display models.

ami_map_approximation_model specifies the model used to extrapolate a set of received Attenuation Map sample values(s) from a set of decoded auxiliary picture(s) with individual energy reduction rate(s) to another set of Attenuation Map sample values with a different energy reduction rate.

ami_map_approximation_model equal to 0 specifies that a linear scaling of the Attenuation Map sample values of the provided auxiliary picture given its respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate. In case several auxiliary pictures picAMI are provided, the auxiliary picture with the lowest ami_energy_reduction_rate is used for the linear scaling. This is the preferred type.

ami_map_approximation_model equal to 1 specifies that a bilinear interpolation between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 2 specifies that an interpolation of type Lanczos between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 3 specifies that an interpolation of type bicubic between the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate should be considered to obtain corresponding Attenuation Map sample values for another energy reduction rate.

ami_map_approximation_model equal to 4 specifies that a proprietary user defined process should be used to infer corresponding Attenuation Map sample values for another energy reduction rate from the Attenuation Map sample values of the provided auxiliary picture(s) given their respective ami_energy_reduction_rate.

Table 3 - Interpretation of ami_map_approximation_model

| ami_map_approximation_model | Attenuation Map interpolation process |
| --- | --- |
| 0 | Linear scaling |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | Bicubic interpolation |
| 4 | User defined |
| 5..15 | Reserved for future uses |

ami_global_flag equal to 0 indicates that ami_attenuation_use_idc[ i ], ami_attenuation_comp_idc[ i ], ami_preprocessing_flag[ i ], ami_preprocessing_type_idc[ i ], ami_preprocessing_scale_idc[ i ], ami_backlight_scaling_idc[ i ] for i=0.. ami_map_number, shall be present. ami_global_flag equal to 1 indicates that only ami_attenuation_use_idc[ 0 ], ami_attenuation_comp_idc[ 0 ], ami_preprocessing_flag[ 0 ], ami_preprocessing_type_idc[ 0 ], ami_preprocessing_scale_idc[ 0 ], ami_backlight_scaling_idc[ 0 ] shall be present.

ami_attenuation_use_idc[ i ] specifies the use of the Attenuation Map sample values of the decoded auxiliary picture of index i.

ami_attenuation_use_idc[ i ] equal to 0 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be subtracted from one or more associated primary picture decoded sample(s) before displayed on screen. This is the preferred type.

ami_attenuation_use_idc[ i ] equal to 1 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be multiplied by one or more associated primary picture decoded sample(s) before displayed on screen.

ami_attenuation_use_idc[ i ] equal to 2 specifies that the Attenuation Map sample values of the decoded auxiliary picture should be used according to a proprietary user defined process to modify the one or more associated primary picture decoded sample(s) before displayed on screen.

Table 4 - Interpretation of ami_attenuation_use_idc[ i ]

| ami_attenuation_use_idc[ i ] | Process to apply on associated primary picture decoded samples |
| --- | --- |
| 0 | Subtraction |
| 1 | Multiplication |
| 2 | User defined |
| 3..15 | Reserved for future uses |

ami_attenuation_comp_idc[ i ] specifies on which colour component(s) of the associated primary picture(s) decoded samples the decoded auxiliary picture of type AUX_ATTENUATION of index i should be applied using the process defined by ami_attenuation_use_idc[ i ]. It also specifies how many components the decoded auxiliary picture of index i should contain.

ami_attenuation_comp_idc[ i ] equal to 0 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains only one component and that this component should be applied to the luma component of the associated primary picture(s) decoded samples. This is the preferred type.

ami_attenuation_comp_idc[ i ] equal to 1 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains two components and that the first component should be applied to the luma component of the associated primary picture(s) decoded samples and the second component should be applied to both chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 2 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains only one component and that this component should be applied to the luma component and the chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 3 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains only one component and that this component should be applied to the RGB components (after YUV to RGB conversion) of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 4 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains three components and that these components should be applied respectively to the luma and chroma components of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 5 specifies that the decoded auxiliary picture of type AUX_ATTENUATION of index i contains three components and that these components should be applied respectively to the RGB components (after YUV to RGB conversion) of the associated primary picture(s) decoded samples.

ami_attenuation_comp_idc[ i ] equal to 6 specifies that the mapping between the components of the decoded auxiliary picture of type AUX_ATTENUATION of index i and the components of which to apply the decoded auxiliary picture of type AUX_ATTENUATION of index i corresponds to some proprietary user-defined process.

Table 5 - Interpretation of ami_attenuation_comp_idc[ i ]

| ami_attenuation_comp_idc[ i ] | Mapping between components of the Attenuation Map and primary picture components on which to apply the Attenuation Map |
| --- | --- |
| 0 | One component in the picAMI applied to the luma component (Y=>Y) of the associated primary picture |
| 1 | Two components in the picAMI applied as: - First component to luma component - Second component to the chroma components of the associated primary picture |
| 2 | One component in the picAMI applied to the luma and chroma components of the associated primary picture |
| 3 | One component in the picAMI applied to the three RGB components of the associated primary picture |
| 4 | Three components in the picAMI applied to resp. luma and chroma components of the associated primary picture |
| 5 | Three components in the picAMI applied to resp. the RGB components of the associated primary picture |
| 6 | User defined |
| 7..15 | Reserved for future uses |

ami_backlight_scaling_idc[ i ] specifies the process to compute the scaling factor of the backlight of transmissive pixel displays, derived from the Attenuation Map sample values of the decoded auxiliary picture of index i.

ami_backlight_scaling_idc[ i ] equal to 0 specifies that the scaling to apply to the backlight of the display is computed as the ratio between the maximal values of the associated primary picture decoded samples after and before applying the Attenuation Map sample values of the decoded auxiliary picture of index i. The associated primary picture decoded sample(s) on which the Attenuation Map sample values of the decoded auxiliary picture of index I are applied are further rescaled to their maximal value before the application of the Attenuation Map sample values of the decoded auxiliary picture of index i. This is the preferred type.

ami_backlight_scaling_idc[ i ] equal to 1 specifies that the scaling to apply to the backlight of the display is determined according to a proprietary user defined process derived from the Attenuation Map sample values of the decoded auxiliary picture of index i.

Table 6 - Interpretation of ami_backlight_scaling_idc[ i ]

| ami_backlight_scaling_idc[ i ] | Backlight scaling processing type |
| --- | --- |
| 0 | Scaling by the ratio between maximum values before and after the use of the Attenuation Map |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_preprocessing_flag[ i ] specifies whether some pre-upsampling is to be used on the Attenuation Map sample values of the decoded auxiliary picture of index i. In that case it is supposed that the auxiliary coded picture(s) and the primary coded picture have different sizes.

ami_preprocessing_type_idc[ i ] specifies the recommended type of the interpolation (e.g., bicubic) used to resample the Attenuation Map sample values of the decoded auxiliary picture of index i at the same resolution as the associated decoded picture.

ami_preprocessing_type_idc[ i ] equal to 0 specifies that an interpolation of type bicubic between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_type_idc[ i ] equal to 1 specifies that a bilinear interpolation between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 2 specifies that an interpolation of type Lanczos between the Attenuation Map sample values of the provided auxiliary picture of index i should be considered to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

ami_preprocessing_type_idc[ i ] equal to 3 specifies that a proprietary user defined process should be used to pre-upsample the Attenuation Map sample values of the provided auxiliary picture of index i to obtain the Attenuation Map sample values to apply to the sample values of the decoded picture.

Table 8 - Interpretation of ami_preprocessing_type_idc[ i ]

| ami_preprocessing_type_idc[ i ] | Attenuation Map preprocessing type |
| --- | --- |
| 0 | Bicubic interpolation |
| 1 | Bilinear interpolation |
| 2 | Lanczos interpolation |
| 3 | User defined |
| 4..15 | Reserved for future uses |

ami_preprocessing_scale_idc[ i ] specifies which scaling should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture.

ami_preprocessing_scale_idc[ i ] equal 0 specifies that a scaling of 1255 should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture. This is the preferred type.

ami_preprocessing_scale_idc[ i ] equal 1 specifies that a proprietary user defined scaling should be applied to the Attenuation Map of index i to obtain the Attenuation Map sample values before applying them on the sample values of the decoded picture.

Table 9 - Interpretation of ami_preprocessing_scale_idc[ i ]

| ami_preprocessing_scale_idc[ i ] | Attenuation Map scaling preprocess |
| --- | --- |
| 0 | Scaling of 1255 |
| 1 | User defined |
| 2..15 | Reserved for future uses |

ami_layer_id[ i ] specifies the identifier of the decoded layer for the Attenuation Map of index i.

ami_ols_number[ i ] specifies the number of Output Layer Sets to which the Attenuation Map of index i belongs.

ami_ols_id[ i ][ j ] specifies the identifier of the Output Layer Set of index j for the Attenuation Map of index i. This identifier shall be used to select the OLS to output both the primary decoded picture and the Attenuation Map of index i.

ami_energy_reduction_rate[ i ] indicates the expected energy saving rate when the video is displayed after applying the Attenuation Map sample values of the decoded auxiliary picture of index i on the sample values of the decoded picture.

8.19.2 Scalability dimension information SEI message semantics

Table 10 - Mapping of sdi_aux_id[ i ] to the type of auxiliary pictures

| sdi_aux_id[ i ] | Name | Type of auxiliary pictures |
| --- | --- | --- |
| 1 | AUX_ALPHA | Alpha plane |
| 2 | AUX_DEPTH | Depth picture |
| 3 | AUX_ATTENUATION | Attenuation picture |
| 4..127 |  | Reserved |
| 128..255 |  | Unspecified |

# Conclusion

In this contribution, a new type of auxiliary picture is proposed, named Attenuation Picture. In addition, a new SEI message aimed at properly interpreting the Attenuation Pictures is proposed.

We recommend adopting the proposed metadata in the upcoming amendment of the VSEI specification.

# References

| [1] | B. Anand, K. Thirugnanam, J. Sebastian, P. G. Kannan, A. L. Ananda, M. C. Chan and R. K. Balan, "Adaptive display power management for mobile games," in Proceedings of the 9th international conference on Mobile systems, applications, and services, 2011. |
| --- | --- |
| [2] | Y.-G. Shin, S. Park, Y.-J. Yeo, M.-J. Yoo and S.-J. Ko, "Unsupervised Deep Power Saving and Contrast Enhancement for OLED Displays," arXiv preprint arXiv:1905.05916, 2019. |
| [3] | K. A. Nugroho and S.-J. Ruan, "R-ACE Network for OLED Image Power Saving," in IEEE 4th Global Conference on Life Sciences and Technologies, 2022. |
| [4] | J.-L. Yin, B.-H. Chen, Y.-T. Peng and C.-C. Tsai, "Deep battery saver: End-to-end learning for power constrained contrast enhancement," IEEE Transactions On Multimedia, vol. 23, pp. 1049-1059, 2020. |

# Acknowledgements

This work has been achieved in the context of the project 3EMS-2 funded by the “Region Bretagne” and “Images et Réseaux”.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Table of figures

Figure 1: Linearity between energy consumed and display intensity (luminance) (courtesy of [1] )	3