| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0123-v1 |
| --- | --- |

| Title: | MTT maximum depth correction for class B sequences at QP 22 in ECM. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno | Tel: Email: | guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The purpose of this contribution is to correct an error introduced by one ECM-6.0 adoption related to the maximum tree depth of Multiple Type Tree (MTT) for Class A when QP=22 in Random Access (RA). Unfortunately, the software change related to this adoption also unexpectedly affected the Class B sequences by accidentally increasing (from 2 to 3) the maximum Multiple Type Tree (MTT) depth when the QP=22 in RA. The intention of this contribution is to correct this error and to return to the initial setting for Class B sequences at QP=22 for RA to save encoder runtime while maintaining the intended adoption for the ECM 6.0.

# Introduction

The intent of this contribution is to correct an error introduced by JVET-AA0098 contribution that was adopted for ECM-6.0 related to the common testing conditions. The main aspect of this contribution was to modify the maximum depth of the MTT splitting for all Class A sequences when the QP value is equal to 22 for RA. The main purpose was to save execution time for the ECM-6.0 encoder. This contribution reduced the encoding worst case runtime by changing the encoder configuration for the lowest QP in Class A, random access. The idea was to change the value of the following parameter MaxMTTHierarchyDepthByTid from 333322 to 333222 for QP=22.

Unfortunately, the software change related to this adoption also unexpectedly affected the Class B sequences by accidentally increasing (from 2 to 3) the maximum MTT depth for some temporal ID when the QP=22 for RA.

The consequence of this modification is a much longer encoder execution runtime for the ECM-6.0 (and ECM-7.0) for a very negligible coding gain for Class B sequences at QP=22.

This contribution proposes to correct this mistake and to return to the initial setting for Class B sequences at QP=22 to save encoder runtime while keeping the original intent of JVET-AA0098 contribution regarding the restriction of the maximum tree depth of MTT for Class A when QP is equal to 22.

# Results

The proposed modification is implemented on top of the ECM-7.0 [1] and tested according to the common test condition (CTC) for ECM-7.0 [2].

Our proposed modification impacts only Class B for RA configuration. The BDR impact for this class is 0.07% for 93% encoding time reduction. In average for all classes the BDR impact is 0.02% for 97% encoding time.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.07% | 0.10% | 0.11% | 93% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.04% | 97% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 96% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

The following table shows the percentage of encoding difference for Random Access for the Class B. In average the encoding time is around 74% for QP 22.

|  |  | QPISlice | EncR |
| --- | --- | --- | --- |
| Class B | MarketPlace | 22 | 76% |
| 1080p |  | 27 | 100% |
|  |  | 32 | 100% |
|  |  | 37 | 100% |
|  | RitualDance | 22 | 71% |
|  |  | 27 | 100% |
|  |  | 32 | 100% |
|  |  | 37 | 100% |
|  | Cactus | 22 | 75% |
|  |  | 27 | 100% |
|  |  | 32 | 100% |
|  |  | 37 | 100% |
|  | BasketballDrive | 22 | 72% |
|  |  | 27 | 100% |
|  |  | 32 | 100% |
|  |  | 37 | 100% |
|  | BQTerrace | 22 | 74% |
|  |  | 27 | 100% |
|  |  | 32 | 100% |
|  |  | 37 | 100% |

# Conclusion

In this contribution, a correction regarding the MTT maximum depth for sequences of Class B at QP=22 is proposed for RA. The results above regarding this correction show a very similar coding performance while reducing considerably (25%) the encoding time.

We therefore suggest adopting this correction in the ECM-8.0.

# References

- ECM-7.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Canon Research Centre France does not have any current or pending patent rights relating to the technology described in this contribution.