| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0124-v3 |
| --- | --- |

| Title: | Non-EE2: Bi-prediction with block-level only out-of-bound management |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec M. Blestel P. Andrivon M. Radosavljević | Tel: Email: | fabricel@xiaomi.com |
| Source: | Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes to remove all sample-adaptive processes in the enhanced bi-prediction method of JVET-Z0136 used in ECM-7.0. Instead, a block-level management of out-of-bound (OOB) samples is proposed. No more sample-by-sample OOB criteria evaluation is used, and no more sample-adaptive bi-prediction is applied.

The proposed method reportedly leads to the following BD-rate performances over ECM-7.0 anchors.

RA: 0.00% / -0.01% / -0.08% (Y/Cb/Cr). Runtime: 101% (Enc), 102% (Dec)

LDB : -0.01% / 0.19% / 0.09% (Y/Cb/Cr). Runtime: 97% (Enc), 98% (Dec)

# Introduction

In ECM-7.0, bi-prediction is performed in a way that avoids relying on reference samples out of a reference picture bounds (OOB), if possible. To do so, in the case of a bi-predicted block with an OOB reference block in one of the two reference pictures, the OOB prediction samples are not used. The concerned part of the block is rather uni-predicted based on non-OOB prediction samples, if available in the other reference picture. This is illustrated on the following Figure 1.

Figure 1: enhanced bi-prediction process of ECM (JVET-Z0136)

The criteria to determine a prediction sample position as OOB in JVET-Z0136 [1] is the following one. Let Pos_xi,j and Pos_yi,j denote the position of one prediction sample in one current block, Mv_xi,jLx and Mv_yi,jLx (x = 0,1) denote the MV of the current block; PosLeftBdry, PosRightBdry, PosTopBdry and PosBottomBdry are the positions of four boundaries of the picture. A prediction sample is regarded as OOB when at least one of the following conditions is satisfied:

(Pos_xi,j+Mv_xi,jLx)>(PosRightBdry+half_pixel),

(Pos_xi,j+Mv_xi,jLx)<(PosLeftBdry- half_pixel),

(Pos_yi,j+Mv_yi,jLx)>(PosBottomBdry+ half_pixel),

(Pos_yi,j+Mv_yi,jLx)<(PosTopBdry- half_pixel)

Additionally, in the ECM-7.0 implementation, a block-level OOB condition is computed first. Depending on that condition, a block being bi-predicted is considered OOB or not. If the block is not OOB, then the usual bi-prediction process occurs.

Otherwise, an OOB/non-OOB check is performed for each sample position in the block, i.e. above conditions are evaluated for all samples. Then, in case of OOB prediction samples, the bi-prediction is performed as follows for each sample position (i,j) in the block to predict:

If prediction sample Pi,jL0 is OOB and prediction sample Pi,jL1 is non-OOB

Pi,jfinal=Pi,jL1

else if Pi,jL0 is non-OOB and Pi,jL1 is OOB

Pi,jfinal=Pi,jL0

else

Pi,jfinal=(Pi,jL0+Pi,jL1+1)≫1

To perform this sample-wise adaptive prediction, some binary masks are computed and stored for each block that is detected as OOB. Also, during bi-prediction, some conditional process is performed for each sample position according to the computed binary mask, to compute the final prediction sample.

In addition, for sub-block-based motion compensation, intermediate sub-masks are computed for each sub-block, based on the whole block level binary OOB mask, in order to determine the OOB criteria at sub-block level.

The goal of this contribution is to remove all sample-based conditional processes for bi-prediction and remove the use of binary masks to identify OOB and non-OOB prediction samples, and to replace them by a block-level process.

# Proposed fully block-based bi-prediction of OOB blocks

## Overall process

The proposed method consists in the following steps.

- Determine the block-level OOB conditions as in ECM-7.0, for reference picture lists L0 and L1.

- If block PL0 is OOB, then:

  - Compute the coordinates of the non-OOB zone ZL0 in prediction block PL0 which delimits the set of non-OOB prediction samples in PL0. This zone is rectangular and consists in samples of block PL0 that lie within the boundaries of corresponding reference picture (see Figure 2 example).

  - Compute intermediate block PL0' of same size as PL0, which consists in PL0 inside zone ZL0 and values of block PL1 outside ZL0 (see Figure 3).

- Otherwise (PL0 is not OOB)

  - Set intermediate block PL0' equal to PL0.

- Similarly, compute intermediate block PL1'

- Compute the final prediction block as the average of PL0' and PL1'.

Figure 2: illustration of the non-OOB zone ZL0 inside the prediction block PL0.

Figure 3: illustration of proposed method for block-based OOB management during bi-prediction (case where block PL0 is OOB and block PL1 is non-OOB)

Note step 5 in above process applies exactly same averaging as in the usual case where none of the two blocks PL0 and PL1 are OOB. Moreover, the intermediate blocks PL0' and PL1' are easy to compute and can be produced in place, without necessarily allocating additional memory buffer.

In the proposed bi-prediction averaging mechanism, no more sample-level switching between different computations of predicted values is needed. Only a whole-block-level process is employed.

The proposed block-level OOB management process is applied to both whole-block-based and sub-block-based motion compensation cases, and to the BCW case. No more binary mask-based sample-level process is achieved.

It is thus asserted that the proposed method simplifies the bi-prediction design of ECM.

Finally, the following Figure 4 shows a more general illustration of proposed method, when both prediction blocks PL0 and PL1 are OOB. This illustration demonstrates that:

- for sample locations where both uni-predictions PL0 and PL1 are OOB, the resulting final prediction is equal to the averaging of PL0 and PL1. In this case, both predictors come from padded areas of Figure 1.

- for sample locations where one uni-prediction is OOB and not the other one, the resulting final prediction is equal to the non-OOB uni-prediction

- for sample locations where both uni-predictions are non-OOB, the resulting final prediction is equal to the averaging of PL0 and PL1.

Therefore, similar bi-prediction process as in ECM is reproduced with the proposed simplified mechanism.

Figure 4: illustration of proposed method for a whole block-based combination of uni-predicted blocks, in case where block-level OOB condition is true for both L0 and L1

## Details on non-OOB rectangular zone (ZL0, ZL1) computation

The following Table 1 depicts the process used to compute the position and size of the non-OOB zone inside a uni-predicted block PL0 or PL1.

More precisely, the operations emphasized in bold in Table 1 are performed at block-level. Other operations to compute (xMax,xMin,yMax,yMin) can be computed only once at picture level.

It can be seen that only simple coordinate operations are involved and are performed only once for each uni-predicted block, after that block is detected as OOB. This computation is thus claimed to be of low complexity.

Table 1: computation of non-OOB zone coordinates in a uni-predicted block

// inputs: picture size (picWidth, picHeight) // block position (blockX,blockY) and size (blockW,blockH) in MV accuracy steps // motion vector (mvx,mvy) // output: // non-OOB zone full-pel position (xZ,yZ) and size wZ,hZ relative to current block top-left corner xMax = (PosRightBdry+half_pixel) // right upper bound for OOB criteria xMin = (PosLeftBdry- half_pixel) // left upper bound for OOB criteria yMax = (PosBottomBdry+ half_pixel) // right upper bound for OOB criteria yMin = (PosTopBdry- half_pixel) // left upper bound for OOB criteria xleft = blockX+mvx ytop = blockY+mvy xright = xleft+(blockW-1) ybottom = ytop+(blockH -1) xZ = ( max(xMin,xleft )-xleft) ) / D // D = full pel distance in MV steps yZ = ( max(yMin,ytop )-ytop )/D wZ = ( min(xMax,xright )-xleft )/D - xZ+1 hZ = ( min(yMax,ybottom )-ytop )/D - yZ+1

# Experimental results

The ECM software modified as proposed in this contribution has been run in JVET EE2 CTCs [1]. The following Table 1 shows the results obtained with the proposed block-level OOB process. It can be seen the proposed method leads to same coding efficiency as ECM-7.0, and runtime is also unchanged.

Table 2: simulation results obtained with bi-prediction method using proposed block-based OOB process

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | 0.00% | 103% | 103% |
| Class A2 | 0.02% | 0.04% | -0.06% | 103% | 101% |
| Class B | -0.01% | 0.01% | -0.15% | 98% | 102% |
| Class C | 0.00% | -0.04% | -0.08% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.08% | 101% | 102% |
| Class D | 0.01% | -0.22% | -0.14% | 103% | 101% |
| Class F | -0.01% | 0.00% | 0.03% | 100% | 102% |
| Class TGM | -0.01% | 0.00% | -0.02% | 99% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | -0.16% | 98% | 104% |
| Class C | 0.01% | -0.41% | 0.05% | 94% | 92% |
| Class E | -0.08% | -0.13% | 0.56% | 100% | 98% |
| Overall | -0.01% | -0.19% | 0.09% | 97% | 98% |
| Class D | 0.05% | 0.01% | -0.02% | 98% | 95% |
| Class F | 0.06% | 0.16% | -0.10% | 100% | 98% |
| Class TGM | 0.05% | 0.09% | 0.11% | 94% | 96% |

# Conclusion

This contribution proposes to remove all sample-adaptive tests and bi-prediction processes in the enhanced bi-prediction method of JVET-Z0136 used in ECM-7.0.

Instead, a block-level management of out-of-bound (OOB) samples is proposed. It consists, in case of an-OOB block, in identifying the rectangular non-OOB zone in the block and computing an intermediate prediction block according to the non-OOB zone. The usual averaging is then applied to the intermediate computed block.

The proposed method reportedly leads to the following BD-rate performance over ECM-7.0 anchors.

RA: 0.00% / -0.01% / -0.08% (Y/Cb/Cr). Runtime: 101% (Enc), 102% (Dec)

LDB : -0.01% / 0.19% / 0.09% (Y/Cb/Cr). Runtime: 97% (Enc), 98% (Dec)

Given the unchanged performance of ECM-7.0 and the simplification brought by the proposed method, it is proposed to adopt it in next version of ECM.

# Cited references

[1] Yi-Wen Chen, Xiaoyu Xiu, C.-W. Kuo, Ning Yan, Xianglin Wang (Kwai), Han Huang, Yao-Jen Chang, Chun-Chi Chen, Marta Karczewicz, Vadim Seregin, Yan Zhang, Zhi Zhang (Qualcomm), “EE2: Enhanced bi-directional motion compensation (test 2.2)”, JVET-Z0136, April 2022.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Xiaomi may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).