| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC125-v3 |
| --- | --- |

| Title: | EE2-3.8: Combination of chroma IBC tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yingbin Wang, Xiaozhong Xu, Xin Zhao, Roman Chernyak, Shan Liu (Tencent) Junyan Huo, Xue Hao, Yanzhuo Ma, Fuzheng Yang (Xidian Univ) Jing Ren, Ming Li (OPPO) | Email: | yingbinwang@tencent.com jyhuo@mail.xidian.edu.cn renjing1@oppo.com |
| Source: | Tencent, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution reports test results of EE2-3.8, two combination tests of EE2-3.1 and EE2-3.7 are included. In EE2-3.8a, a BV is derived for each 2x2 sub-block according to the method in EE2-3.7. If no valid BV can be found, the default BV which is derived from the pre-defined luma position in EE2-3.1 will be used. In EE2-3.8b, both the BV derivation and refinement method of EE2-3.1 are used, but this mode is signalled as IBC mode rather than intra mode. The overall performance impact of the proposed method on top of ECM-7.0 for SCC content is reported as below.

- EE2-3.8a:

For Class-F test sequence:

- AI:	 -0.30%/-0.17%/-0.43% with 99% EncT and 100% DecT

- RA:	 -0.22%/-0.23%/-0.40% with 99% EncT and 100% DecT

For Class-TGM test sequence:

- AI:	 -1.85%/-2.40%/-2.91% with 99% EncT and 99% DecT

- RA:	 -0.57%/-0.84%/-0.84% with 99% EncT and 99% DecT

- EE2-3.8b:

For Class-F test sequence:

- AI:	 -0.28%/-0.24%/-0.48% with 100% EncT and 99% DecT

- RA:	 -0.19%/-0.22%/-0.45% with 100% EncT and 99% DecT

For Class-TGM test sequence:

- AI:	 -2.14%/-2.59%/-3.22% with 100% EncT and 97% DecT

- RA:	 -0.64%/-0.84%/-0.97% with 100% EncT and 99% DecT

Introduction

In EE2-3.1, a chroma intra prediction mode called direct block vector (DBV) mode is proposed for screen content [1]. This mode is only enabled for chroma dual-tree partition. As shown in Table 1, a CU level flag is signalled to indicate whether the DBV mode is applied.

Table 1 The binarization process for intra_chroma_pred_mode in the proposed method

| intra_chroma_pred_mode | bin string | chroma intra mode |
| --- | --- | --- |
| 0 | 11100 | list[0] |
| 1 | 11101 | list[1] |
| 2 | 11110 | list[2] |
| 3 | 11111 | list[3] |
| 4 | 110 | DIMD chroma |
| 5 | 10 | DM |
| 6 | 0 | DBV |

If one of the luma blocks (the following five locations in Figure 1) is coded with IBC mode or IntraTMP mode, its block vector bvL is used and scaled to derive chroma block vector bvC. The scaling factor depends on the chroma format sampling structure. Then a block copying prediction is performed according to the derived chroma block vector bvC.

Figure 1 the five locations in reconstructed luma samples

The following refinements are also included in EE2-3.1:

1) The DBV flag is conditionally signalled according to the prediction modes of candidates.

2) Template matching is applied to refine chroma BV.

In EE2-3.7, the chroma IBC mode in VTM5.0 is studied on top of ECM7.0. The chroma IBC mode does not need to signal BV, but directly derives the block vector from its luma position. For each 2x2 chroma sub-block, a block vector (BV) is derived and used for motion compensation. If one of the subblocks cannot find a valid block vector, the chroma IBC mode will be disabled by the encoder. 1 additional bit is signalled to indicate whether the chroma IBC mode is used or not.

Besides, the following modifications are made to harmonized with existing ECM methods:

1) Luma blocks in IntraTMP mode can be used to derive the chroma BV.

2) The flip-aware BV adjustment approach in [2] is applied to refine the chroma block vector, if the co-located luma position is inside a RRIBC coded block.

Proposed Methods

The following two combination tests of EE2-3.1 and EE2-3.7 are included in this contribution.

In EE2-3.8a, a BV is derived for each 2x2 sub-block according to the method in EE2-3.7. If no valid BV can be found, the default BV which is derived from the pre-defined luma position in EE2-3.1 will be used.

In EE2-3.8b, both the BV derivation and refinement methods of EE2-3.1 are used, but this mode is signalled as IBC mode rather than intra mode.

Simulation Results

The proposed method was implemented based on ECM-7.0 [3] under common test conditions [4]. The simulation results of EE2-3.8a and EE2-3.8b are provided in Table 2 and Table 3, respectively. In v3, the simulation results of EE2-3.8b are updated since a bugfix is introduced. Specially, the chroma IBC flag is signalled using a separated context rather than reusing the context of the luma IBC flag. For information, simulation results of EE2-3.8 using only the center position for BV derivation are provided in Table 4.

Table 2. Simulation results of EE2-3.8a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.30% | -0.17% | -0.43% | 99% | 100% |
| Class TGM | -1.85% | -2.40% | -2.91% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.22% | -0.23% | -0.40% | 99% | 100% |
| Class TGM | -0.57% | -0.84% | -0.84% | 99% | 99% |

Table 3. Simulation results of EE2-3.8b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | -0.37% | -0.61% | 99% | 98% |
| Class TGM | -2.11% | -2.58% | -3.19% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.26% | -0.51% | -0.49% | 98% | 98% |
| Class TGM | -0.67% | -0.85% | -0.93% | 99% | 97% |

Table 4. Simulation results of EE2-3.8b using only the center position for BV derivation.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.30% | -0.32% | -0.55% | 98% | 98% |
| Class TGM | -2.07% | -2.50% | -3.08% | 99% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.29% | -0.31% | -0.49% | 99% | 98% |
| Class TGM | -0.55% | -0.67% | -0.77% | 98% | 98% |

Conclusions

This contribution reports the test results of EE2-3.8a and EE2-3.8b. The experimental result shows that the proposed method achieves coding performance for screen contents under AI and RA configuration with negligible complexity impact. It is recommended to include one of the proposed methods into the next version of ECM.

References

- Junyan Huo, et. al., “EE2-3.1: Direct block vector mode for chroma prediction,” JVET-AC0071, January 2023.

- Zhipin Deng, Kai Zhang, Li Zhang, “EE2-3.2: Reconstruction-Reordered IBC for screen content coding,” JVET-AA0070, July 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).