| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0127-v4 |
| --- | --- |

| Title: | AHG9: Combination of picture rate upsampling with other NNPF purposes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Jizheng Xu1, Yue Li1, Li Zhang1, Kai Zhang1, Junru Li2, Chaoyi Lin3 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 3Bafangcheng 9th building F14, Yuhang District, 311100, Hangzhou City, Zhejiang Province, China | Email: | yekui.wang@bytedance.com xujizheng@bytedance.com yue.li@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com lijunru@bytedance.com linchaoyi.cy@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, neural-network post-filter (NNPF) purposes with chroma upsampling only, resolution upsampling only, combination of chroma upsampling and resolution upsampling, and picture rate upsampling only are specified.

However, picture rate upsampling in combination with other types of upsampling can also be used in video applications. This contribution proposes to allow such combination of NNPF purposes.

In JVET-AC0127-v2, the following two alternative approaches for adding the support of a combination of picture rate upsampling with visual quality improvement were included:

- Add a new purpose.

- Add controlling on which input pictures the NNPF generates corresponding output pictures.

The authors prefer option 2, because with that, it is still possible to use the values of the purposes to indicate whether certain upsampling operations are performed, such that no upsampling of any type meaning just visual quality improvement.

An editorial change to the sentence introducing Equation 92 was also included in JVET-AC0127-v2.

In JVET-AC0127-v3, an attachment was included for providing text changes for the following five items collectively:

- JVET-0127-v2 Option 2 (Item 20 in JVET-AC0324-v2), but with the representation of nnpfc_purpose replaced by JVET-AC0152 Method 1 (bit-mask based representation).

- JVET-AC0152 Method 1 (Item 28 in JVET-AC0324-v2): Bit-mask based representation of nnpfc_purpose. This item was noted in JVET-AC0324-v2 to be further discussed together with Item 20 (JVET-AC0127).

- JVET-AC0153 (Item 30 in JVET-AC0324-v2): Enabling indicating bit depth upsampling by nnpfc_purpose. This item was noted in JVET-AC0324-v2 to be further discussed together with Item 20 (JVET-AC0127) for signalling details to be worked out regarding combinations, although the proposed ability was already recommended for adoption by the BoG on neural-network based post-filtering.

- JVET-AC0174 (Item 32 in JVET-AC0324-v2): Enabling input multiple pictures for NNPF purposes without picture rate upsampling. This was already recommended for adoption by the BoG on neural-network based post-filtering. Included because the interaction with JVET-AC0127.

- JVET decision on JVET-AC0061 proposal 3 (Item 6 in in JVET-AC0324-v2): To have separate bit depth syntax elements for both input and output tensors. Included because the heavy interaction with JVET-AC0153 on enabling indicating bit depth upsampling by nnpfc_purpose.

In JVET-AC0127-v4, in addition to the text as in JVET-AC0127-v3, one more attachment was included for providing text changes for the same five items collectively as in JVET-AC0127-v3, with the following updates to the bit-mask based representation of nnpfc_purpose as suggested by Miska M. Hannuksela during side activity discusions:

- Spend 1 bit for indicating visual quality improvement.

- nnpfc_purpose = 0 indicates "may be used as determined by the application".

# Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

## Option 1

In this option, in addition to the changes in JVET-AC0127-v1, the combination of picture rate upsampling with visual quality improvement was added, with the use of a new purpose indicated by nnpfc_purpose equal to 9.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ¦ ¦ nnpfc_purpose = = 6 ¦ ¦ nnpfc_purpose = = 8 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ¦ ¦ nnpfc_purpose = = 7 ¦ ¦ nnpfc_purpose = = 8 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ¦ ¦ nnpfc_purpose = = 6 ¦ ¦ nnpfc_purpose = = 7 ¦ ¦ nnpfc_purpose = = 8 ¦ ¦ nnpfc_purpose = = 9 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 9 5, inclusive, in bitstreams conforming to this edition of this document. Values of 10 5 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 10 5 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement, without any of chroma upsampling, resolution upsampling, and picture rate upsampling |
| 2 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 3 | Resolution upsampling (increasing Increasing the width or height of the cropped decoded output picture) without chroma upsampling and without picture rate upsampling changing the chroma format |
| 4 | Resolution upsampling and chroma upsampling, without picture rate upsampling Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling, without chroma upsampling and without resolution upsampling |
| 6 | Picture rate upsampling and chroma upsampling, without resolution upsampling |
| 7 | Picture rate upsampling and resolution upsampling, without chroma upsampling |
| 8 | Picture rate upsampling, chroma upsampling, and resolution upsampling |
| 9 | Picture rate upsampling and visual quality improvement, without chroma upsampling and without resolution upsampling |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2, 4, 6, or 8 or 4.

nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC. When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_num_input_pics_minus2 plus 2 specifies the number of decoded output pictures used as input for the post-processing filter.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter.

The variables numInputPics, specifying the number of pictures used as input for the post-processing filter, and numOutputPics, specifying the total number of pictures resulting from the post-processing filter, are derived as follows:

if( nnpfc_purpose  = =  5  | |  nnpfc_purpose  = =  6  | |  nnpfc_purpose  = =  7  | |  nnpfc_purpose  = =  8  | |
		nnpfc_purpose  = =  9 ) {
	numInputPics = nnpfc_num_input_pics_minus2 + 2
	numOutputPics = ( nnpfc_purpose  = =  5 ) ? 0: numInputPics
	for( i = 0, numOutputPics = 0; i  <=  numInputPics − 2; i++ ) 	(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else {
	numInputPics = 1
	numOutputPics = 1
}

...

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose is equal to 2, 4, 6, or 8 or 4, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics numInputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(83)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

...

The following example process may be used, to filter the cropped decoded output picture patch-wise with the post-processing filter PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contains Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc.

if( nnpfc_inp_order_idc  = =  0 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight)	(92)
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

...

## Option 2

In this option, in addition to the changes in JVET-AC0127-v1, the combination of picture rate upsampling with visual quality improvement is added, by adding controlling on which input pictures the NNPF generates corresponding output pictures.

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ¦ ¦ nnpfc_purpose = = 6 ¦ ¦ nnpfc_purpose = = 8 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ¦ ¦ nnpfc_purpose = = 7 ¦ ¦ nnpfc_purpose = = 8 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ¦ ¦ nnpfc_purpose = = 6 ¦ ¦ nnpfc_purpose = = 7 ¦ ¦ nnpfc_purpose = = 8 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| if( numInputPics > 1 ) |  |
| for( i = 0; i < numInputPics; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| ... |  |
| } |  |

### 8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 8 5, inclusive, in bitstreams conforming to this edition of this document. Values of 9 6 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 9 6 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement, without any of chroma upsampling, resolution upsampling, and picture rate upsampling |
| 2 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 3 | Resolution upsampling (increasing Increasing the width or height of the cropped decoded output picture) without chroma upsampling and without picture rate upsampling changing the chroma format |
| 4 | Resolution upsampling and chroma upsampling, without picture rate upsampling Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling, without chroma upsampling and without resolution upsampling |
| 6 | Picture rate upsampling and chroma upsampling, without resolution upsampling |
| 7 | Picture rate upsampling and resolution upsampling, without chroma upsampling |
| 8 | Picture rate upsampling, chroma upsampling, and resolution upsampling |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2, 4, 6, or 8 or 4.

nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC. When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_num_input_pics_minus2 plus 2 specifies the number of decoded output pictures used as input for the post-processing filter.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter.

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture.

The variables numInputPics, specifying the number of pictures used as input for the post-processing filter, and numOutputPics, specifying the total number of pictures resulting from the post-processing filter, are derived as follows:

if( nnpfc_purpose  = =  5  | |  nnpfc_purpose  = =  6  | |  nnpfc_purpose  = =  7  | |  nnpfc_purpose  = =  8 ) {
	numInputPics = nnpfc_num_input_pics_minus2 + 2
	for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
		if( nnpfc_input_pic_output_flag[ i ] )
			numOutputPics++
	for( i = 0, numOutputPics = 0; i  <=  numInputPics − 2; i++ ) 	(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else {
	numInputPics = 1
	numOutputPics = 1
}

...

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose is equal to 2, 4, 6, or 8 or 4, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics numInputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(83)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

...

The following example process may be used, to filter the cropped decoded output picture patch-wise with the post-processing filter PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contains Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc.

if( nnpfc_inp_order_idc  = =  0 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight)	(92)
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).