| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 28th Meeting, Mainz, DE, 20–28 October 2022 | Document: JVET-AB2006-v1 |
| --- | --- |

| Title: | Additional SEI messages for VSEI (Draft 3) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Sean McCarthy Takeshi Chujoh Miska Hannuksela Gary Sullivan Ye-Kui Wang | Email: | sean.mccarthy@dolby.com chujoh.takeshi@sharp.co.jp miska.hannuksela@nokia.com garysull@microsoft.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the versatile supplemental enhancement information messages for coded video bitstreams (VSEI) standard (Rec. ITU-T H.274 | ISO/IEC 23002-7), to specify additional SEI messages, including the shutter interval information SEI message, neural-network post-filter characteristics SEI message, neural-network post-filter activation SEI message, phase indication SEI message, and post-filter hint SEI message.

Changes to the specification text:

# Clause 2

Add the following references:

ISO/IEC 15938-17, Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

IETF Standard 66, Uniform Resource Identifiers (URI): Generic Syntax, http://tools.ietf.org/html/rfc3986

IETF RFC 4151, The 'tag' URI Scheme, October 2005, http://tools.ietf.org/html/rfc4151

# Clause 4

Add the following abbreviated terms:

NNPFA	neural-network post-filter activation

NNPFC	neural-network post-filter characteristics

URI	uniform resource identifier

# Subclause 5.8

Add the following function definitions in alphabetical order and adjust the intermediate and subsequent formula indices accordingly:

Reflect( x, y ) =  Min( -y,  x );y < 0Max( x-( y-x ), 0 );y > xy;otherwise	(11)

Wrap( x, y ) =   Max( 0,  x+y+1 );y < 0Min( x, y-x-1 );y > xy;otherwise	(17)

Subclause 6.3

Add the following function definition before the paragraph starting with "read_bits( n )".

more_data_in_payload( ) is specified as follows:

–	If byte_aligned( ) is equal to TRUE and the current position in an SEI message syntax structure or vui_parameters( ) syntax structure is 8 * payloadSize bits from the beginning of the syntax structure, the return value of more_data_in_payload( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_payload( ) is equal to TRUE.

# Subclause 8.1

Replace Table 4 with the following:

| SEI message | Persistence scope |
| --- | --- |
| Filler payload | The PU containing the SEI message |
| User data registered by Rec. ITU-T T.35 | Unspecified |
| User data unregistered | Unspecified |
| Film grain characteristics | Specified by the syntax of the SEI message |
| Frame packing arrangement | Specified by the syntax of the SEI message |
| Display orientation | Specified by the syntax of the SEI message |
| Referenced parameter sets | The CLVS containing the SEI message |
| Decoded picture hash | The PU containing the SEI message |
| Mastering display colour volume | The CLVS containing the SEI message |
| Colour transform information | Specified by the syntax of the SEI message |
| Content light level information | The CLVS containing the SEI message |
| DRAP indication | The picture associated with the SEI message |
| Alternative transfer characteristics | The CLVS containing the SEI message |
| Ambient viewing environment | The CLVS containing the SEI message |
| Content colour volume | Specified by the syntax of the SEI message |
| Equirectangular projection | Specified by the syntax of the SEI message |
| Generalized cubemap projection | Specified by the syntax of the SEI message |
| Sphere rotation | Specified by the syntax of the SEI message |
| Region-wise packing | Specified by the syntax of the SEI message |
| Omnidirectional viewport | Specified by the syntax of the SEI message |
| Alpha channel information | Specified by the syntax of the SEI message |
| Frame-field information | The PU containing the SEI message |
| Depth representation information | Specified by the semantics of the SEI message |
| Multiview acquisition information | The CVS containing the SEI message |
| Multiview view position | The CVS containing the SEI message |
| Annotated regions | Specified by the syntax of the SEI message |
| Sample aspect ratio information | Specified by the syntax of the SEI message |
| Scalability dimension information | The CVS containing the SEI message |
| Extended DRAP indication | The picture associated with the SEI message |
| Shutter interval information | The CVS containing the SEI message |
| Neural-network post-filter characteristics | The CVS containing the SEI message |
| Neural-network post-filter activation | Specified by the syntax of the SEI message |
| Phase indication | Specified by the semantics of the SEI message |
| Post-filter hint | Specified by the syntax of the SEI message |

# Subclause 8.24.2

Replace the following paragraph:

The presence of the EDRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the EDRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures, which consists of a list of IRAP or EDRAP pictures in decoding order that are within the same CLVS and identified by the edrap_ref_rap_id[ i ] syntax elements.

with the following:

The presence of the EDRAP indication SEI message indicates that the constraints on picture order and picture referencing specified in this subclause apply. These constraints can enable a decoder to properly decode the EDRAP picture and the pictures that are in the same layer and follow it in both decoding order and output order without needing to decode any other pictures in the same layer except the list of pictures referenceablePictures. The list referenceablePictures consists of the list of IRAP or EDRAP pictures that are present in the current CLVS and are identified by the edrap_ref_rap_id[ i ] syntax elements, and these pictures are listed in decoder order in the list.

# New subclauses 8.27 to 8.31

Add subclauses 8.27 to 8.31 as follows:

  - Shutter interval information SEI message

### 8.27.1 Shutter interval information SEI message syntax

| shutter_interval_info( payloadSize ) { | Descriptor |
| --- | --- |
| sii_time_scale | u(32) |
| fixed_shutter_interval_within_clvs_flag | u(1) |
| if( fixed_shutter_interval_within_clvs_flag ) |  |
| sii_num_units_in_shutter_interval | u(32) |
| else { |  |
| sii_max_sub_layers_minus1 | u(3) |
| for( i = 0; i <= sii_max_sub_layers_minus1; i++ ) |  |
| sub_layer_num_units_in_shutter_interval[ i ] | u(32) |
| } |  |
| } |  |

### 8.27.2 Shutter interval information SEI message syntax

The shutter interval information SEI message indicates the shutter interval for the associated video source pictures prior to encoding, e.g., for camera-captured content, the shutter interval is amount of time that an image sensor is exposed to produce each source picture.

When a shutter interval information SEI message is present for any picture of a CLVS of a particular layer, a shutter interval information SEI message shall be present for the first picture of the CLVS. The shutter interval information SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All shutter interval information SEI messages that apply to the same CLVS shall have the same content.

sii_time_scale specifies the number of time units that pass in one second. The value of sii_time_scale shall not be equal to 0. For example, a time coordinate system that measures time using a 27 MHz clock has an sii_time_scale of 27 000 000.

fixed_shutter_interval_within_clvs_flag equal to 1 specifies that the indicated shutter interval is the same for all temporal sublayers in the CLVS. fixed_shutter_interval_within_clvs_flag equal to 0 specifies that the indicated shutter interval may not be the same for all temporal sublayers in the CLVS.

sii_num_units_in_shutter_interval, when fixed_shutter_interval_within_clvs_flag is equal to 1, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the indicated shutter interval of each picture in the CLVS. The value 0 may be used to indicate that the associated video content contains screen capture content, computer generated content, or other non-camera-captured content.

The indicated shutter interval, denoted by the variable shutterInterval, in units of seconds, is equal to the quotient of sii_num_units_in_shutter_interval divided by sii_time_scale. For example, to represent a shutter interval equal to 0.04 seconds, sii_time_scale may be equal to 27 000 000 and sii_num_units_in_shutter_interval may be equal to 1 080 000.

sii_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in each CLVS referring to the SPS.

NOTE – For example, the information conveyed in this SEI message is intended to be adequate for purposes corresponding to the use of ATSC A/341:2022-03 Annex D when sii_max_sub_layers_minus1 is equal to 1 and fixed_shutter_interval_within_clvs_flag is equal to 0.

sub_layer_num_units_in_shutter_interval[ i ], when present, specifies the number of time units of a clock operating at the frequency sii_time_scale Hz that corresponds to the shutter interval of each picture with temporal sublayer identifier equal to i in the CLVS. The shutter interval for each picture with temporal sublayer identifier equal to i in the CLVS, denoted by the variable subLayerShutterInterval[ i ], in units of seconds, is equal to the quotient of sub_layer_num_units_in_shutter_interval[ i ] divided by sii_time_scale.

The variable subLayerShutterInterval[ i ], corresponding to the indicated shutter interval of each picture with temporal sublayer identifier equal to i in the CLVS, is thus derived as follows:

if( fixed_shutter_interval_within_clvs_flag )
	subLayerShutterInterval[ i ] = sii_num_units_in_shutter_interval ÷ sii_time_scale	(75)
else
	subLayerShutterInterval[ i ] = sub_layer_num_units_in_shutter_interval[ i ] ÷ sii_time_scale

  - Neural-network post-filter characteristics SEI message

### 8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | u(10) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_purpose & 0x01 != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( nnpfc_purpose & 0x02 != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| if( nnpfc_purpose & 0x04 != 0 ) { |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_output_flag[ i ] | u(1) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| if( nnpfc_inp_format_idc = = 1 ) { |  |
| nnpfc_inp_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_inp_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_inp_order_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_separate_colour_description_present_flag | u(1) |
| if( nnpfc_separate_colour_description_present_flag ) { |  |
| nnpfc_colour_primaries | u(8) |
| nnpfc_transfer_characteristics | u(8) |
| nnpfc_matrix_coeffs | u(8) |
| } |  |
| nnpfc_out_format_idc | ue(v) |
| if( nnpfc_out_format_idc = = 1 ) { |  |
| nnpfc_out_tensor_luma_bitdepth_minus8 | ue(v) |
| nnpfc_out_tensor_chroma_bitdepth_minus8 | ue(v) |
| } |  |
| nnpfc_out_order_idc | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| nnpfc_overlap | ue(v) |
| nnpfc_padding_type | ue(v) |
| if( nnpfc_padding_type = = 4 ) { |  |
| nnpfc_luma_padding_val | ue(v) |
| nnpfc_cb_padding_val | ue(v) |
| nnpfc_cr_padding_val | ue(v) |
| } |  |
| nnpfc_complexity_info_present_flag | ue(v) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

    - Neural-network post-filter characteristics SEI message semantics

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

Use of this SEI message requires the definition of the following variables:

–	Cropped decoded output picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	Luma sample array CroppedYPic[ idx ] and chroma sample arrays CroppedCbPic[ idx ] and CroppedCrPic[ idx ], when present, of the cropped decoded output pictures with idx in the range of 0 to numInputPics − 1, inclusive, that are used as input for the post-processing filter.

–	Bit depth BitDepthY for the luma sample array of the cropped decoded output pictures.

–	Bit depth BitDepthC for the chroma sample arrays, if any, of the cropped decoded output pictures.

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

–	When nnpfc_auxiliary_inp_idc is equal to 1, a filtering strength control value StrengthControlVal that shall be a real number in the range of 0 to 1, inclusive.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

NOTE 1 – More than one NNPFC SEI message can be present for the same picture. When more than one NNPFC SEI message with different values of nnpfc_id is present or activated for the same picture, they can have the same or different values of nnpfc_purpose and nnpfc_mode_idc.

nnpfc_id contains an identifying number that may be used to identify a post-processing filter. The value of nnpfc_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfc_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFC SEI message with nnpfc_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message specifies a base post-processing filter.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS.

When an NNPFC SEI message is a repetition of a previous NNPFC SEI message, in decoding order, in the current CLVS, the subsequent semantics apply as if this SEI message were the only NNPFC SEI message having the same content within the current CLVS.

When an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the following applies:

–	This SEI message defines an update relative to the preceding base post-processing filter in decoding order with the same nnpfc_id value.

–	This SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or the next NNPFC SEI message having that particular nnpfc_id value, in output order, within the current CLVS.

nnpfc_mode_idc equal to 0 indicates that this SEI message contains an ISO/IEC 15938-17 bitstream that specifies a base post-processing filter or is an update relative to the base post-processing filter with the same nnpfc_id value.

When an NNPFC SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_mode_idc equal to 1 specifies that the base post-processing filter associated with the nnpfc_id value is a neural network identified by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

When an NNPFC SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_mode_idc equal to 1 specifies that an update relative to the base post-processing filter with the same nnpfc_id value is defined by the URI indicated by nnpfc_uri with the format identified by the tag URI nnpfc_tag_uri.

The value of nnpfc_mode_idc shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values of 2 to 255, inclusive, for nnpfc_mode_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_mode_idc in the range of 2 to 255, inclusive. Values of nnpfc_mode_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, the post-processing filter PostProcessingFilter( ) is assigned to be the same as the base post-processing filter.

When this SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, a post-processing filter PostProcessingFilter( ) is obtained by applying the update defined by this SEI message to the base post-processing filter.

Updates are not cumulative but rather each update is applied on the base post-processing filter, which is the post-processing filter specified by the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS.

nnpfc_reserved_zero_bit_a shall be equal to 0 in bitstreams conforming to this edition of this document. Decoders shall ignore NNPFC SEI messages in which nnpfc_reserved_zero_bit_a is not equal to 0.

nnpfc_tag_uri contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the format and associated information about the neural network used as a base post-processing filter or an update relative to the base post-processing filter with the same nnpfc_id value specified by nnpfc_uri.

NOTE 2 – nnpfc_tag_uri enables uniquely identifying the format of neural network data specified by nnrpf_uri without needing a central registration authority.

nnpfc_tag_uri equal to "tag:iso.org,2023:15938-17" indicates that the neural network data identified by nnpfc_uri conforms to ISO/IEC 15938-17.

nnpfc_uri contains a URI with syntax and semantics as specified in IETF Internet Standard 66 identifying the neural network used as a base post-processing filter or an update relative to the base post-processing filter with the same nnpfc_id value.

nnpfc_formatting_and_purpose_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_formatting_and_purpose_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 1. When this SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 0.

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 15, inclusive, or equal to 1023 in bitstreams conforming to this edition of this document. Values of 16 to 1022, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 16 to 1022, inclusive.

When no upsampling of any type is indicated by the value of nnpfc_purpose, the NNPF may be used for the purpose of visual quality improvement.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |  |
| --- | --- | --- |
| nnpfc_purpose = = 1023 | May be used as determined by the application |  |
| nnpfc_purpose & 0x01 = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |  |
| nnpfc_purpose & 0x01 != 0 | With chroma upsampling |
| nnpfc_purpose & 0x02 = = 0 | No resolution upsampling (increasing the width or height of the cropped decoded output picture without changing the chroma format) |  |
| nnpfc_purpose & 0x02 != 0 | With resolution upsampling |
| nnpfc_purpose & 0x04 = = 0 | No picture rate upsampling |  |
| nnpfc_purpose & 0x04 != 0 | With picture rate upsampling |  |
| nnpfc_purpose & 0x08 = = 0 | No bit depth upsampling |
| nnpfc_purpose & 0x08 != 0 | With bit depth upsampling |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose & 0x01 shall be equal to 0.

nnpfc_out_sub_c_flag equal to 1 specifies that outSubWidthC is equal to 1 and outSubHeightC is equal to 1. nnpfc_out_sub_c_flag equal to 0 specifies that outSubWidthC is equal to 2 and outSubHeightC is equal to 1. When nnpfc_out_sub_c_flag is not present, outSubWidthC is inferred to be equal to SubWidthC and outSubHeightC is inferred to be equal to SubHeightC. When ChromaFormatIdc is equal to 2 and nnpfc_out_sub_c_flag is present, the value of nnpfc_out_sub_c_flag shall be equal to 1.

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_num_input_pics_minus1 plus 1 specifies the number of decoded output pictures used as input for the post-processing filter. When nnpfc_purpose & 0x04 is not equal to 0, the value of nnpfc_num_input_pics_minus1 shall be greater than 0.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter.

nnpfc_input_pic_output_flag[ i ] equal to 1 indicates that for the i-th input picture the NNPF generates a corresponding output picture. nnpfc_input_pic_output_flag[ i ] equal to 0 indicates that for the i-th input picture the NNPF does not generate a corresponding output picture.

The variables numInputPics, specifying the number of pictures used as input for the post-processing filter, and numOutputPics, specifying the total number of pictures resulting from the post-processing filter, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
if( nnpfc_purpose & 0x04  !=  0 ) {
	for( i = 0, numOutputPics = 0; i < numInputPics; i++ )
		if( nnpfc_input_pic_output_flag[ i ] )
			numOutputPics++
	
	for( i = 0; i  <=  numInputPics − 2; i++ )		(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else
	numOutputPics = 1

nnpfc_component_last_flag equal to 1 indicates that the last dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for a current channel. nnpfc_component_last_flag equal to 0 indicates that the third dimension in the input tensor inputTensor to the post-processing filter and the output tensor outputTensor resulting from the post-processing filter is used for a current channel.

NOTE 4 – The first dimension in the input tensor and in the output tensor is used for the batch index, which is a practice in some neural network frameworks. While formulae in the semantics of this SEI message use the batch size corresponding to the batch index equal to 0, it is up to the post-processing implementation to determine the batch size used as input to the neural network inference.

NOTE 5 – For example, when nnpfc_inp_order_idc is equal to 3 and nnpfc_auxiliary_inp_idc is equal to 1, there are 7 channels in the input tensor, including four luma matrices, two chroma matrices, and one auxiliary input matrix. In this case, the process DeriveInputTensors( ) would derive each of these 7 channels of the input tensor one by one, and when a particular channel of these channels is processed, that channel is referred to as the current channel during the process.

nnpfc_inp_format_idc indicates the method of converting a sample value of the cropped decoded output picture to an input value to the post-processing filter. When nnpfc_inp_format_idc is equal to 0, the input values to the post-processing filter are real numbers and the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) − 1 )		(77)

InpC( x )= x ÷ ( ( 1  <<  BitDepthC ) − 1 )		(78)

When nnpfc_inp_format_idc is equal to 1, the input values to the post-processing filter are unsigned integer numbers and the functions InpY( ) and InpC( ) are specified as follows:

shiftY = BitDepthY − inpTensorBitDepthY
if( inpTensorBitDepthY  >=  BitDepthY)
	InpY( x ) = x  <<  ( inpTensorBitDepthY − BitDepthY )	(79)
else
	InpY( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthY ) − 1, ( x + ( 1  <<  ( shiftY − 1 ) ) )  >>  shiftY )

shiftC = BitDepthC − inpTensorBitDepthC
if( inpTensorBitDepthC  >=  BitDepthC )
	InpC( x ) = x  <<  ( inpTensorBitDepthC − BitDepthC ) 	(80)
else
	InpC( x ) = Clip3(0, ( 1  <<  inpTensorBitDepthC ) − 1, ( x + ( 1  <<  ( shiftC − 1 ) ) )  >>  shiftC )

The variable inpTensorBitDepthY is derived from the syntax element nnpfc_inp_tensor_luma_bitdepth_minus8 as specified below. The variable inpTensorBitDepthC is derived from the syntax element nnpfc_inp_tensor_chroma_bitdepth_minus8 as specified below.

Values of nnpfc_inp_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_inp_format_idc.

nnpfc_inp_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the input integer tensor. The value of inpTensorBitDepthY is derived as follows:

inpTensorBitDepthY = nnpfc_inp_tensor_luma_bitdepth_minus8 + 8	(81)

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the input integer tensor. The value of inpTensorBitDepthC is derived as follows:

inpTensorBitDepthC = nnpfc_inp_tensor_chroma_bitdepth_minus8 + 8	(82)

[Ed. (YK): Update equation indices for subsequent equations.]

It is a requirement of bitstream conformance that the value of nnpfc_inp_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of a cropped decoded output picture as one of the input pictures to the post-processing filter.

The value of nnpfc_inp_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When ChromaFormatIdc is not equal to 1, nnpfc_inp_order_idc shall not be equal to 3.

Table 21 contains an informative description of nnpfc_inp_order_idc values.

Table 21 – Description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix is present in the input tensor for each input picture, and the number of channels is 1. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix and one auxiliary input matrix are present, and the number of channels is 2. |
| 1 | If nnpfc_auxiliary_inp_idc is equal to 0, two chroma matrices are present in the input tensor, and the number of channels is 2. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 3. |
| 2 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma and two chroma matrices are present in the input tensor, and the number of channels is 3. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, one luma matrix, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 4. |
| 3 | If nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices and two chroma matrices are present in the input tensor, and the number of channels is 6. Otherwise when nnpfc_auxiliary_inp_idc is equal to 1, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 7. The luma channels are derived in an interleaved manner as illustrated in Figure 12. This nnpfc_inp_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | Reserved |

Figure 12 – Illustration of deriving the four luma channles (right) from the luma component (left) when nnpfc_inp_order_idc is equal to 3

A patch is a rectangular array of samples from a component (e.g., a luma or chroma component) of a picture.

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the neural-network post-filter. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1 specifies that auxiliary input data is derived as specified in Formula 82.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values of 2 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 2 to 255, inclusive. Values of nnpfc_inp_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++)
			for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = StrengthControlVal
					else
						inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = StrengthControlVal
			}
	else if( nnpfc_inp_order_idc  = =  1 )		(82)
		for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++)
			for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = StrengthControlVal
					else
						inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = StrengthControlVal
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++)
			for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ] ) )
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yP + overlapSize ][ xP + overlapSize ] = StrengthControlVal
					else
						inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 3 ] = StrengthControlVal
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −overlapSize; yP < inpPatchHeight + overlapSize; yP++)
			for( xP = −overlapSize; xP < inpPatchWidth + overlapSize; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ] ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ] ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ] ) )
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yP + overlapSize ][ xP + overlapSize ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yP + overlapSize ][ xP + overlapSize ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yP + overlapSize ][ xP + overlapSize ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yP + overlapSize ][ xP + overlapSize ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yP + overlapSize ][ xP + overlapSize ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yP + overlapSize ][ xP + overlapSize ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 )
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yP + overlapSize ][ xP + overlapSize ] = StrengthControlVal
					else
						inputTensor[ 0 ][ i ][ yP + overlapSize ][ xP + overlapSize ][ 6 ] = StrengthControlVal
			}
}

nnpfc_separate_colour_description_present_flag equal to 1 indicates that a distinct combination of colour primaries, transfer characteristics, and matrix coefficients for the picture resulting from the post-processing filter is specified in the SEI message syntax structure. nnfpc_separate_colour_description_present_flag equal to 0 indicates that the combination of colour primaries, transfer characteristics, and matrix coefficients for the picture resulting from the post-processing filter is the same as indicated in VUI parameters for the CLVS.

nnpfc_colour_primaries has the same semantics as specified in subclause 7.3 for the vui_colour_primaries syntax element, except as follows:

–	nnpfc_colour_primaries specifies the colour primaries of the picture resulting from applying the neural-network post-filter specified in the SEI message, rather than the colour primaries used for the CLVS.

–	When nnpfc_colour_primaries is not present in the NNPFC SEI message, the value of nnpfc_colour_primaries is inferred to be equal to vui_colour_primaries.

nnpfc_transfer_characteristics has the same semantics as specified in subclause 7.3 for the vui_transfer_characteristics syntax element, except as follows:

–	nnpfc_transfer_characteristics specifies the transfer characteristics of the picture resulting from applying the neural-network post-filter specified in the SEI message, rather than the transfer characteristics used for the CLVS.

–	When nnpfc_transfer_characteristics is not present in the NNPFC SEI message, the value of nnpfc_transfer_characteristics is inferred to be equal to vui_transfer_characteristics.

nnpfc_matrix_coeffs has the same semantics as specified in subclause 7.3 for the vui_matrix_coeffs syntax element, except as follows:

–	nnpfc_matrix_coeffs specifies the matrix coefficients of the picture resulting from applying the neural-network post-filter specified in the SEI message, rather than the matrix coefficients used for the CLVS.

–	When nnpfc_matrix_coeffs is not present in the NNPFC SEI message, the value of nnpfc_matrix_coeffs is inferred to be equal to vui_matrix_coeffs.

–	The values allowed for nnpfc_matrix_coeffs are not constrained by the chroma format of the decoded video pictures that is indicated by the value of ChromaFormatIdc for the semantics of the VUI parameters.

–	When nnpfc_matrix_coeffs is equal to 0, nnpfc_out_order_idc shall not be equal to 1 or 3.

nnpfc_out_format_idc equal to 0 indicates that the sample values output by the post-processing filter are real numbers where the value range of 0 to 1, inclusive, maps linearly to the unsigned integer value range of 0 to ( 1  <<  bitDepth ) – 1, inclusive, for any desired bit depth bitDepth for subsequent post-processing or displaying.

nnpfc_out_format_idc equal to 1 indicates that the luma sample values output by the post-processing filter are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_luma_bitdepth_minus8 + 8 ) ) − 1, inclusive, and the chroma sample values output by the post-processing filter are unsigned integer numbers in the range of 0 to ( 1  <<  ( nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 ) ) − 1, inclusive.

Values of nnpfc_out_format_idc greater than 1 are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages that contain reserved values of nnpfc_out_format_idc.

nnpfc_out_tensor_luma_bitdepth_minus8 plus 8 specifies the bit depth of luma sample values in the output integer tensor. The value of nnpfc_out_tensor_luma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

nnpfc_out_tensor_chroma_bitdepth_minus8 plus 8 specifies the bit depth of chroma sample values in the output integer tensor. The value of nnpfc_out_tensor_chroma_bitdepth_minus8 shall be in the range of 0 to 24, inclusive.

When nnpfc_purpose & 0x08 is not equal to 0, the value of nnpfc_out_format_idc shall be equal to 1 and at least one of the following conditions shall be true:

–	nnpfc_out_tensor_luma_bitdepth_minus8 + 8 is greater than BitDepthY.

–	nnpfc_out_tensor_chroma_bitdepth_minus8 + 8 is greater than BitDepthC.

nnpfc_out_order_idc indicates the output order of samples resulting from the post-processing filter.

The value of nnpfc_out_order_idc shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this edition of this document. Values of 4 to 255, inclusive, for nnpfc_out_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_out_order_idc in the range of 4 to 255, inclusive. Values of nnpfc_out_order_idc greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When nnpfc_purpose & 0x01 is not equal to 0, nnpfc_out_order_idc shall not be equal to 3.

Table 22 contains an informative description of nnpfc_out_order_idc values.

Table 22 – Description of nnpfc_out_order_idc values

| nnpfc_out_ order_idc | Description |
| --- | --- |
| 0 | Only the luma matrix is present in the output tensor, thus the number of channels is 1. |
| 1 | Only the chroma matrices are present in the output tensor, thus the number of channels is 2. |
| 2 | The luma and chroma matrices are present in the output tensor, thus the number of channels is 3. |
| 3 | Four luma matrices and two chroma matrices are present in the output tensor, thus the number of channels is 6. This nnpfc_out_order_idc can only be used when the chroma format is 4:2:0. |
| 4..255 | Reserved |

The process StoreOutputTensors( ), for deriving sample values in the filtered output sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic from the output tensor outputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numOutputPics; i++ ) {
	if( nnpfc_out_order_idc  = =  0 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				if ( yY < nnpfc_pic_height_in_luma_samples  &&  xY < nnpfc_pic_width_in_luma_samples )
					if( !nnpfc_component_last_flag )
						FilteredYPic[ i ][ xY ][yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
					else
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]			}
	else if( nnpfc_out_order_idc  = =  1 )		(83)
		for( yP = 0; yP < outPatchCHeight; yP++)
			for( xP = 0; xP < outPatchCWidth; xP++ ) {
				xSrc = cLeft * horCScaling + xP
				ySrc = cTop * verCScaling + yP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / outSubHeightC  &&
						xSrc < nnpfc_pic_width_in_luma_samples / outSubWidthC )
					if( !nnpfc_component_last_flag ) {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
					} else {
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  2 )
		for( yP = 0; yP < outPatchHeight; yP++)
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				yY = cTop * outPatchHeight / inpPatchHeight + yP
				xY = cLeft * outPatchWidth / inpPatchWidth + xP
				yC = yY / outSubHeightC 
				xC = xY / outSubWidthC 
				yPc = ( yP / outSubHeightC ) * outSubHeightC
				xPc = ( xP / outSubWidthC ) * outSubWidthC
				if ( yY < nnpfc_pic_height_in_luma_samples && xY < nnpfc_pic_width_in_luma_samples)
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 1 ][ yPc ][ xPc ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ 2 ][ yPc ][ xPc ]
					} else {
						FilteredYPic[ i ][ xY ][ yY ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredCbPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 1 ]
						FilteredCrPic[ i ][ xC ][ yC ] = outputTensor[ 0 ][ i ][ yPc ][ xPc ][ 2 ]
					}
			}
	else if( nnpfc_out_order_idc  = =  3 )
		for( yP = 0; yP < outPatchHeight; yP++ )
			for( xP = 0; xP < outPatchWidth; xP++ ) {
				ySrc = cTop / 2 * outPatchHeight / inpPatchHeight + yP
				xSrc = cLeft / 2 * outPatchWidth / inpPatchWidth + xP
				if ( ySrc < nnpfc_pic_height_in_luma_samples / 2  &&
						xSrc < nnpfc_pic_width_in_luma_samples / 2 )
					if( !nnpfc_component_last_flag ) {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 0 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ 1 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 2 ][ yP ][ xP ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ 3 ][ yP ][ xP ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 4 ][ yP ][ xP ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ 5 ][ yP ][ xP ]
					} else {
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 0 ]
						FilteredYPic[ i ][ xSrc * 2 + 1 ][ ySrc * 2 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 1 ]
						FilteredYPic[ i ][ xSrc * 2 ][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 2 ]
						FilteredYPic[ i ][ xSrc * 2 + 1][ ySrc * 2 + 1 ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 3 ]
						FilteredCbPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 4 ]
						FilteredCrPic[ i ][ xSrc ][ ySrc ] = outputTensor[ 0 ][ i ][ yP ][ xP ][ 5 ]
					}
			}
}

nnpfc_constant_patch_size_flag equal to 1 indicates that the post-processing filter accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the post-processing filter accepts any patch size that is a positive integer multiple of the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input.

nnpfc_patch_width_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32766, CroppedWidth − 1 ), inclusive.

nnpfc_patch_height_minus1 + 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the post-processing filter. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32766, CroppedHeight − 1 ), inclusive.

Let the variables inpPatchWidth and inpPatchHeight be the patch size width and the patch size height, respectively.

If nnpfc_constant_patch_size_flag is equal to 0, the following applies:

–	The values of inpPatchWidth and inpPatchHeight are either provided by external means not specified in this document or set by the post-processor itself.

–	The value of inpPatchWidth shall be a positive integer multiple of nnpfc_patch_width_minus1 + 1 and shall be less than or equal to CroppedWidth. The value of inpPatchHeight shall be a positive integer multiple of nnpfc_patch_height_minus1 + 1 and shall be less than or equal to CroppedHeight.

Otherwise (nnpfc_constant_patch_size_flag is equal to 1), the value of inpPatchWidth is set equal to nnpfc_patch_width_minus1 + 1 and the value of inpPatchHeight is set equal to nnpfc_patch_height_minus1 + 1.

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the post-processing filter. The value of nnpfc_overlap shall be in the range of 0 to 16383, inclusive.

The variables outPatchWidth, outPatchHeight, horCScaling, verCScaling, outPatchCWidth, outPatchCHeight, and overlapSize are derived as follows:

outPatchWidth = ( nnpfc_pic_width_in_luma_samples * inpPatchWidth ) / CroppedWidth	(84)

outPatchHeight = ( nnpfc_pic_height_in_luma_samples * inpPatchHeight ) / CroppedHeight	(85)

horCScaling = SubWidthC / outSubWidthC		(86)

verCScaling = SubHeightC / outSubHeightC		(87)

outPatchCWidth = outPatchWidth * horCScaling		(88)

outPatchCHeight = outPatchHeight * verCScaling		(89)

overlapSize = nnpfc_overlap		(90)

It is a requirement of bitstream conformance that outPatchWidth * CroppedWidth shall be equal to nnpfc_pic_width_in_luma_samples * inpPatchWidth and outPatchHeight * CroppedHeight shall be equal to nnpfc_pic_height_in_luma_samples * inpPatchHeight.

nnpfc_padding_type indicates the process of padding when referencing sample locations outside the boundaries of the cropped decoded output picture as described in Table 23. The value of nnpfc_padding_type shall be in the range of 0 to 15, inclusive.

Table 23 – Informative description of nnpfc_padding_type values

| nnpfc_padding_type | Description |
| --- | --- |
| 0 | zero padding |
| 1 | replication padding |
| 2 | reflection padding |
| 3 | wrap-around padding |
| 4 | fixed padding |
| 5..15 | Reserved |

nnpfc_luma_padding_val indicates the luma value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cb_padding_val indicates the Cb value to be used for padding when nnpfc_padding_type is equal to 4.

nnpfc_cr_padding_val indicates the Cr value to be used for padding when nnpfc_padding_type is equal to 4.

The function InpSampleVal( y, x, picHeight, picWidth, croppedPic ) with inputs being a vertical sample location y, a horizontal sample location x, a picture height picHeight, a picture width picWidth, and sample array croppedPic returns the value of sampleVal derived as follows:

NOTE 6 – For the inputs to the function InpSampleVal( ), the vertical location is listed before the horizontal location for compatibility with input tensor conventions of some inference engines.

if( nnpfc_padding_type  = =  0 )
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal = 0
	else
		sampleVal = croppedPic[ x ][ y ]		(91)
else if( nnpfc_padding_type  = =  1 )
	sampleVal = croppedPic[ Clip3( 0, picWidth − 1, x ) ][ Clip3( 0, picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  2 ) 
	sampleVal = croppedPic[ Reflect( picWidth − 1, x ) ][ Reflect( picHeight − 1, y ) ]
else if( nnpfc_padding_type  = =  3 ) 
	if( y >= 0  &&  y < picHeight )
		sampleVal = croppedPic[ Wrap( picWidth − 1, x ) ][ y ] 
else if( nnpfc_padding_type  = =  4 ) 
	if( y < 0  | |  x < 0  | |  y >= picHeight  | |  x  >=  picWidth )
		sampleVal[ 0 ]  = nnpfc_luma_padding_val			sampleVal[ 1 ]  = nnpfc_cb_padding_val
		sampleVal[ 2 ] = nnpfc_cr_padding_val
	else
		sampleVal = croppedPic[ x ][ y ]

The following example process may be used, with the post-processing filter PostProcessingFilter( ), to generate, in a patch-wise manner, the filtered and/or interpolated picture(s), which contain Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, as indicated by nnpfc_out_order_idc.

if( nnpfc_inp_order_idc  = =  0 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  1 )
	for( cTop = 0; cTop < CroppedHeight / SubHeightC; cTop  +=  inpPatchHeight )
		for( cLeft = 0; cLeft < CroppedWidth / SubWidthC; cLeft  +=  inpPatchWidth ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  2 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight)	(92)
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}
else if( nnpfc_inp_order_idc  = =  3 )
	for( cTop = 0; cTop < CroppedHeight; cTop  +=  inpPatchHeight * 2 )
		for( cLeft = 0; cLeft < CroppedWidth; cLeft  +=  inpPatchWidth * 2 ) {
			DeriveInputTensors( )
			outputTensor = PostProcessingFilter( inputTensor )
			StoreOutputTensors( )
		}

nnpfc_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. nnpfc_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present.

nnpfc_parameter_type_idc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters. nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters. nnpfc_parameter_type_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_parameter_type_idc equal to 3.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present the neural network does not use parameters of bit length greater than 1.

nnpfc_num_parameters_idc indicates the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is unknown. The value nnpfc_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of nnpfc_num_parameters_idc greater than 52 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_num_parameters_idc greater than 52.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2048  <<  nnpfc_num_parameters_idc ) − 1	(93)

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 indicates that the maximum number of multiply-accumulate operations per sample of the post-processing filter is less than or equal to nnpfc_num_kmac_operations_idc * 1000. nnpfc_num_kmac_operations_idc equal to 0 indicates that the maximum number of multiply-accumulate operations of the network is unknown. The value of nnpfc_num_kmac_operations_idc shall be in the range of 0 to 232 − 1, inclusive.

nnpfc_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the neural network. The total size in bits is a number equal to or greater than the sum of bits used to store each parameter. nnpfc_total_kilobyte_size is the total size in bits divided by 8000, rounded up. nnpfc_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the neural network is unknown. The value of nnpfc_total_kilobyte_size shall be in the range of 0 to 232 − 1, inclusive.

nnpfc_reserved_zero_bit_b shall be equal to 0 in bitstreams conforming to this edition of this document. Decoders shall ignore NNPFC SEI messages in which nnpfc_reserved_zero_bit_b is not equal to 0.

nnpfc_payload_byte[ i ] contains the i-th byte of a bitstream conforming to ISO/IEC 15938-17. The byte sequence nnpfc_payload_byte[ i ] for all present values of i shall be a complete bitstream that conforms to ISO/IEC 15938-17.

  - Neural-network post-filter activation SEI message

    - Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| } |  |

    - Neural-network post-filter activation SEI message semantics

The neural-network post-filter activation (NNPFA) SEI message activates or de-activates the possible use of the target neural-network post-processing filter, identified by nnpfa_target_id, for post-processing filtering of a set of pictures.

NOTE 1 – There can be several NNPFA SEI messages present for the same picture, for example, when the post-processing filters are meant for different purposes or filter different colour components.

nnpfa_target_id indicates the target neural-network post-processing filter, which is specified by one or more neural-network post-processing filter characteristics SEI messages that pertain to the current picture and have nnpfc_id equal to nnfpa_target_id.

The value of nnpfa_target_id shall be in the range of 0 to 232 − 2, inclusive. Values of nnpfa_target_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering an NNPFA SEI message with nnpfa_target_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, shall ignore the SEI message.

An NNPFA SEI message with a particular value of nnpfa_target_id shall not be present in a current PU unless one or both of the following conditions are true:

–	Within the current CLVS there is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id present in a PU preceding the current PU in decoding order.

–	There is an NNPFC SEI message with nnpfc_id equal to the particular value of nnpfa_target_id in the current PU.

When a PU contains both an NNPFC SEI message with a particular value of nnpfc_id and an NNPFA SEI message with nnpfa_target_id equal to the particular value of nnpfc_id, the NNPFC SEI message shall precede the NNPFA SEI message in decoding order.

nnpfa_cancel_flag equal to 1 indicates that the persistence of the target neural-network post-processing filter established by any previous NNPFA SEI message with the same nnpfa_target_id as the current SEI message is cancelled, i.e., the target neural-network post-processing filter is no longer used unless it is activated by another NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 0. nnpfa_cancel_flag equal to 0 indicates that the nnpfa_persistence_flag follows.

nnpfa_persistence_flag specifies the persistence of the target neural-network post-processing filter for the current layer.

nnpfa_persistence_flag equal to 0 specifies that the target neural-network post-processing filter may be used for post-processing filtering for the current picture only.

nnpfa_persistence_flag equal to 1 specifies that the target neural-network post-processing filter may be used for post-processing filtering for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1 is output that follows the current picture in output order.

NOTE 2 – The target neural-network post-processing filter is not applied for this subsequent picture in the current layer associated with a NNPFA SEI message with the same nnpfa_target_id as the current SEI message and nnpfa_cancel_flag equal to 1.

  - Phase indication SEI message

    - Phase indication SEI message syntax

| phase_indication( payloadSize ) { | Descriptor |
| --- | --- |
| hor_phase_num | u(8) |
| hor_phase_den_minus1 | u(8) |
| ver_phase_num | u(8) |
| ver_phase_den_minus1 | u(8) |
| } |  |

    - Phase indication SEI message semantics

The phase indication SEI message provides the decoder with information about the position of luma sampling locations in cropped decoded pictures relative to a rendering window. This information may be used by a decoder to ensure the correct spatial alignment of rendered pictures, for example when switching between picture resolutions.

Figure 13: The ratios ab and cd represent the horizontal and vertical locations of the luma samples (marked with x) relative to a rendering window. ab is equal to hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ), and cd to ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ).

NOTE 1 – When the number of luma output samples in horizontal direction is equal to the width of the rendering window, and hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ) is equal to 1÷2, the picture is intended to be rendered without applying any horizontal phase shift. Correspondingly, when the number of luma output samples in vertical direction is equal to the height of the rendering window, and ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ) is equal to 1÷2, the picture is intended to be rendered without applying any vertical phase shift.

Use of this SEI message requires the definition of the following variables:

–	A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

The phase indication SEI message applies to the current cropped decoded picture and persists for all subsequent pictures of the current layer in output order with the same value of CroppedWidth as the current picture and the same value of CroppedHeight as the current picture until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer with an associated phase indication SEI message and the same value of CroppedWidth as the current picture and the same value of CroppedHeight as the current picture is output and follows the current picture in output order.

hor_phase_num and hor_phase_den_minus1 specify the horizontal position of luma sampling locations relative to a rendering window. The horizontal position hor_phase_num ÷ ( hor_phase_den_minus1 + 1 ) is expressed in units of the horizontal distance between two horizontally adjacent luma sampling locations. The value of hor_phase_num shall be greater than or equal to 0 and less than or equal to hor_phase_den_minus1 + 1.

ver_phase_num and ver_phase_den_minus1 specify the vertical position of luma sampling locations relative to a rendering window. The vertical position ver_phase_num ÷ ( ver_phase_den_minus1 + 1 ) is expressed in units of the vertical distance between two vertically adjacent luma sampling locations. The value of ver_phase_num shall be greater than or equal to 0 and less than or equal to ver_phase_den_minus1 + 1.

NOTE 2 – The phase indicators can be used during the rendering process. For example, texture coordinates can be offset by an amount proportional to the signalled horizontal and vertical phase indicators.

NOTE 3 – The signalled phase indicators applies to the luma samples of the decoded pictures. The phase offset for chroma samples can be derived from the signalled phase indicators taking into account the chroma sample location relative to luma sample location as indicated by ChromaFormatIdc, vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field. When ChromaFormatIdc is equal to 1 (4:2:0 chroma format) and the value of vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field, as applicable, are equal to 6 or are inferred to be equal to 6, the nominal vertical and horizontal relative locations of luma and chroma samples that corresponds to vui_chroma_sample_loc_type_frame, vui_chroma_sample_loc_type_top_field and vui_chroma_sample_loc_type_bottom_field equal to 0 may be assumed.

  - Post-filter hint SEI message

    - Post-filter hint SEI message syntax

| post_filter_hint( payloadSize ) { | Descriptor |
| --- | --- |
| filter_hint_cancel_flag | u(1) |
| if( !filter_hint_cancel_flag ) { |  |
| filter_hint_persistence_flag | u(1) |
| filter_hint_size_y | ue(v) |
| filter_hint_size_x | ue(v) |
| filter_hint_type | u(2) |
| filter_hint_chroma_coeff_present_flag | u(1) |
| for( cIdx = 0; cIdx < ( filter_hint_chroma_coeff_present_flag ? 3 : 1 ); cIdx++ ) |  |
| for( cy = 0; cy < filter_hint_size_y; cy++ ) |  |
| for( cx = 0; cx < filter_hint_size_x; cx++ ) |  |
| filter_hint_value[ cIdx ][ cy ][ cx ] | se(v) |
| } |  |
| } |  |

    - Post-filter hint SEI message semantics

This SEI message provides the coefficients of a post-filter or correlation information for the design of a post-filter for potential use in post-processing of a set of pictures after they are decoded and output to obtain improved displayed quality.

filter_hint_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous post-filter hint SEI message in output order that applies to the current layer. filter_hint_cancel_flag equal to 0 indicates that post-filter hint information follows.

filter_hint_persistence_flag specifies the persistence of the post-filter hint SEI message for the current layer. filter_hint_persistence_flag equal to 0 specifies that the post-filter hint applies to the current decoded picture only. filter_hint_persistence_flag equal to 1 specifies that the post-filter hint SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

- A new CLVS of the current layer begins.

- The bitstream ends.

- A picture in the current layer in an AU associated with a post-filter hint SEI message is output that follows the current picture in output order.

filter_hint_size_y specifies the vertical size of the filter coefficient or correlation array. The value of filter_hint_size_y shall be in the range of 1 to 15, inclusive.

filter_hint_size_x specifies the horizontal size of the filter coefficient or correlation array. The value of filter_hint_size_x shall be in the range of 1 to 15, inclusive.

filter_hint_type identifies the type of the transmitted filter hints as specified in Table 24. The value of filter_hint_type shall be in the range of 0 to 2, inclusive. The value of filter_hint_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore post-filter hint SEI messages having filter_hint_type equal to 3.

Table 24 – filter_hint_type values

| Value | Description |
| --- | --- |
| 0 | Coefficients of a 2D-FIR filter |
| 1 | Coefficients of two 1D-FIR filters |
| 2 | Cross-correlation matrix |

filter_hint_chroma_coeff_present_flag equal 1 specifies that filter coefficients for chroma are present. filter_hint_chroma_coeff_present_flag equal 0 specifies that filter coefficients for chroma are not present.

filter_hint_value[ cIdx ][ cy ][ cx ] specifies a filter coefficient or an element of a cross-correlation matrix between the original and the decoded signal with 16-bit precision. The value of filter_hint_value[ cIdx ][ cy ][ cx ] shall be in the range of −231 + 1 to 231 − 1, inclusive. cIdx specifies the related colour component, cy represents a counter in vertical direction and cx represents a counter in horizontal direction. Depending on the value of filter_hint_type, the following applies:

–	If filter_hint_type is equal to 0, the coefficients of a 2-dimensional finite impulse response (FIR) filter with the size of filter_hint_size_y * filter_hint_size_x are transmitted.

–	Otherwise, if filter_hint_type is equal to 1, the filter coefficients of two 1-dimensional FIR filters are transmitted. In this case, filter_hint_size_y shall be equal to 2. The index cy equal to 0 specifies the filter coefficients of the horizontal filter and cy equal to 1 specifies the filter coefficients of the vertical filter. In the filtering process, the horizontal filter is applied first and the result is filtered by the vertical filter.

–	Otherwise (filter_hint_type is equal to 2), the transmitted hints specify a cross-correlation matrix between the original signal s and the decoded signal s′.

NOTE 1 – The normalized cross-correlation matrix for a related colour component identified by cIdx with the size of filter_hint_size_y * filter_hint_size_x is defined as follows:

(94)

where s denotes array of samples of the colour component cIdx of the original picture, s′ denotes corresponding array of the decoded picture, h denotes the vertical height of the related colour component, w denotes the horizontal width of the related colour component, bitDepth denotes the bit depth of the colour component, OffsetY is equal to ( filter_hint_size_y  >>  1 ), OffsetX is equal to ( filter_hint_size_x  >>  1 ), 0  <=  cy < filter_hint_size_y and 0  <=  cx < filter_hint_size_x.

NOTE 2 – A decoder can derive a Wiener post-filter from the cross-correlation matrix of original and decoded signal and the auto-correlation matrix of the decoded signal.

# Bibliography

Add a bibliographic reference as follows:

[16]	ATSC A/341:2022-03, ATSC Standard: Video – HEVC.