| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0128-v1 |
| --- | --- |

| Title: | AHG9: On the signalling of complexity information in NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung hwan Kim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the current VSEI specification, the signalling of complexity information in NNPFC SEI message may be present only in the SEI message that contains base filter (i.e., the first NNPFC SEI message with a particular nnpfc_id). It is asserted that an NNPFC SEI message that updates a base NNPFC SEI message may have different complexity, which can be higher or lower. Therefore, the current design of the complexity information signalling in NNPFC SEI message puts limitation such that an NNPFC SEI message that updates a base NNPFC SEI message cannot have different complexity.

This contribution proposes 2 options for modifying the signalling of complexity information in NNPFC SEI message, they are:

Option 1: Move the signalling related to complexity information outside the nnpfc_formatting_and_purpose_flag condition. Furthermore, update the semantics for the complexity signalling such when complexity information is not present in an NNPFC SEI message that contains update is not present, it is inferred to be the same as the complexity information signalled in the NNPFC SEI message with the same Id that contains the base filter.

Option 2: Move the signalling related to complexity information outside the nnpfc_formatting_and_purpose_flag condition and change the nnpfc_complexity_info_present_flag to be nnpfc_complexity_info_present_idc (2 bits). The semantics of this indication is as follows:

- Value 0 means that syntax elements related to complexity information are not present in the SEI message. If the SEI message is an NNPFC SEI message that is not a base NNPFC SEI message, the complexity information signalled in the base SEI message does not apply to the SEI message.

- Value 1 means that syntax elements related to complexity information are not present in the SEI message. If the SEI message is an NNPFC SEI message that is not a base NNPFC SEI message and the base SEI message associated with this SEI message contains complexity information, the complexity information applies to this SEI message as well.

- Value 2 means that syntax elements related to complexity information are present in the SEI message.

- Value 3 is reserved.

# Problem Statement

Neural-network post-filter characteristics (NNPFC) SEI message includes feature to signal complexity information which may be helpful for decoder to decide whether or not it has enough resource to meet the complexity to do the filtering process. The presence of the complexity information is gated by two flags: the first one is nnpfc_formatting_and_purpose_flag and the second one is nnpfc_complexity_info_present_flag. The second flag is signalled only when the first one is equal to 1. Furthermore, it is constrained that the value of the first one shall be equal to 1 only for the NNPFC SEI message that contains the base filter (i.e., the first NNPFC SEI message, in decoding order, with a particular nnpfc_id) whereas for NNPFC SEI message that contains the update filter (i.e., the NNPFC SEI message with a particular nnpfc_id that is not the first one in decoding order), the value of the first flag shall be equal to 0. Table below show the syntax table for the signalling with syntax elements not related to the complexity information signalling removed.

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| … |  |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

It is asserted that the current signalling disallows the signalling of complexity information for NNPFC SEI message that contains filter that update the base filter. An NNPFC SEI message that updates a base NNPFC SEI message may have different complexity than that of the base NNPFC SEI message, which can be higher or lower. Therefore, the current signalling design basically putting a constraint or limit on the NNPFC SEI message that updates a base NNPFC SEI message such that it cannot have different (or at least higher) complexity.

# Proposal

To overcome the asserted problem, we propose that the presence of complexity information in NNPFC SEI message is not limited to only in NNPFC SEI message that contains base filter. For the detail of how the signalling would be, we suggest one of the following options:

## Option 1

Move the signalling related to complexity information outside the nnpfc_formatting_and_purpose_flag condition. Furthermore, update the semantics for the complexity signalling such when complexity information is not present in an NNPFC SEI message that contains update is not present, it is inferred to be the same as the complexity information signalled in the NNPFC SEI message with the same Id that contains the base filter.

## Option 2

For indicating the presence of the complexity information signalling, instead of using a flag, use a two-bit indication (i.e., nnpfc_complexity_info_present_idc) and signal it outside the nnpfc_formatting_and_purpose_flag condition. The semantics of this indication is as follows:

- Value 0 means that syntax elements related to complexity information are not present in the SEI message. If the SEI message is an NNPFC SEI message that is not a base NNPFC SEI message, the complexity information signalled in the base SEI message may not apply to the SEI message.

- Value 1 means that syntax elements related to complexity information are not present in the SEI message. If the SEI message is an NNPFC SEI message that is not a base NNPFC SEI message and the base SEI message associated with this SEI message contains complexity information, the complexity information applies to this SEI message as well.

- Value 2 means that syntax elements related to complexity information are present in the SEI message.

- Value 3 is reserved.

# Text implementation

## Option 1

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| nnpfc_complexity_info_present_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| … |  |
| nnpfc_complexity_info_present_flag | ue(v) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| … |  |
| } |  |
| } |  |

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

…

formatting_and_purpose_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_formatting_and_purpose_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 1. When this SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 0.

nnpfc_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. nnpfc_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present.

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

…

nnpfc_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. nnpfc_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present.

nnpfc_parameter_type_idc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters. nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters. nnpfc_parameter_type_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_parameter_type_idc equal to 3. When not present, the SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, and the value of nnpfc_complexity_info_present_flag is equal to 1 in the first NNPFC SEI message that has the particular nnpfc_id, the value of nnpfc_parameter_type_idc is inferred to be equal to the value of nnpfc_parameter_type_idc in that first NNPFC SEI message.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present the neural network does not use parameters of bit length greater than 1. When not present, the SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, and the value of nnpfc_complexity_info_present_flag is equal to 1 in the first NNPFC SEI message that has the particular nnpfc_id, the value of nnpfc_log2_parameter_bit_length_minus3 is inferred to be equal to the value of nnpfc_log2_parameter_bit_length_minus3 in that first NNPFC SEI message.

nnpfc_num_kmac_operations_idc indicates the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is unknown. The value nnpfc_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of nnpfc_num_parameters_idc greater than 52 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_num_parameters_idc greater than 52. When not present, the SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, and the value of nnpfc_complexity_info_present_flag is equal to 1 in the first NNPFC SEI message that has the particular nnpfc_id, the value of nnpfc_num_kmac_operations_idc is inferred to be equal to the value of nnpfc_num_kmac_operations_idc in that first NNPFC SEI message.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2048  <<  nnpfc_num_parameters_idc ) − 1	(93)

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 indicates that the maximum number of multiply-accumulate operations per sample of the post-processing filter is less than or equal to nnpfc_num_kmac_operations_idc * 1000. nnpfc_num_kmac_operations_idc equal to 0 indicates that the maximum number of multiply-accumulate operations of the network is unknown. The value of nnpfc_num_kmac_operations_idc shall be in the range of 0 to 232 − 1, inclusive. When not present, the SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, and the value of nnpfc_complexity_info_present_flag is equal to 1 in the first NNPFC SEI message that has the particular nnpfc_id, the value of nnpfc_num_kmac_operations_idc is inferred to be equal to the value of nnpfc_num_kmac_operations_idc in that first NNPFC SEI message.

nnpfc_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the neural network. The total size in bits is a number equal to or greater than the sum of bits used to store each parameter. nnpfc_total_kilobyte_size is the total size in bits divided by 8000, rounded up. nnpfc_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the neural network is unknown. The value of nnpfc_total_kilobyte_size shall be in the range of 0 to 232 − 1, inclusive. When not present, the SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, and the value of nnpfc_complexity_info_present_flag is equal to 1 in the first NNPFC SEI message that has the particular nnpfc_id, the value of nnpfc_total_kilobyte_size is inferred to be equal to the value of nnpfc_total_kilobyte_size in that first NNPFC SEI message.

…

## Option 2

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_id | ue(v) |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| nnpfc_complexity_info_present_idc | u(2) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| … |  |
| nnpfc_complexity_info_present_flag | ue(v) |
| if( nnpfc_complexity_info_present_flag ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| } |  |
| if( nnpfc_complexity_info_present_idc = = 2 ) { |  |
| nnpfc_parameter_type_idc | u(2) |
| if( nnpfc_parameter_type_idc != 2 ) |  |
| nnpfc_log2_parameter_bit_length_minus3 | u(2) |
| nnpfc_num_parameters_idc | u(6) |
| nnpfc_num_kmac_operations_idc | ue(v) |
| nnpfc_total_kilobyte_size | ue(v) |
| } |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| … |  |
| } |  |
| } |  |

The neural-network post-filter characteristics (NNPFC) SEI message specifies a neural network that may be used as a post-processing filter. The use of specified post-processing filters for specific pictures is indicated with neural-network post-filter activation SEI messages.

…

formatting_and_purpose_flag equal to 1 specifies that syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present. nnpfc_formatting_and_purpose_flag equal to 0 specifies that no syntax elements related to the filter purpose, input formatting, output formatting, and complexity are present.

When this SEI message is the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 1. When this SEI message is not the first NNPFC SEI message, in decoding order, that has a particular nnpfc_id value within the current CLVS, nnpfc_formatting_and_purpose_flag shall be equal to 0.

nnpfc_complexity_info_present_idc equal to 0 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present and complexity of the filter associated with the SEI message is unknown. nnpfc_complexity_info_present_idc equal to 1 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present and their values are inferred to be equal to the corresponding syntax element in the first NNPFC SEI message with the same nnpfc_id. nnpfc_complexity_info_present_idc equal to 2 specifies that syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. Value 3 of nnpfc_complexity_info_present_idc is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document.

It is a requirement of bitstream conformance that the value of nnpfc_complexity_info_present_idc shall not be equal to 1 when the SEI message contains the base post_processing filter associated with the nnpfc_id.

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

…

nnpfc_complexity_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the complexity of the post-processing filter associated with the nnpfc_id are present. nnpfc_complexity_info_present_flag equal to 0 specifies that no syntax elements that indicates the complexity of the post-processing filter associated with the nnpfc_id are present.

nnpfc_parameter_type_idc equal to 0 indicates that the neural network uses only integer parameters. nnpfc_parameter_type_flag equal to 1 indicates that the neural network may use floating point or integer parameters. nnpfc_parameter_type_idc equal to 2 indicates that the neural network uses only binary parameters. nnpfc_parameter_type_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_parameter_type_idc equal to 3.

nnpfc_log2_parameter_bit_length_minus3 equal to 0, 1, 2, and 3 indicates that the neural network does not use parameters of bit length greater than 8, 16, 32, and 64, respectively. When nnpfc_parameter_type_idc is present and nnpfc_log2_parameter_bit_length_minus3 is not present the neural network does not use parameters of bit length greater than 1.

nnpfc_num_kmac_operations_idc indicates the maximum number of neural network parameters for the post processing filter in units of a power of 2048. nnpfc_num_parameters_idc equal to 0 indicates that the maximum number of neural network parameters is unknown. The value nnpfc_num_parameters_idc shall be in the range of 0 to 52, inclusive. Values of nnpfc_num_parameters_idc greater than 52 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_num_parameters_idc greater than 52.

If the value of nnpfc_num_parameters_idc is greater than zero, the variable maxNumParameters is derived as follows:

maxNumParameters = ( 2048  <<  nnpfc_num_parameters_idc ) − 1	(93)

It is a requirement of bitstream conformance that the number of neural network parameters of the post-processing filter shall be less than or equal to maxNumParameters.

nnpfc_num_kmac_operations_idc greater than 0 indicates that the maximum number of multiply-accumulate operations per sample of the post-processing filter is less than or equal to nnpfc_num_kmac_operations_idc * 1000. nnpfc_num_kmac_operations_idc equal to 0 indicates that the maximum number of multiply-accumulate operations of the network is unknown. The value of nnpfc_num_kmac_operations_idc shall be in the range of 0 to 232 − 1, inclusive.

nnpfc_total_kilobyte_size greater than 0 indicates a total size in kilobytes required to store the uncompressed parameters for the neural network. The total size in bits is a number equal to or greater than the sum of bits used to store each parameter. nnpfc_total_kilobyte_size is the total size in bits divided by 8000, rounded up. nnpfc_total_kilobyte_size equal to 0 indicates that the total size required to store the parameters for the neural network is unknown. The value of nnpfc_total_kilobyte_size shall be in the range of 0 to 232 − 1, inclusive.

…

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).