| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 29th Meeting, by teleconference, 11-20 January 2023. | Document: JVET-AC0130-v2 |
| --- | --- |

| Title: | EE2-4.2: Non-Separable Primary Transform for Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Patrick Garus Muhammed Coban Bappaditya Ray Vadim Seregin Marta Karczewicz Moonmo Koo Jie Zhao Jaehyun Lim Seung-Hwan Kim | Tel: Email: | {pgarus, mcoban, bray, vseregin, martak}@qti.qualcomm.com moonmo.koo@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | Qualcomm Incorporated, LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of Test EE2-4.2 on non-separable primary transform (NSPT) for intra coding. In Test 4.2, the application of DCT-II plus LFNST is replaced by KLT-based NSPT for certain block sizes.

Two configurations are tested introducing NSPT for the following block shapes:

Test A: 4x4, 4x8, 8x4 and 8x8,

Test B: 4x4, 4x8, 8x4, 8x8, 4x16, 16x4, 8x16 and 16x8

For both configurations the following results are reported:

Test A: AI: -0.23% (Y), -0.29% (U), -0.29 % (V), 100% (EncT), 97% (DecT).

RA: -0.13% (Y), -0.17% (U), -0.22 % (V), 102% (EncT), 99% (DecT).

Test B: AI: -0.43% (Y), -0.57% (U), -0.53 % (V), 106% (EncT), 99% (DecT).

RA: -0.23% (Y), -0.26% (U), -0.41 % (V), 101% (EncT), 100% (DecT).

The runtimes of the previous experiments are further improved through SIMD optimization:

Test A: AI: -0.23% (Y), -0.29% (U), -0.29 % (V), 99% (EncT), 98% (DecT).

RA: -0.13% (Y), -0.17% (U), -0.22 % (V), 101% (EncT), 99% (DecT).

Test B: AI: -0.43% (Y), -0.57% (U), -0.53 % (V), 103% (EncT), 100% (DecT).

RA: -0.23% (Y), -0.26% (U), -0.41 % (V), 101% (EncT), 100% (DecT).

# Introduction

It was proposed in JVET-AB0175 [1] to replace the separable DCT-II plus LFNST transform combination with NSPT for the block shapes 4x4, 4x8, 8x4 and 8x8, which was subsequently included as Test 4.2 in EE2 [2] in the 28th JVET Meeting. The results of this configuration are reported in Test A.

Furthermore, the NSPT design has been extended to block shapes 4x16, 16x4, 8x16 and 16x8. The results of this configuration are reported in Test B.

# Proposal

The affected block sizes for Test A and B are summarized in Figure 1.

Figure 1: Overview of proposed NSPTs among existing LFNSTs in Test A and Test B.

All NSPTs consist of 35 sets and 3 candidates (similar to the current LFNST). The kernels of have the following shapes:

- NSPT4x4: 16x16

- NSPT4x8/NSPT8x4: 32x20

- NSPT8x8: 64x32

- NSPT4x16/NSPT16x4: 64x24

- NSPT8x16/NSPT16x8: 128x40

Therefore, 12, 32, 40 and 88 coefficients are zeroed-out using NSPT4x8/NSPT8x4, NSPT8x8, NSPT4x16/NSPT16x4 and NSPT8x16/NSPT16x8 respectively.

# Experimental results

Following the common test conditions [3] the subsequent results are reported for all intra and random access configurations on top of ECM-7.0:

## Test A

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.21% | -0.03% | 98% | 99% |
| Class A2 | -0.03% | -0.27% | -0.16% | 99% | 99% |
| Class B | -0.23% | -0.23% | -0.34% | 99% | 99% |
| Class C | -0.53% | -0.56% | -0.61% | 101% | 96% |
| Class E | -0.23% | -0.11% | -0.18% | 98% | 96% |
| Overall | -0.23% | -0.29% | -0.29% | 99% | 98% |
| Class D | -0.51% | -0.78% | -0.69% | 103% | 95% |
| Class F | -0.35% | -0.27% | -0.20% | 98% | 97% |
| Class TGM | -0.04% | -0.02% | -0.05% | 101% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.19% | -0.06% | 100% | 100% |
| Class A2 | -0.04% | 0.16% | -0.02% | 101% | 96% |
| Class B | -0.15% | -0.24% | -0.38% | 100% | 100% |
| Class C | -0.26% | -0.31% | -0.31% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.17% | -0.22% | 101% | 99% |
| Class D | -0.26% | -0.36% | -0.31% | 102% | 101% |
| Class F | -0.26% | -0.37% | -0.18% | 101% | 100% |
| Class TGM | 0.09% | 0.02% | 0.10% | 102% | 103% |

### Test B

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.36% | -0.14% | 104% | 101% |
| Class A2 | -0.19% | -0.48% | -0.43% | 103% | 97% |
| Class B | -0.44% | -0.56% | -0.58% | 106% | 100% |
| Class C | -0.85% | -0.89% | -0.83% | 110% | 100% |
| Class E | -0.40% | -0.47% | -0.52% | 105% | 99% |
| Overall | -0.43% | -0.57% | -0.53% | 106% | 99% |
| Class D | -0.61% | -0.90% | -0.81% | 111% | 102% |
| Class F | -0.57% | -0.30% | -0.26% | 105% | 100% |
| Class TGM | -0.03% | 0.06% | 0.03% | 104% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.27% | -0.08% | 101% | 101% |
| Class A2 | -0.10% | 0.10% | -0.28% | 101% | 101% |
| Class B | -0.27% | -0.34% | -0.49% | 101% | 100% |
| Class C | -0.40% | -0.42% | -0.67% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.26% | -0.41% | 101% | 100% |
| Class D | -0.26% | -0.64% | -0.62% | 102% | 102% |
| Class F | -0.35% | -0.31% | -0.16% | 101% | 100% |
| Class TGM | 0.08% | -0.01% | 0.14% | 103% | 103% |

# Additional results with new SIMD optimization

The SIMD implementation of NSPT is further optimized mainly targeting larger blocks. This change is lossless with respect to the previously reported Test A and B results.

## Test A

|  |  |  | All Intra Main 10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.21% | -0.03% | 98% | 99% |
| Class A2 | -0.03% | -0.27% | -0.16% | 99% | 99% |
| Class B | -0.23% | -0.23% | -0.34% | 99% | 99% |
| Class C | -0.53% | -0.56% | -0.61% | 101% | 96% |
| Class E | -0.23% | -0.11% | -0.18% | 98% | 96% |
| Overall | -0.23% | -0.29% | -0.29% | 99% | 98% |
| Class D | -0.51% | -0.78% | -0.69% | 103% | 95% |
| Class F | -0.35% | -0.27% | -0.20% | 98% | 97% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.19% | -0.06% | 100% | 100% |
| Class A2 | -0.04% | 0.16% | -0.02% | 101% | 96% |
| Class B | -0.15% | -0.24% | -0.38% | 100% | 100% |
| Class C | -0.26% | -0.31% | -0.31% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.17% | -0.22% | 101% | 99% |
| Class D | -0.26% | -0.36% | -0.31% | 102% | 101% |
| Class F | -0.26% | -0.37% | -0.18% | 101% | 100% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Test B

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.36% | -0.14% | 102% | 100% |
| Class A2 | -0.19% | -0.48% | -0.43% | 103% | 100% |
| Class B | -0.44% | -0.56% | -0.58% | 104% | 101% |
| Class C | -0.85% | -0.89% | -0.83% | 105% | 100% |
| Class E | -0.40% | -0.47% | -0.52% | 103% | 100% |
| Overall | -0.43% | -0.57% | -0.53% | 103% | 100% |
| Class D | -0.61% | -0.90% | -0.81% | 107% | 98% |
| Class F | -0.57% | -0.30% | -0.26% | 102% | 99% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.27% | -0.08% | 100% | 101% |
| Class A2 | -0.10% | 0.10% | -0.28% | 101% | 100% |
| Class B | -0.27% | -0.34% | -0.49% | 100% | 100% |
| Class C | -0.40% | -0.42% | -0.66% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.26% | -0.41% | 101% | 100% |
| Class D | -0.26% | -0.64% | -0.62% | 102% | 101% |
| Class F | -0.35% | -0.31% | -0.16% | 101% | 100% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution reports the results of EE2-4.2 on Non-separable Primary Transforms as a replacement for DCT-II plus LFNST for certain block shapes. Two configurations have been evaluated, each providing promising coding gains for all intra as well as random access over ECM-7.0. It is proposed to adopt the NSPT into the next version of ECM.

# References

[1] P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz, “Non-EE2: Non-Separable Primary Transforms for Intra Coding”, JVET-AB0175, Oct. 2022.

[2] V. Seregin, J. Chen, G. Li, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AB2024, Mainz, Oct 2022.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

[4] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0

# Patent rights declaration(s)

Qualcomm and LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).