| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0134-v1 |
| --- | --- |

| Title: | AHG9: Signalling of NNPF quality improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Ye-Kui Wang2, Kai Zhang2, Junru Li3, Yue Li2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | linchaoyi.cy@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com lijunru@bytedance.com yue.li@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current design of the neural network post-filter characteristics (NNPFC) SEI message, the complexity of neural network post filter (NNPF) may be signalled. Given the complexity of NNPF, the decoder might decide whether to apply these post filters. However, if the quality improvement by applying the NNPF is signalled, the decoder could make more educated choice on whether to apply the NNPF. For example, if the complexity is moderate to high, by that information itself the decoder may choose not to apply the NNPF. However, given the same complexity, and at the same time the indicated quality improvement is significant, the decoder may choose to apply the NNPF anyway.

This contribution proposes to enable signalling of information of quality improvement by applying an NNPF. Since this information depends on the set of pictures to which the NNPF is applied. It is proposed to signal the information in the NNPFA SEI message instead of in the NNPFC SEI message. It is proposed to adopt one of the following three options:

- Signal the value difference of one metric

- Signal the value differences of multiple metrics.

- Signal the values of one metric with and without applying NNPF.

# Proposal option 1: signal the value difference of one metric

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_quality_info_present_flag | u(1) |
| if( nnpfa_quality_info_present_flag ) { |  |
| nnpfa_metric_type | u(8) |
| nnpfa_metric_value_diff | i(16) |
| } |  |
| } |  |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

…

nnpfa_quality_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present. nnpfa_quality_info_present_flag equal to 0 specifies that no syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present.

nnpfa_metric_type indicates the type of the objective quality metric as shown in the following table:

Table X – specification of nnpfa_metric_type

| Value | Description |
| --- | --- |
| 0 | PSNR |
| 1 | SSIM |
| 2 | wPSNR |
| 3-255 | User-defined |

nnpfa_metric_value_diff specifies the difference of metric values, with the difference being equal to QWNNPF-QWONNPF, where QWNNPF denotes the averaged metric value across all the pictures to which this NNPFA SEI message applies after applying the associated NNPF and QWONNPF denotes the averaged metric value across the same set of pictures without applying the associated NNPF.

When nnpfa_metric_type is equal to 0, the 16-bit signed integer nnpfa_metric_value_diff is interpreted as a floating-point VPSNRvalue (in dB) as follows, with m set equal to nnpfa metric_value_diff:

VPSNR=m100

When nnpfa_metric_type is equal to 1, the 16-bit signed integer nnpfa_metric_value_diff is interpreted as a floating-point VSSIM value as follows, with m set equal to nnpfa metric_value_diff:

VSSIM=m100

When nnpfa_metric_type is equal to 2, the 16-bit signed integer nnpfa_metric_value_diff is interpreted as a floating-point wPSNR value (in dB) as follows, with m set equal to nnpfa metric_value_diff:

VwPSNR=m100

# Proposal option 2: Signal the value differences of multiple metrics

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_quality_info_present_flag | u(1) |
| if( nnpfa_quality_info_present_flag ) { |  |
| nnpfa_metric_number | u(8) |
| for( n = 0; n < nnpfa_metric_number; n++ ) { |  |
| nnpfa_metric_type[ i ] | u(8) |
| nnpfa_ metric_value_diff[ i ] | i(16) |
| } |  |
| } |  |
| } |  |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

…

nnpfa_quality_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present. nnpfa_quality_info_present_flag equal to 0 specifies that no syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present.

nnpfa_metric_number specifies the number of signalled metric types.

nnpfa_metric_type[ i ] indicates the i-th type of objective quality metric as shown in the following table:

Table X – specification of nnpfa_metric_type

| Value | Description |
| --- | --- |
| 0 | PSNR |
| 1 | SSIM |
| 2 | wPSNR |
| 3-255 | User-defined |

nnpfa_metric_value_diff[ i ] specifies the difference of metric values of the i-th type of objective quality metric, with the difference being equal to QWNNPF-QWONNPF, where QWNNPF denotes the averaged metric value across all the pictures to which this NNPFA SEI message applies after applying associated NNPF and QWONNPF denotes the averaged metric value across the same set of pictures without using associated NNPF.

When nnpfa_metric_type[ i ] is equal to 0, the 16-bit signed integer nnpfa_metric_value_diff[ i ] is interpreted as a floating-point VPSNRvalue (in dB) as follows, with m set equal to nnpfa metric_value_diff[ i ]:

VPSNR=m100

When nnpfa_metric_type[ i ] is equal to 1, the 16-bit signed integer nnpfa_metric_value_diff[ i ] is interpreted as a floating-point VSSIM value as follows, with m set equal to nnpfa metric_value_diff[ i ]:

VSSIM=m100

When nnpfa_metric_type[ i ] is equal to 2, the 16-bit signed integer nnpfa_metric_value_diff[ i ] is interpreted as a floating-point wPSNR value (in dB) as follows, with m set equal to nnpfa metric_value_diff[ i ]:

VwPSNR=m100

# Proposal option 3: Signal the values of one metric with and without applying NNPF

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_quality_info_present_flag | u(1) |
| if( nnpfa_quality_info_present_flag ) { |  |
| nnpfa_metric_type | u(8) |
| nnpfa_metric_value_without_nnpf | u(16) |
| nnpfa_metric_value_with_nnpf | u(16) |
| } |  |
| } |  |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

nnpfa_quality_info_present_flag equal to 1 specifies that one or more syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present. nnpfa_quality_info_present_flag equal to 0 specifies that no syntax elements that indicate the quality information of the post processing filter associated with the nnpfa_target_id are present.

nnpfa_metric_type indicates the type of the objective quality metric as shown in the following table:

Table X – specification of nnpfa_metric_type

| Value | Description |
| --- | --- |
| 0 | PSNR |
| 1 | SSIM |
| 2 | wPSNR |
| 3-255 | User-defined |

nnpfa_metric_value_without_nnpf specifies the averaged metric value across all the pictures to which this NNPFA SEI message applies without applying associated NNPF.

When nnpfa_metric_type is equal to 0, the 16-bit unsigned integer nnpfa metric_value_without_nnpf is interpreted as a floating-point VPSNRvalue (in dB) as follows, with m set equal to nnpfa metric_value_without_nnpf:

VPSNR=m100

When nnpfa_metric_type is equal to 1, the 16-bit unsigned integer nnpfa metric_value_without_nnpf is interpreted as a floating-point VSSIM value as follows, with m set equal to nnpfa metric_value_without_nnpf:

VSSIM=m100

When nnpfa_metric_type is equal to 2, the 16-bit unsigned integer nnpfa metric_value_without_nnpf is interpreted as a floating-point wPSNR value (in dB) as follows, with m set equal to nnpfa metric_value_without_nnpf:

VwPSNR=m100

nnpfa metric_value_with_nnpf specifies the averaged metric value across all the pictures to which this NNPFA SEI message applies by applying the associated NNPF.

When nnpfa_metric_type is equal to 0, the 16-bit unsigned integer nnpfa metric_value_with_nnpf is interpreted as a floating-point VPSNRvalue (in dB) as follows, with m set equal to nnpfa metric_value_with_nnpf:

VPSNR=m100

When nnpfa_metric_type is equal to 1, the 16-bit unsigned integer nnpfa metric_value_with_nnpf is interpreted as a floating-point VSSIM value as follows, with m set equal to nnpfa metric_value_with_nnpf:

VSSIM=m100

When nnpfa_metric_type is equal to 2, the 16-bit unsigned integer nnpfa metric_value_with_nnpf is interpreted as a floating-point wPSNR value (in dB) as follows, with m set equal to nnpfa metric_value_with_nnpf:

VwPSNR=m100

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).