| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0135-v1 |
| --- | --- |

| Title: | AHG9: Separate activation of colour components for neural-network post-filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Ye-Kui Wang2, Kai Zhang2, Junru Li3, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com lijunru@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, all colour components are controlled collectively in the neural-network post-filter activation (NNPFA) SEI message. With such a design, all of, or none of, the colour components should be processed by the neural-network post-processing filter (NNPF). However, the NNPF may degrade the performance of one colour component while improving the performance of the others in certain cases. Therefore, it is beneficial to only activate the target NNPF for certain colour components.

This contribution proposes to provide the capability of specifying activation of NNPF separately for different components (luma or chroma) in the NNPFA SEI message. It is proposed to use one of the following two options:

1) Signal an indicator to indicate which of the components present in the output tensor are used.

2) Signal four flags to indicate which of the components present in the output tensor are used.

# Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

# Solution 1: Signal nnpfa_output_component

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_output_component | ue(v) |
| } |  |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

…

nnpfa_output_component indicates the activated components present in the output tensor as specified in Table 22. The value of nnpfa_output_component shall be in the range of 0 to 6, inclusive.

Table 22 – Definition of nnpfa_output_component

| Value | Interpretation |
| --- | --- |
| 0 | All the components present in the output tensor are used. |
| 1 | Only the Cb and Cr components present in the output tensor are used. |
| 2 | Only the Cb components present in the output tensor are used. |
| 3 | Only the Cr components present in the output tensor are used. |
| 4 | Only the luma component present in the output tensor is used. |
| 5 | Only the luma and Cb components in the output tensor are used. |
| 6 | Only the luma and Cr components present in the output tensor are used. |

# Solution 2: Signal four flags

8.29.1 Neural-network post-filter activation SEI message syntax

| nn_post_filter_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfa_target_id | ue(v) |
| nnpfa_cancel_flag | u(1) |
| if( !nnpfa_cancel_flag ) { |  |
| nnpfa_persistence_flag | u(1) |
| nnpfa_default_output_flag | u(1) |
| if( !nnpfa_default_output_flag ) { |  |
| nnpfa_luma_output_flag | u(1) |
| nnpfa_cb_output_flag | u(1) |
| nnpfa_cr_output_flag | u(1) |
| } |  |
| } |  |
| } |  |

8.29.2 Neural-network post-filter activation SEI message semantics

…

nnpfa_default_output_flag equal to 1 specifies that all the components present in the output tensor are used. nnpfa_default_output_flag equal to 0 indicates that some of the components present in the output tensor may or may not be used.

nnpfa_luma_output_flag equal to 1 specifies that the luma component present in the output tensor is used. nnpfa_luma_output_flag equal to 0 specifies that the luma component present in the output tensor is not used.

nnpfa_cb_output_flag equal to 1 specifies that the Cb components present in the output tensor are used. nnpfa_cb_output_flag equal to 0 specifies that the Cb components present in the output tensor are not used.

nnpfa_cr_output_flag equal to 1 specifies that the Cr components present in the output tensor are used. nnpfa_Cr_output_flag equal to 0 specifies that the Cr components present in the output tensor are not used.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).