| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0136-v1 |
| --- | --- |

| Title: | AHG9: On additional NNPF purposes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Ye-Kui Wang2, Kai Zhang2, Yue Li2, Junru Li3, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA 3Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | linchaoyi.cy@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com yue.li@bytedance.com lijunru@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, some neural network post-filter (NNPF) purposes, such as quality improvements, resolution upsampling, and picture rate upsampling, are defined. However, it is asserted that more purposes could be added, as indicated by the following sentences in the meeting minutes of the previous JVET meeting:

"There are many other potential purposes of post processing, e.g. artistic effects (style generation), de-hazing, view synthesis, object tracking, annotation, … all these would currently fall under the “unspecified” category."

"Further study appears necessary to come up with a list of purposes that is useful for the current version of this SEI message."

For example, machine vision tasks are widely used, and they could be added as additional NNPF purposes. Besides, style transfer and object removal are also considered to be additional NNPF purpose.

This contribution proposes to signal machine vision tasks as an additional purpose for post processing filter in NNPFC SEI message. It is proposed to use one of the following three options:

- Add style transfer and object removal as additional NNPF purposes.

- Add machine vision tasks as an additional NNPF purpose and add further indicate the type of the machine vision task.

- Change the purpose of visual quality improvement to be visual information processing, and further indicate the visual information processing type.

# Proposal option 1

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

8.28.2 Neural-network post-filter activation SEI message semantics

…

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5, inclusive, in bitstreams conforming to this edition of this document. Values of 6 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |
| 6 | Style transfer |
| 7 | Object removal task |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

…

# Proposal option 2

The proposed text changes for this option are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts.

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| if( nnpfc_purpose = = 6 ) |  |
| nnpfc_machine_vision_task_type | ue(v) |
| nnpfc_component_last_flag | u(1) |
| … |  |

8.28.2 Neural-network post-filter activation SEI message semantics

...

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5, inclusive, in bitstreams conforming to this edition of this document. Values of 6 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |
| 6 | Machine vision tasks |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

nnpfc_machine_vision_task_type indicates the type of machine vision task as specified in Table 21-x. The value of nnpfc_machine_vision_task_type shall be in the range of 0 to 255, inclusive. Values of nnpfc_machine_vision_task_type that do not appear in Table 21-x are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification encountering a value of nnpfc_machine_vision_task_type greater than 3 shall ignore it.

Table 21-x – Definition of  nnpfc_machine_vision_task_type

| Value | Interpretation |
| --- | --- |
| 0 | Enhancement task, targets enhancing pictures to provide better input for other processing techniques. For example, Edge enhancement may be defined as such an enhancement task. |
| 1 | Analysis task, targets changing the representation of an image into something that is more meaningful and easier to analyze. Face recognition, object recognition, object tracking, object segmentation, etc. may be examples of such tasks. |

NOTE x – When a reserved value of nnpfc_machine_vision_task_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_machine_vision_task_type being equal to that value.

# Proposal option 3

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_formatting_and_purpose_flag | u(1) |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 1 ) |  |
| nnpfc_visual_information_processing_type | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| nnpfc_component_last_flag | u(1) |
| … |  |

8.28.2 Neural-network post-filter activation SEI message semantics

...

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5, inclusive, in bitstreams conforming to this edition of this document. Values of 6 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement Visual information processing |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |

NOTE 3– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

When SubWidthC is equal to 1 and SubHeightC is equal to 1, nnpfc_purpose shall not be equal to 2 or 4.

nnpfc_visual_information_processing_type indicates the type of visual information processing as specified in Table 21-x. The value of nnpfc_visual_information_processing_type shall be in the range of 0 to 255, inclusive. Values of nnpfc_visual_information_processing_type that do not appear in Table 21-x are reserved for future specification by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification encountering a value of nnpfc_visual_information_processing_type greater than 3 shall ignore it.

Table 21-x – Definition of nnpfc_visual_information_processing_type

| Value | Interpretation |
| --- | --- |
| 0 | General visual quality improvement, targeting at increasing either the fidelity or the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The improvement may be measured by any objective or subjective metric. |
| 1 | Objective-oriented/fidelity-oriented visual quality improvement, targeting at increasing the fidelity of the reconstructed picture after applying the neural-network post-processing filter. The fidelity may be measured by PSNR, Ms-SSIM etc. |
| 2 | Subjective-oriented visual quality improvement, targeting at increasing the subjective visual quality of the reconstructed picture after applying the neural-network post-processing filter. The subjective visual quality may be measured by LPIPS or MOS. The dehazing and deraining tasks could be treated as subjective-oriented visual quality improvement. |
| 3 | Film grain-oriented visual quality improvement, with synthesizing filter grain on the reconstructed picture after applying the neural-network post-processing filter. |
| 4 | Machine vision-oriented processing and the purpose is to improve the performance of general machine vision tasks, such as image/video understanding/recognition/detect/segmentation. |
| 5 | Change the appearance or visual style of input image/video. The image/video style transfer task and object removal task are included in this type. |

NOTE x – When a reserved value of nnpfc_visual_information_processing_type is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_visual_information_processing_type being equal to that value.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).