| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0138-v1 |
| --- | --- |

| Title: | Adjusting luma/chroma BD-rate balance in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard Francois, Ya Chen, Charles Salmon-Legagneur | Tel: Email: | [phone contact number] edouard.francois@interdigital.com ya.chen@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports experiments based on ECM-7.0 aiming at obtaining a better repartition of BD-rate variations between luma and chroma, when using VTM-19.0 as reference. As a result, the contribution proposes adjustments of the configuration parameters “CbQpOffset”, “CrQpOffset” and “LMCSOffset” for the configurations of the ECM CTCs for SDR and HDR contents.

# Introduction

New cross-component tools in ECM bring significant chroma gains. Using the current CTCs, it is observed that the luma/chroma BD-rate (BDR) variations of ECM-7.0 compared to VTM-19.0 are not well balanced. Chroma coding BDR gain is noticeably more important than luma coding BDR gain, as depicted in Table 1 for SDR content. Same observation for HDR content is shown in Table 2.

Besides the chroma QP offset parameters CbQpOffset and CrQpOffset allowing adjusting the luma/chroma balance, the LMCS parameter LMCSOffset allows for a fine tuning of the chroma residual scaling and therefore allows finely adjusting the balance between luma and chroma BDR variations as well. Increasing the CbQpOffset and CrQpOffset values results in shifting some chroma coding gains towards the luma component, and vice-versa. Decreasing the LMCSOffset value results in a lower chroma residual scaling at encoder, and therefore results in favouring luma compared to chroma, and vice-versa.

Results with various settings of this parameter are reported in the next section, and recommendations for new settings in CTCs are proposed.

Table 1. BD-rate variations of ECM-7.0 using VTM-19.0 as reference under SDR test conditions.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.67% | -17.52% | -24.10% | 553% | 346% |
| Class A2 | -12.88% | -21.39% | -24.64% | 526% | 328% |
| Class B | -7.62% | -20.59% | -18.79% | 508% | 340% |
| Class C | -8.12% | -12.50% | -12.91% | 499% | 313% |
| Class E | -9.72% | -17.97% | -16.91% | 479% | 357% |
| Overall | -9.13% | -17.98% | -19.03% | 511% | 336% |
| Class D | -6.68% | -10.00% | -10.01% | 469% | 274% |
| Class F | -19.56% | -27.17% | -26.59% | 346% | 325% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.25% | -21.63% | -28.78% | 582% | 691% |
| Class A2 | -21.56% | -27.89% | -31.05% | 547% | 849% |
| Class B | -17.64% | -27.90% | -25.77% | 590% | 702% |
| Class C | -18.92% | -22.19% | -21.92% | 588% | 691% |
| Class E |  |  |  |  |  |
| Overall | -19.08% | -25.12% | -26.40% | 579% | 724% |
| Class D | -19.83% | -23.16% | -23.32% | 570% | 679% |
| Class F | -23.19% | -29.37% | -29.62% | 480% | 418% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.42% | -30.26% | -28.00% | 512% | #DIV/0! |
| Class C | -17.49% | -25.37% | -25.13% | 564% | #DIV/0! |
| Class E | -14.58% | -21.91% | -22.91% | 542% | #DIV/0! |
| Overall | -16.32% | -26.54% | -25.77% | 536% | #DIV/0! |
| Class D | -19.51% | -25.88% | -25.96% | 568% | #DIV/0! |
| Class F | -21.09% | -29.49% | -28.27% | 503% | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -18.47% | -25.50% | -32.36% | 531% | #DIV/0! |
| Class C | -17.07% | -24.43% | -24.07% | 475% | #DIV/0! |
| Class E | -14.40% | -21.12% | -22.06% | 503% | #DIV/0! |
| Overall | -16.98% | -24.05% | -27.02% | 505% | #DIV/0! |
| Class D | -19.82% | -26.15% | -25.72% | 468% | #DIV/0! |
| Class F | -20.37% | -29.13% | -28.12% | 502% | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2. BD-rate variations of ECM-7.0 using VTM-19.0 as reference under HDR test conditions.

# Experiments results

In the reported experiments, reference is VTM-19.0. Test is ECM-7.0, with various values of the parameters CbQpOffset/CrQpOffset (CbQpOffset/CrQpOffset can vary from –12 to 12) and LMCSOffset (LMCSOffset can vary from -7 to 7). Simulations have been performed for AI, RA, LDB and LDP configurations, for ECM CTC test content.

## SDR content

The original and recommended settings for AI, RA, LDB and LDP configurations are depicted in Table 3. Full results are provided in attached files “VTM-19.0_vs_ECM-7.0_SDR_QpOffset_LMCSOffset.xlsm”.

Table 3. proposed SDR CTC modifications with modified CbQpOffset, CrQpOffset, LMCSOffset parameters.

|  |  | CbQpOffset | CrQpOffset | LMCSOffset |
| --- | --- | --- | --- | --- |
| AI | ECM-7.0 | 0 | 0 | 2 |
|  | proposed config | 1 | 1 | 5 |
| RA | ECM-7.0 | 0 | 0 | 6 |
|  | proposed config | 0 | 0 | -7 |
| LDB | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 0 | 0 | -7 |
| LDP | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 0 | 0 | -7 |

All Intra case

For AI configuration, CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 5 provides a good balance between luma and chroma BDR variations, as shown in Table 4.

| AI ECM-7 vs VTM-19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 2 | -9.13% | -17.98% | -19.03% | -18.50% |
| Proposed | 1 | 1 | 5 | -10.08% | -10.19% | -11.66% | -10.93% |

Random Access case

For RA configuration, CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7 provides a good balance between luma and chroma BDR variations, as shown in Table 4.

| RA ECM-7 vs VTM-19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 6 | -19.08% | -25.12% | -26.40% | -25.76% |
| Proposed | 0 | 0 | -7 | -19.25% | -22.63% | -24.01% | -23.32% |

Low Delay B case

For LDB configuration, CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7 provides a good balance between luma and chroma BDR variations, as shown in Table 4.

| LDB ECM-7 vs VTM-19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 1 | -16.32% | -26.54% | -25.77% | -26.16% |
| Proposed | 0 | 0 | -7 | -16.48% | -21.99% | -21.47% | -21.73% |

Low Delay P case

For LDP configuration, CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7 provides a good balance between luma and chroma BDR variations, as shown in Table 4.

| LDP ECM-7 vs VTM-19 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 1 | -16.98% | -24.05% | -27.02% | -25.53% |
| Proposed | 0 | 0 | -7 |  |  |  |  |

Table 4. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-19.0 as reference under SDR test conditions.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.91% | -8.01% | -18.01% | 569% | 374% |
| Class A2 | -14.59% | -13.75% | -18.04% | 541% | 355% |
| Class B | -8.32% | -12.76% | -10.03% | 510% | 358% |
| Class C | -8.89% | -5.23% | -5.55% | 509% | 338% |
| Class E | -10.29% | -11.17% | -9.78% | 498% | 373% |
| Overall | -10.08% | -10.19% | -11.66% | 523% | 358% |
| Class D | -7.44% | -2.47% | -1.98% | 491% | 326% |
| Class F | -20.30% | -21.81% | -21.32% | 360% | 344% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.45% | -19.41% | -26.42% | 584% | 739% |
| Class A2 | -21.70% | -26.57% | -29.72% | 566% | 899% |
| Class B | -17.77% | -25.38% | -23.45% | 595% | 730% |
| Class C | -19.12% | -18.66% | -18.63% | 573% | 678% |
| Class E |  |  |  |  |  |
| Overall | -19.25% | -22.63% | -24.01% | 581% | 748% |
| Class D | -19.99% | -20.58% | -20.92% | 592% | 767% |
| Class F | -23.52% | -27.23% | -27.64% | 492% | 449% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.52% | -26.54% | -24.22% | 545% | #DIV/0! |
| Class C | -17.81% | -18.84% | -20.10% | 562% | #DIV/0! |
| Class E | -14.66% | -18.61% | -18.71% | 543% | #DIV/0! |
| Overall | -16.48% | -21.99% | -21.47% | 550% | #DIV/0! |
| Class D | -19.75% | -19.05% | -19.29% | 580% | #DIV/0! |
| Class F | -21.48% | -27.26% | -26.41% | 508% | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-19.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class C | -17.33% | -18.34% | -18.70% | 492% | #DIV/0! |
| Class E | -14.47% | -17.02% | -17.34% | 528% | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! |
| Class D | -20.12% | -19.17% | -18.71% | 494% | #DIV/0! |
| Class F | -20.72% | -26.96% | -25.92% | 507% | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## HDR content

The original and recommended settings for PQ and HLG for AI and RA configurations are depicted in Table 5.

Full results are provided in attached files “VTM-19.0_vs_ECM-7.0_HDR_QpOffset_LMCSOffset.xlsm”.

Table 5. proposed HDR CTC modifications with modified CbQpOffset, CrQpOffset, LMCSOffset parameters.

|  |  | CbQpOffset | CrQpOffset | LMCSOffset |
| --- | --- | --- | --- | --- |
| AI/H1 | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 1 | 1 | -1 |
| AI/H2 | ECM-7.0 | 0 | 0 | 0 |
|  | proposed config | 1 | 1 | -1 |
| RA/H1 | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 1 | 1 | -1 |
| RA/H2 | ECM-7.0 | 0 | 0 | 0 |
|  | proposed config | 1 | 1 | -1 |

All Intra case

For AI configuration, CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1 provides a good balance between luma and chroma BDR variations for both H1 and H2 configurations, as shown in Table 6.

Random Access case

For RA configuration, CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1 provides a good balance between luma and chroma BDR variations for both H1 and H2 configurations, as shown in Table 6.

Table 6. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-19.0 as reference under HDR test conditions.

# Conclusions

To get better balance between luma and chroma BDR variations when assessing ECM against VTM19.0, it is proposed to modify the ECM CTCs as follows:

For SDR content

- AI: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 2

 CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 5

- RA: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 6

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

- LDB: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

- LDP: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

For HDR content

- AI/RA: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1 (H1), 0 (H2)

 CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1 (H1), -1 (H2)

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.