| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0138-v2 |
| --- | --- |

| Title: | Adjusting luma/chroma BD-rate balance in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard Francois, Ya Chen, Charles Salmon-Legagneur | Tel: Email: | [phone contact number] edouard.francois@interdigital.com ya.chen@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports experiments based on ECM-7.0 aiming at obtaining a better repartition of BD-rate variations between luma and chroma, when using VTM-11.0ecm6 as reference. As a result, the contribution proposes adjustments of the configuration parameters “CbQpOffset”, “CrQpOffset” and “LMCSOffset” for the configurations of the ECM CTCs for SDR and HDR contents.

Several encoder configurations are tested for SDR contents, leading to the following AI/RA/LB results over VTM-11.0ecm6:

- ECM-7.0: AI: -9.1%/ -18.0% / -19.0%;   RA: -19.2%/ -25.4%/ -26.7%;   LB: -16.2%/ -26.9%/ -26.3%

- config. 1:  AI: -10.1%/ -10.2% / -11.7%; RA: -19.4%/ -22.9%/ -24.3%;   LB: -16.4%/ -22.3%/ -22.1%

- config. 2:  AI: n/a                                      RA: -20.4%/ -15.4%/ -17.5%;  LB: -16.7%/ -16.1%/ -14.7%

One encoder configuration is tested for HDR contents, leading to the following AI/RA results over VTM-11.0ecm6 (provided numbers are BD-rate wPSNR for H1, BD-rate PSNR for H2):

- ECM-7.0: AI/H1: -7.37% / -25.17% / -30.93%; 	AI/H2: -7.23% / -17.05% / -20.56%;

- config:      AI/H1: -8.65% / -8.52% / -15.25%; 	AI/H2: -7.93% / -7.75% / -11.82%;

- ECM-7.0: RA/H1: -13.89% / -31.38% / -37.00%;  	RA/H2: -xx.x%/ -xx.x%/ -xx.x%

- config:      RA/H1: -14.93% / -15.07% / -21.03%;      RA/H2: -xx.x%/ -xx.x%/ -xx.x%

# Introduction

New cross-component tools in ECM bring significant chroma gains. Using the current CTCs, it is observed that the luma/chroma BD-rate (BDR) variations of ECM-7.0 compared to VTM-11.0ecm6 are not well balanced. Chroma coding BDR gain is noticeably more important than luma coding BDR gain, as depicted in Table 1 for SDR content. Same observation for HDR content is shown in Table 2.

Besides the chroma QP offset parameters CbQpOffset and CrQpOffset allowing adjusting the luma/chroma balance, the LMCS parameter LMCSOffset allows for a fine tuning of the chroma residual scaling and therefore allows finely adjusting the balance between luma and chroma BDR variations as well. Increasing the CbQpOffset and CrQpOffset values results in shifting some chroma coding gains towards the luma component, and vice-versa. Decreasing the LMCSOffset value results in a lower chroma residual scaling at encoder, and therefore results in favouring luma compared to chroma, and vice-versa.

Results with various settings of this parameter are reported in the next section, and recommendations for new settings in CTCs are proposed.

Table 1. BD-rate variations of ECM-7.0 using VTM-11.0ecm6 as reference under SDR test conditions.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.67% | -17.52% | -24.11% | 632% | 411% |
| Class A2 | -12.88% | -21.39% | -24.64% | 582% | 386% |
| Class B | -7.62% | -20.59% | -18.79% | 638% | 438% |
| Class C | -8.12% | -12.50% | -12.91% | 615% | 421% |
| Class E | -9.72% | -17.97% | -16.91% | 588% | 473% |
| Overall | -9.13% | -17.98% | -19.03% | 614% | 426% |
| Class D | -6.68% | -10.00% | -10.01% | 593% | 391% |
| Class F | -18.45% | -26.68% | -26.24% | 449% | 435% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.46% | -21.91% | -29.05% | 666% | 768% |
| Class A2 | -21.45% | -28.23% | -31.25% | 646% | 1022% |
| Class B | -17.65% | -28.04% | -25.93% | 667% | 889% |
| Class C | -19.26% | -22.56% | -22.39% | 649% | 877% |
| Class E |  |  |  |  |  |
| Overall | -19.20% | -25.39% | -26.67% | 658% | 885% |
| Class D | -20.25% | -23.80% | -24.06% | 625% | 984% |
| Class F | -22.05% | -29.05% | -29.34% | 554% | 545% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.16% | -30.50% | -28.78% | 583% | 746% |
| Class C | -17.44% | -25.63% | -25.66% | 602% | 781% |
| Class E | -14.58% | -22.41% | -23.17% | 611% | 498% |
| Overall | -16.19% | -26.85% | -26.34% | 596% | 685% |
| Class D | -19.65% | -26.43% | -26.39% | 585% | 881% |
| Class F | -20.49% | -29.26% | -27.88% | 558% | 528% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.41% | -30.16% | -27.87% | 524% | 741% |
| Class C | -16.91% | -24.79% | -24.39% | 491% | 762% |
| Class E | -14.28% | -22.01% | -22.86% | 550% | 519% |
| Overall | -15.63% | -26.33% | -25.46% | 519% | 684% |
| Class D | -19.78% | -26.11% | -26.07% | 477% | 796% |
| Class F | -19.49% | -28.95% | -27.62% | 542% | 499% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. BD-rate variations of ECM-7.0 using VTM-11.0ecm6 as reference under HDR test conditions.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11_ANC HDR |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -19.91% | -7.99% | -7.37% | -25.17% | -30.93% | -7.45% | -24.81% | -30.44% | 570% | 393% |
| Class H2 |  |  |  |  |  | -7.23% | -17.05% | -20.56% | 589% | 417% |
| Overall | -19.91% | -7.99% | -7.37% | -25.17% | -30.93% | -7.37% | -21.99% | -26.84% | 577% | 401% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM11_ANC HDR |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -18.23% | -8.29% | -13.89% | -31.38% | -37.00% | -13.90% | -30.46% | -35.81% | 480% | 687% |
| Class H2 |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | -18.23% | -8.29% | -13.89% | -31.38% | -37.00% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

(note: HLG RA results are not yet available)

# Experiments results

In the reported experiments, reference is VTM-11.0ecm6. Test is ECM-7.0, with various values of the parameters CbQpOffset/CrQpOffset (CbQpOffset/CrQpOffset can vary from –12 to 12) and LMCSOffset (LMCSOffset can vary from -7 to 7). Simulations have been performed for AI, RA, LDB and LDP configurations, for ECM CTC test content.

## SDR content

The original and recommended settings for AI, RA, LDB and LDP configurations are depicted in Table 3.

Full results are provided in attached files “VTM-11.0ecm6_vs_ECM-7.0_SDR_QpOffset_LMCSOffset.xlsm” and “VTM-11.0ecm6_vs_ECM-7.0_SDR_QpOffset_LMCSOffset_proposed_cfg2.xlsm”.

Table 3. proposed SDR CTC modifications with modified CbQpOffset, CrQpOffset, LMCSOffset parameters.

|  |  | CbQpOffset | CrQpOffset | LMCSOffset |
| --- | --- | --- | --- | --- |
| AI | ECM-7.0 | 0 | 0 | 2 |
|  | proposed config | 1 | 1 | 5 |
| RA | ECM-7.0 | 0 | 0 | 6 |
|  | proposed config 1 | 0 | 0 | -7 |
|  | proposed config 2 | 1 | 1 | 7 |
| LDB | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config 1 | 0 | 0 | -7 |
|  | proposed config 2 | 1 | 1 | 7 |
| LDP | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 0 | 0 | -7 |

All Intra case

For AI configuration, there is around 9-10% imbalance between luma and chroma. The proposed configuration “CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 5” provides a good balance between luma and chroma BDR variations, as shown in Table 4. From the result, it can be seen the proposed configuration allows shifting around 10% chroma gain towards the 1% luma gain.

Table 4. BD-rate variation of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-11ecm6 as reference under SDR AI test condition.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.91% | -8.00% | -18.02% | 651% | 443% |
| Class A2 | -14.59% | -13.75% | -18.03% | 599% | 418% |
| Class B | -8.32% | -12.76% | -10.03% | 641% | 461% |
| Class C | -8.89% | -5.23% | -5.55% | 628% | 454% |
| Class E | -10.29% | -11.17% | -9.78% | 611% | 494% |
| Overall | -10.08% | -10.19% | -11.66% | 628% | 454% |
| Class D | -7.44% | -2.47% | -1.98% | 622% | 467% |
| Class F | -19.20% | -21.31% | -20.99% | 468% | 459% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| AI Proposed vs ECM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Proposed | 1 | 1 | 5 | -1.07% | 10.32% | 9.56% | 9.94% |

Random Access case

For RA configuration, there is around 6-7% imbalance between luma and chroma. The proposed configuration 1 “CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7 (Proposed 1)” provides a gentle balance shift between luma and chroma BDR variations, and the proposed configuration 2 “CbQpOffset = 1, CrQpOffset=1 and LMCSOffset =7 (Proposed 2)” provides a stronger balance shift between luma and chroma BDR variations (luma gain would be beyond 20%, while chroma gain would be lower than the luma gain), as shown in Table 5. From the results, it can be seen the proposed configurations allow shifting around 10% chroma gain towards the 1% luma gain.

Table 5. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-11ecm6 as reference under SDR RA test condition.

|  | Random Access Main 10 (Proposed 1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.66% | -19.69% | -26.70% | 668% | 768% |
| Class A2 | -21.59% | -26.92% | -29.92% | 669% | 1022% |
| Class B | -17.78% | -25.53% | -23.62% | 673% | 889% |
| Class C | -19.46% | -19.04% | -19.10% | 633% | 877% |
| Class E |  |  |  |  |  |
| Overall | -19.37% | -22.91% | -24.29% | 660% | 885% |
| Class D | -20.41% | -21.23% | -21.67% | 649% | 984% |
| Class F | -22.39% | -26.90% | -27.35% | 568% | 545% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Random Access Main 10 (Proposed 2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -21.63% | -11.66% | -22.25% | 678% | 763% |
| Class A2 | -23.18% | -19.91% | -24.03% | 674% | 1024% |
| Class B | -18.23% | -17.60% | -15.06% | 699% | 936% |
| Class C | -19.97% | -12.16% | -11.91% | 667% | 950% |
| Class E |  |  |  |  |  |
| Overall | -20.36% | -15.43% | -17.45% | 681% | 919% |
| Class D | -20.91% | -13.50% | -13.54% | 666% | 1002% |
| Class F | -23.01% | -22.03% | -22.36% | 587% | 570% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| RA Proposed vs ECM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Proposed 1 | 0 | 0 | -7 | -0.21% | 3.46% | 3.23% | 3.34% |
| Proposed 2 | 1 | 1 | 7 | -1.43% | 13.79% | 12.73% | 13.26% |

Low Delay B case

For LDB configuration, there is around 10% imbalance between luma and chroma. The proposed configuration 1 “CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7 (Proposed 1)” provides a gentle balance shift between luma and chroma BDR variations, and the proposed configuration 2 “CbQpOffset = 1, CrQpOffset=1 and LMCSOffset =7 (Proposed 2)” provides a stronger balance shift between luma and chroma BDR variations (chroma gain would be slightly lower than the luma gain), as shown in Table 6. From the results, it can be seen the proposed configurations allow shifting around 26-30% chroma gain towards the 1% luma gain.

Table 6. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-11ecm6 as reference under SDR LDB test condition.

|  | Low delay B Main 10 (Proposed 1) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.26% | -26.78% | -25.05% | 619% | 816% |
| Class C | -17.76% | -19.12% | -20.68% | 599% | 775% |
| Class E | -14.66% | -19.14% | -18.98% | 612% | 496% |
| Overall | -16.36% | -22.31% | -22.08% | 611% | 708% |
| Class D | -19.88% | -19.69% | -19.71% | 598% | 849% |
| Class F | -20.89% | -27.07% | -26.05% | 564% | 502% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main 10 (Proposed 2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -16.66% | -18.93% | -16.44% | 609% | 768% |
| Class C | -18.10% | -14.52% | -13.77% | 618% | 779% |
| Class E | -14.87% | -13.47% | -13.02% | 637% | 508% |
| Overall | -16.69% | -16.09% | -14.70% | 619% | 696% |
| Class D | -20.10% | -15.45% | -14.03% | 611% | 877% |
| Class F | -21.22% | -21.50% | -20.41% | 591% | 532% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| LDB Proposed vs ECM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Proposed 1 | 0 | 0 | -7 | -0.21% | 6.53% | 6.05% | 6.29% |
| Proposed 2 | 1 | 1 | 7 | -0.60% | 15.47% | 16.23% | 15.85% |

Low Delay P case

For LDP configuration, there is around 9-10% imbalance between luma and chroma. The proposed configuration “CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7” provides a gentle balance shift between luma and chroma BDR variations, as shown in Table 7. From the result, it can be seen the proposed configuration allows shifting around 30% chroma gain towards the 1% luma gain.

Table 7. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-11ecm6 as reference under SDR LDP test condition.

|  | Low delay P Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm6 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -15.55% | -26.25% | -24.09% | 528% | 724% |
| Class C | -17.18% | -18.71% | -19.05% | 509% | 771% |
| Class E | -14.34% | -17.95% | -18.16% | 578% | 522% |
| Overall | -15.79% | -21.66% | -20.93% | 534% | 681% |
| Class D | -20.09% | -19.13% | -19.04% | 504% | 850% |
| Class F | -19.85% | -26.81% | -25.43% | 548% | 511% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

| LDP Proposed vs ECM-7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Proposed | 0 | 0 | -7 | -0.20% | 6.64% | 6.24% | 6.44% |

## HDR content

The original and recommended settings for PQ and HLG for AI and RA configurations are depicted in Table 8.

Full results are provided in attached files “VTM-11.0_vs_ECM-7.0_HDR_chQP1_CS-1.xlsm”.

Table 8. proposed HDR CTC modifications with modified CbQpOffset, CrQpOffset, LMCSOffset parameters.

|  |  | CbQpOffset | CrQpOffset | LMCSOffset |
| --- | --- | --- | --- | --- |
| AI/H1 | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 1 | 1 | -1 |
| AI/H2 | ECM-7.0 | 0 | 0 | 0 |
|  | proposed config | 1 | 1 | -1 |
| RA/H1 | ECM-7.0 | 0 | 0 | 1 |
|  | proposed config | 1 | 1 | -1 |
| RA/H2 | ECM-7.0 | 0 | 0 | 0 |
|  | proposed config | 1 | 1 | -1 |

All Intra case

For AI configuration, “CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1” provides a good balance between luma and chroma BDR variations for both H1 and H2 configurations, as shown in Table 9.

| H1 (HDR PQ) – AI ECM-7 vs VTM-11.0ecm6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | wPSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 1 | -7.37% | -25.17% | -30.93% | -28.05% |
| Proposed | 1 | 1 | -1 | -8.65% | -8.52% | -15.25% | -11.88% |

| H2 (HDR HLG) – AI ECM-7 vs VTM-11.0ecm6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 0 | -7.23% | -17.05% | -20.56% | -18.81% |
| Proposed | 1 | 1 | -1 | -7.93% | -7.75% | -11.82% | -9.78% |

Random Access case

For RA configuration, “CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1” provides a good balance between luma and chroma BDR variations for H1 configuration as shown in Table 9. It is expected to provide a good balance between luma and chroma BDR variations for H2 configuration as well.

(note: HLG RA results are not yet available)

| H1 (HDR PQ) – RA ECM-7 vs VTM-11.0ecm6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | wPSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 1 | -13.89% | -31.38% | -37.00% | -34.19% |
| Proposed | 1 | 1 | -1 | -14.93% | -15.07% | -21.03% | -18.05% |

| H2 (HDR HLG) – RA ECM-7 vs VTM-11.0ecm6 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QP_Cb | QP_Cr | LMCS | PSNR |  |  |  |
|  |  |  |  | Y | U | V | UV |
| Original | 0 | 0 | 0 | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Proposed | 1 | 1 | -1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 9. BD-rate variations of ECM-7.0 with modified CbQpOffset, CrQpOffset, LMCSOffset using VTM-11.0ecm6 as reference under HDR test conditions.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM11_ANC HDR |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -11.99% | -9.16% | -8.65% | -8.52% | -15.25% | -8.72% | -8.02% | -13.98% | 543% | 379% |
| Class H2 |  |  |  |  |  | -7.93% | -7.75% | -11.82% | 547% | 392% |
| Overall | -11.99% | -9.16% | -8.65% | -8.52% | -15.25% | -8.43% | -7.92% | -13.19% | 545% | 384% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM11_ANC HDR |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -9.38% | -9.31% | -14.93% | -15.07% | -21.03% | -14.93% | -13.95% | -18.99% | 465% | 680% |
| Class H2 |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | -9.38% | -9.31% | -14.93% | -15.07% | -21.03% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

To get better balance between luma and chroma BDR variations when assessing ECM against VTM-11.0ecm6, it is observed that the ECM CTCs may be modified as follows:

For SDR content

- AI: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 2

 CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 5

- RA: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 6

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

or  CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 7

- LDB: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

or  CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = 7

- LDP: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1

 CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = -7

For HDR content

- AI/RA: CbQpOffset = 0, CrQpOffset=0 and LMCSOffset = 1 (H1), 0 (H2)

 CbQpOffset = 1, CrQpOffset=1 and LMCSOffset = -1 (H1), -1 (H2)

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.