| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0140-v1 |
| --- | --- |

| Title: | EE2-related: Additional results for test EE2-3.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

This contribution reports the additional simulation results of different combinations of Test EE2-3.3 with Test EE2-3.1 and Test EE2-3.4. On top of ECM-7.0, the following BD-rate gain results are reportedly obtained for these combined tests:

- Test EE2-3.3a combined with Test EE2-3.1a: AI configuration: {-0.85%, -0.73%, -0.99%} for class F and {-3.37%, -3.50%, -4.05%} for class TGM; RA configuration: {-0.55%, -0.67%, -0.87%} for class F and {-1.48%, -1.48%, -1.61%} for class TGM;

- Test EE2-3.3b combined with Test EE2-3.1a: AI configuration: {-0.87%, -0.97%, -1.09%} for class F and {-3.55%, -3.67%, -4.26%} for class TGM; RA configuration: {-0.60%, -0.82%, - 0.79%} for class F and {-1.56%, -1.56%, -1.61%} for class TGM;

- Test EE2-3.3c combined with Test EE2-3.1a: AI configuration: {-0.91%, -0.96%, -1.26%} for class F and {-3.63%, -3.70%, -4.35%} for class TGM; RA configuration: {-0.60%, -0.81%, -0.85%} for class F and {-1.66%, -1.63%, -1.74%} for class TGM;

- Test EE2-3.3d combined with Test EE2-3.1a: AI configuration: {-0.92%, -0.99%, -1.12%} for class F and {-3.67%, -3.75%, -4.35%} for class TGM; RA configuration: {-0.64%, -0.84%, - 0.77%} for class F and {-1.66%, -1.62%, -1.76%} for class TGM;

- Test EE2-3.3a combined with Test EE2-3.1b: AI configuration: {-0.81%, -0.65%, -0.92%} for class F and {-3.36%, -3.42%, -3.98%} for class TGM; RA configuration: {-0.60%, -0.82%, - 0.81%} for class F and {-1.44%, -1.36%, -1.49%} for class TGM;

- Test EE2-3.3a combined with Test EE2-3.4: AI configuration: {-0.51%, -0.37%, -0.49%} for class F and {-1.34%, -1.33%, -1.33%} for class TGM; RA configuration: {-0.37%, -0.39%, - 0.42%} for class F and {-0.98%, -0.96%, -0.90%} for class TGM;

- Test EE2-3.3b combined with Test EE2-3.4: AI configuration: {-0.57%, -0.38%, -0.62%} for class F and {-1.53%, -1.49%, -1.51%} for class TGM; RA configuration: {-0.37%, -0.40%, - 0.42%} for class F and {-1.04%, -1.01%, -1.02%} for class TGM;

- Test EE2-3.3c combined with Test EE2-3.4: AI configuration: {-0.59%, -0.43%, -0.43%} for class F and {-1.57%, -1.55%, -1.56%} for class TGM; RA configuration: {-0.41%, -0.57%, - 0.59%} for class F and {-1.15%, -1.12%, -1.05%} for class TGM;

- Test EE2-3.3d combined with Test EE2-3.4: AI configuration: {-0.61%, -0.49%, -0.55%} for class F and {-1.64%, -1.62%, -1.62%} for class TGM; RA configuration: {-0.45%, -0.73%, -0.60%} for class F and {-1.15%, -1.09%, -1.03%} for class TGM.

# Introduction

This contribution reports the simulation results of different combinations of Test EE2-3.3 with Test EE2- 3.1 and Test EE2- 3.4 [1].

# Proposed methods

The descriptions of the techniques included in this proposal can be found in JVET-AC0071, JVET-AC0104, and JVET-AC0060 for Test EE2-3.1, Test EE2-3.3, and Test EE2-3.4, respectively.

# Simulation results

The combined tests listed below were implemented on top of ECM-7.0 and tested under CTC for enhanced compression tool testing [2] for All-Intra and Random-Access configurations. The simulation results are shown in Tables 1-9.

Table 1. The simulation results of Test EE2-3.3a combined with Test EE2-3.1a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.85% | -0.73% | -0.99% | 100% | 101% |
| Class TGM | -3.37% | -3.50% | -4.05% | 100% | 102% |
| Overall | -2.11% | -2.11% | -2.52% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.55% | -0.67% | -0.87% | 100% | 100% |
| Class TGM | -1.48% | -1.48% | -1.61% | 100% | 101% |
| Overall | -1.01% | -1.08% | -1.24% | 100% | 101% |

Table 2. The simulation results of Test EE2-3.3b combined with Test EE2-3.1a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.87% | -0.97% | -1.09% | 101% | 106% |
| Class TGM | -3.55% | -3.67% | -4.26% | 101% | 115% |
| Overall | -2.21% | -2.32% | -2.67% | 101% | 111% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.82% | -0.79% | 102% | 103% |
| Class TGM | -1.56% | -1.56% | -1.61% | 102% | 106% |
| Overall | -1.08% | -1.19% | -1.20% | 102% | 104% |

Table 3. The simulation results of Test EE2-3.3c combined with Test EE2-3.1a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.91% | -0.96% | -1.26% | 102% | 110% |
| Class TGM | -3.63% | -3.70% | -4.35% | 102% | 137% |
| Overall | -2.27% | -2.33% | -2.81% | 102% | 123% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.81% | -0.85% | 102% | 103% |
| Class TGM | -1.66% | -1.63% | -1.74% | 102% | 108% |
| Overall | -1.13% | -1.22% | -1.29% | 102% | 105% |

Table 4. The simulation results of Test EE2-3.3d combined with Test EE2-3.1a

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.92% | -0.99% | -1.12% | 102% | 114% |
| Class TGM | -3.67% | -3.75% | -4.35% | 102% | 154% |
| Overall | -2.30% | -2.37% | -2.74% | 102% | 132% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.64% | -0.84% | -0.77% | 102% | 105% |
| Class TGM | -1.66% | -1.62% | -1.76% | 102% | 112% |
| Overall | -1.15% | -1.23% | -1.26% | 102% | 109% |

Table 5. The simulation results of Test EE2-3.3a combined with Test EE2-3.1b

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.81% | -0.65% | -0.92% | 100% | 101% |
| Class TGM | -3.36% | -3.42% | -3.98% | 101% | 102% |
| Overall | -2.09% | -2.04% | -2.45% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.60% | -0.82% | -0.81% | 100% | 100% |
| Class TGM | -1.44% | -1.36% | -1.49% | 100% | 101% |
| Overall | -1.02% | -1.09% | -1.15% | 100% | 100% |

Table 6. The simulation results of Test EE2-3.3a combined with Test EE2-3.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.51% | -0.37% | -0.49% | 100% | 101% |
| Class TGM | -1.34% | -1.33% | -1.33% | 100% | 102% |
| Overall | -0.92% | -0.85% | -0.91% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | -0.39% | -0.42% | 100% | 100% |
| Class TGM | -0.98% | -0.96% | -0.90% | 100% | 101% |
| Overall | -0.68% | -0.67% | -0.66% | 100% | 100% |

Table 7. The simulation results of Test EE2-3.3b combined with Test EE2-3.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.57% | -0.38% | -0.62% | 101% | 101% |
| Class TGM | -1.53% | -1.49% | -1.51% | 101% | 114% |
| Overall | -1.05% | -0.93% | -1.06% | 101% | 107% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.37% | -0.40% | -0.42% | 101% | 102% |
| Class TGM | -1.04% | -1.01% | -1.02% | 102% | 104% |
| Overall | -0.71% | -0.71% | -0.72% | 101% | 103% |

Table 8. The simulation results of Test EE2-3.3c combined with Test EE2-3.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.59% | -0.43% | -0.43% | 102% | 108% |
| Class TGM | -1.57% | -1.55% | -1.56% | 102% | 135% |
| Overall | -1.08% | -0.99% | -0.99% | 102% | 121% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.41% | -0.57% | -0.59% | 103% | 105% |
| Class TGM | -1.15% | -1.12% | -1.05% | 103% | 109% |
| Overall | -0.78% | -0.85% | -0.82% | 103% | 107% |

Table 9. The simulation results of Test EE2-3.3d combined with Test EE2-3.4

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.61% | -0.49% | -0.55% | 102% | 113% |
| Class TGM | -1.64% | -1.62% | -1.62% | 102% | 154% |
| Overall | -1.12% | -1.06% | -1.08% | 102% | 132% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.45% | -0.73% | -0.60% | 102% | 105% |
| Class TGM | -1.15% | -1.09% | -1.03% | 102% | 111% |
| Overall | -0.80% | -0.91% | -0.82% | 102% | 108% |

# Conclusion

This contribution reports the simulation results of different combinations of Test EE2-3.3 with Test EE2- 3.1 and Test EE2- 3.4. The experimental results show significant BD-bitrate saving. Thus, we recommend adopting the proposed techniques into the next ECM release.

# References

- V. Seregin et al., “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AB2024, October 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Feb. 2022.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).