| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0141-v1 |
| --- | --- |

| Title: | AHG9: Alternative Picture Timing SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Jingying Gao Chong Soon Lim Kiyofumi Abe | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes an Alternative Picture Timing SEI message. In the case if the video bitstream contains slow-motion scenes, this SEI can indicate the actual motion speed of the content at captured time. Two aspects are included. In the first aspect, a time scale factor is introduced. This time scale factor is used to indicate the time scale difference between the Picture Timing SEI and the Alternative Picture Timing SEI. In the second aspect, level_idc may be added for every temporal sublayer. Second aspect is to allow update of the level_idc if the decoding level needs to be changed based on the time scale factor. The proposal suggests to adopt aspect 1 and aspect 2 into VVC specification.

# Motivation

Typically, video content is captured at 60fps or 30fps, and the captured video content is set to output at the same frame rate.

Invented in the early 20th century, slow-motion, is an effect in film-making whereby time appears to be slowed down. This can be accomplished using high-speed cameras for encoding and playing the content at a lower rate. Slow-motion can also be achieved by playing a normally recorded content at a slower speed. Slow-motion is useful to make it easier to see detail movement in fast motion.

Camera systems from many manufacturers supports this capability. Mobile phones from various manufacturers also supports different slow-motion capturing rates.

# Current limitation and problem

Based on current VVC [1] and VSEI [2] specifications, only one set of decoding time and output time can be present in the bitstream.

In the case if the video bitstream contains slow-motion scenes, it is not possible to retrieve the ‘live’ or actual motion speed of the content at captured time.

# Proposal

We propose to add alternative picture timing SEI to introduce a second picture timing. Typically, this SEI is signalled in the first picture of the CLVS. The second picture timing can be derived through the signalled time scale factor. The original picture timing SEI will contain the intended decoding and output time. The alternative picture timing SEI can be used to store the ‘live’ or actual speed of the content. This SEI can also be used to indicate another viewing speed as well.

Sublayer functionality is considered in this proposal. In the case when the decoding level is raised due to the changed in time scale factor, the new level can be signalled in the alternative picture timing SEI.

For a simple illustration of the use case, a simple GOP structure with 2 sublayers is used.

## Aspect 1

Aspect 1 introduces a time scale factor which is used to derive the second picture timing. In this example, the second picture timing is raised to decode and output the pictures 2 times faster.

## Aspect 2

Aspect 2 introduces a way to update the level_idc value for each sublayer based on the second picture timing. In this example, since the decode and output pictures are 2 times faster, the original level_idc value (level 4.1) signalled in the VVC header may need to be increased accordingly.

Decoders at this point can check each sublayer’s level_idc and decode at their designated conformance point. If the decoder cannot operate at the indicated level_idc, the decoder may only decode the sublayers which are within the indicated level_idc. In this case, if the decoder can only operate within level_idc of 4.1, temporal layer 1 may be skipped so that the video can still be decodable.

## Proposed syntax and semantics additions to VVC [1]

D.x.1	Alternative picture timing SEI message syntax

| alternative_pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| apt_time_scale_factor_nominator | u(10) |
| apt_time_scale_factor_denominator | u(10) |
| apt_time_interpretation | u(2) |
| apt_sublayer_info_present_flag | u(1) |
| if( apt_sublayer_info_present_flag ) { |  |
| apt_max_sublayers_minus1 | u(3) |
| for( i = apt_max_sublayers_minus1; i >= 0; i− − ) |  |
| apt_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| apt_reserved_zero_bit | u(1) |
| for( i = apt_max_sublayers_minus1; i >= 0; i− − ) |  |
| if( apt_sublayer_level_present_flag[ i ] ) |  |
| apt_sublayer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

D.x.2	Alternative picture timing SEI message semantics

The alternative picture timing SEI message indicates the scaling factor to use to obtain an alternative decode and output time of the content e.g. for actual content output speed from the slow-motion output speed.

When an alternative picture timing SEI message is present for any picture of a CLVS of a particular layer, an alternative picture timing SEI message shall be present for the first picture of the CLVS. The alternative picture timing SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All alternative picture timing SEI messages that apply to the same CLVS shall have the same content.

apt_time_scale_factor_nominator and apt_time_scale_factor_denominator specifies the value for nominator and denominator respectively, for the calculation of timeScaleFactor. When apt_time_scale_factor_nominator and apt_time_scale_factor_denominator are not present, they are inferred to be equal to 1. The variable timeScaleFactor is derived as follows:

timeScaleFactor = apt_time_scale_factor_nominator ÷ apt_time_scale_factor_denominator

apt_time_interpretation indicates the intended interpretation of the picture timing SEI and alternative picture timing. When apt_time_interpretation is not present, it is inferred to be equal to 0.

Table 1 - Definition of apt_time_interpretation

| Value | Interpretation |
| --- | --- |
| 0 | Unknown or unspecified |
| 1 | Indicates that picture timing SEI is not ‘live’ time and alternative picture timing SEI is ‘live’ time |
| 2 | Indicates that picture timing SEI is ‘live’ time and alternative picture timing SEI is not ‘live’ time |

apt_sublayer_info_present_flag equal to 1 specifies that information about temporal sub-layers is present in the alternative_pic_time( ) syntax structure. apt_sublayer_info_present_flag equal to 0 specifies that information about temporal sub-layers is not present in the alternative_pic_time( ) syntax structure.

apt_max_sub_layers_minus1 plus 1 specifies the maximum number of temporal sub-layers that may be present in each CLVS referring to the SPS. The value of apt_max_sub_layers_minus1 shall be equal to the value of sps_max_sublayers_minus1.

apt_sublayer_level_present_flag[ i ] equal to 1 specifies that level information is present in the alternative_pic_time( ) syntax structure for the sublayer representation with TemporalId equal to i. apt_sublayer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the alternative_pic_time( ) syntax structure for the sublayer representation with TemporalId equal to i.

apt_reserved_zero_bit shall be equal to 0. The value 1 for apt_reserved_zero_bit is reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore the value of apt_reserved_zero_bit.

The semantics of the syntax element apt_sublayer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sublayer representation with TemporalId equal to i.

When not present, the value of apt_sublayer_level_idc[ i ] is inferred as follows:

–	The value of apt_sublayer_level_idc[ apt_max_sub_layers_minus1 ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	For i from apt_max_sub_layers_minus1 − 1 to 0 (in decreasing order of values of i), inclusive, apt_sublayer_level_idc[ i ] is inferred to be equal to apt_sublayer_level_idc[ i + 1 ].

## Proposed amendments to VVC [1]:

New additions are highlighted in yellow.

…

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ ScIdx ] +

InitCpbRemovalDelayOffset[ ScIdx ] ) ÷ timeScaleFactor ÷ 90000 	(1582)

…

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ ScIdx ] ÷

timeScaleFactor ÷ 90000 )						(1583)

…

if( !decodingUnitParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷	(1584)
			 ( BitRate[ Htid ][ ScIdx ] * timeScaleFactor )
	else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷

( BitRate[ Htid ][ ScIdx ] * timeScaleFactor )

…

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ ScIdx ] ÷ timeScaleFactor ÷ 90000 	(1585)

…

if( !concatenationFlag ) {

baseTime = AuNominalRemovalTime[ firstAuInPrevBuffPeriod ]

tmpCpbRemovalDelay = AuCpbRemovalDelayVal

tmpCpbDelayOffset = CpbDelayOffset

} else {

baseTime1 = AuNominalRemovalTime[ prevNonDiscardableAu ]

tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )

baseTime2 = AuNominalRemovalTime[ n − 1 ]

tmpCpbRemovalDelay2 = Ceil( ( InitCpbRemovalDelay[ ScIdx ] ÷ timeScaleFactor  ÷ 90000 +	(1586)
			AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] )

÷ ( ClockTick ÷ timeScaleFactor ) )

if( baseTime1 + ( ClockTick * tmpCpbRemovalDelay1 ÷ timeScaleFactor )  <

baseTime2 + ( ClockTick * tmpCpbRemovalDelay2 ÷ timeScaleFactor ) ) {

baseTime = baseTime2

tmpCpbRemovalDelay = tmpCpbRemovalDelay2

} else {

baseTime = baseTime1

tmpCpbRemovalDelay = tmpCpbRemovalDelay1

}

tmpCpbDelayOffset = 0

}

AuNominalRemovalTime[ n ] = baseTime + ClockTick *

( tmpCpbRemovalDelay − tmpCpbDelayOffset ) ÷ timeScaleFactor

…

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstAuInCurrBuffPeriod ] +

ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset ) ÷ timeScaleFactor 	(1587)

…

if( bp_du_cpb_params_in_pic_timing_sei_flag )

DuNominalRemovalTime[ m ] = DuNominalRemovalTime[ m + 1 ] −

ClockSubTick * duCpbRemovalDelayInc ÷ timeScaleFactor 	(1588)

else

DuNominalRemovalTime[ m ] = AuNominalRemovalTime[ n ] −

ClockSubTick * duCpbRemovalDelayInc ÷ timeScaleFactor

…

if( !low_delay_hrd_flag[ Htid ]  | |  AuNominalRemovalTime[ n ]  >=  AuFinalArrivalTime[ n ] )

AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ]

else												(1589)

AuCpbRemovalTime[ n ] = AuNominalRemovalTime[ n ] + ClockTick *

Ceil( ( AuFinalArrivalTime[ n ] − AuNominalRemovalTime[ n ] ) ÷

( ClockTick ÷ timeScaleFactor ) ) ÷ timeScaleFactor

…

if( !DecodingUnitHrdFlag ) {

DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * ( pt_dpb_output_delay –	(1591)

AuDpbOutputDelta[ Htid ] ) ÷ timeScaleFactor

if( firstPicInBufferingPeriodFlag )

DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset ÷ timeScaleFactor

} else

DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockSubTick * dpbOutputDuDelay ÷

timeScaleFactor

…

# Conclusion

The alternative picture timing SEI was introduced. It allows VVC to carry more than one set of picture timings. This SEI opens the window for new use cases of VVC. We propose to adopt the solution with aspect 1 and aspect 2 into VVC.

# References

- Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding Editorial Refinements on Draft 10”, JVET-T2001-v2, 20th Meeting, by teleconference, 7–16 October 2020.

- Jill Boyce, Virginie Drugeon, Gary Sullivan, Ye-Kui Wang, “Versatile supplemental enhancement information messages for coded video bitstreams (Draft 5)”, JVET-S2007-v7, 18th Meeting, by teleconference, 15-24 April 2020.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).