| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0142 |
| --- | --- |

| Title: | Non-EE2: Cross-Component Discrete Mapping Model for Screen Content Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bharath Vishwanath, Kai Zhang, Li Zhang 8910 University Center Lane, San Diego, CA, 92122, USA | Tel: Email: | bharath.vishwanath@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, a method of cross-component discrete mapping model (CCDMM) prediction is proposed for screen content coding. Similar to CCLM or CCCM, CCDMM learns a discrete mapping      model between luma and chroma from reconstructed neighbors and derive chroma prediction from the co-located luma block with the model. On top of ECM-7.0, simulation results are reported as:

Class TGM: AI:  0.01%,  0.01%, -0.01%, 101%, 101%; RA:  0.01%,  0.13%, -0.20%, 101%, 100%

Class F: AI: AI: -1.22%, -1.44%, -1.80%, 100%, 101%; RA: -0.39%, -0.33%, -0.61%, 101%, 100%

# Introduction

Cross-component prediction models such as CCLM, CCCM, GLM and their variants learn linear or non-linear models to exploit correlation between luma and chroma channels. However, all these models are continuous, which may not be suitable for screen contents.

# Proposed method

In this contribution, a method of cross-component discrete mapping model (CCDMM) prediction is proposed for screen content coding.

CCDMM consists of following steps:

- Derive a discrete mapping model: Reconstructed neighbors are used to derive a mapping function between luma and chroma. In contrast to continuous models in CCLM or CCCM, the discrete mapping model remembers luma chroma pairs directly, without a continuous function.

- Chroma Prediction: To predict a chroma sample in the current block, its co-located luma value Yc is referred to derive the prediction value with the discrete mapping model. If Yc is not found in the input luma set of the model, {Yc+1, Yc ─1, Yc+2, Yc─2} are checked in order. If all the five values cannot be matched by the model, the prediction is set to be the average value of neighboring reconstructed chroma samples.

Furthermore, five candidate down-sampling filters as shown in Fig. 1 are introduced to generate the best co-located luma sample value for the YCbCr 4:2:0 color format. The encoder tries the candidate down-sampling filters and signals an index of the best filter to the decoder in a similar way to GLM.

Fig. 1 Luma down-sampling positions

# Experimental results

The tests were implemented on top of ECM-7.0 [1]. The simulations were performed following the common test conditions [2]. Table 1 shows the overall results for AI and RA tests compared to ECM-7.0.

Table 1: Performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | 0.01% | -0.01% | 101% | 101% |
| Class TGM | -1.22% | -1.44% | -1.80% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | 0.13% | -0.20% | 101% | 100% |
| Class TGM | -0.39% | -0.33% | -0.61% | 101% | 100% |

# Conclusions

In this contribution, a cross-component prediction method is proposed for screen content coding, which brings significant coding gains with a negligible complexity increase. It is recommended to study in EE2.

# References

[1] ECM-7.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).