| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0143-v1 |
| --- | --- |

| Title: | EE1-1.6: NN chroma model without partitioning input |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Du Liu, Kenneth Andersson, Per Wennersten, Mitra Damghanian, and Ruoyang Yu. | Tel: Email: | +46702672192 {jacob.strom, du.liu, kenneth.r.andersson, per.wennersten, mitra.damghanian, ruoyang.yu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents results for a modification of the Neural Network loopfilter filter set 1, where the partitioning input to the intra chroma network has been removed. It is reported that the intra chroma model has been retrained using the same training procedure as for NNVC-3.0 filter set 1. The reported BDR results compared to NNVC3.0 filter set 1 are (SADL, int16):

RA: 	-0.01% (Y), 0.01% (U), -0.07% (V)
LD: 	-0.01% (Y), 0.03% (U), -0.17% (V)
AI: 	 0.00% (Y), 0.07% (U), -0.24% (V)

Worst case complexity is 661 kMACs/pixel, down from 664 kMACs/pixel in NNVC3.0 filter set 1, a reduction of 0.5%.

# Introduction

At the meeting in Mainz, JVET-AB0053 [1] was adopted. It involved removing the partitioning input to the intra luma NN loopfilter for filter set 1. At the meeting it was requested to try this also for the intra chroma NN loopfilter for filter set 1. This contribution is a response to that request.

# Execution

The intra chroma loopfilter from NNVC2.0, originally proposed in JVET-AA0111 [2] was modified by removing the partitioning input. It was then trained using exactly the same training procedure as in NNVC2.0. Since the same training procedure was used, this makes the results directly comparable with NNVC-3.0 filter set 1.

After training, the model was converted to SADL and then quantized to int16.

Only the intra chroma model was modified. The inter chroma model already lacks the partitioning input. When running inference, the new intra chroma model was used together with the old models from NNVC3.0 (inter chroma, intra luma and inter luma).

The complexity in terms of kMACs/sample and number of parameters was measured for the intra chroma model before and after the modification using the torchsummary library. Before the modification the chroma filter needed 110.2 kMACs/sample and after 107.4 kMACs/sample, a difference of 2.8. Since the intra chroma filter is in the worst-case path, this lowers the worst-case frame-wise complexity value from 525 kMACs/pixel in JVET-AB0053 to (525-2.8). To get the block-wise value we multiply by 1442/1282 and thus get a complexity reduction from 664.4 to 660.9, or about 0.5% of the total kMACs/pixel.

The number of parameters, also measured using torchsummary, was likewise lowered from 1566265 to 1555224, a reduction of 11041. The total number of parameters thus goes down from 6.21⋅106 to 6.20⋅106, or about 0.16%.

# Training and inference information

## Training

| Table 2. Network Information for NN-based video coding tool testing in training stage for the intra chroma model. The inter chroma, inter luma and intra luma models are the same as in NNVC-3.0 filter set 1. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Loss function: | L1 and L2 |
|  | Training time: | 10 hours |
|  | Training data information: | DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1e-4 and decreasing |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

## Inference

| Table 3. Network Information for NN-based video coding tool testing in inference stage for the intra chroma model. The inter chroma, inter luma and inter luma models are the same as in NNVC3.0 filter set 1. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1555224 for the chroma model, 6.20⋅106 in total for all models. |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 3.1⋅106 for the chroma model, 24.80⋅106 bytes for all models |
|  | Multiply Accumulate (MAC) | 107.4 kMAC/s for the chroma model, 661 kMACs/s for all models |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

## Results over NNVC-3.0 filter set 1:

Results for RA over NNVC-3.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01% | 0.06% | 0.19% | 99% | 95% |
| Class A2 | 0.01% | 0.04% | -0.01% | 100% | 100% |
| Class B | -0.03% | 0.21% | -0.21% | 105% | 95% |
| Class C | -0.01% | -0.28% | -0.14% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.07% | 103% | 98% |
| Class D | 0.00% | 0.05% | 0.15% | 105% | 105% |
| Class F | -0.04% | -0.23% | -0.78% | 105% | 100% |

Encoding and decoding times are not reliable.

Results for LDB over NNVC-3.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.21% | 0.06% | 98% | 103% |
| Class C | -0.06% | 0.13% | -0.34% | 103% | 98% |
| Class E | 0.01% | 0.30% | -0.31% | 99% | 95% |
| Overall | -0.01% | 0.03% | -0.17% | 100% | 99% |
| Class D | 0.07% | 0.76% | 0.03% | 108% | 106% |
| Class F | -0.01% | -0.22% | 0.26% | 101% | 99% |

Encoding and decoding times are not reliable.

Results for AI over NNVC-3.0 filter set 1:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.26% | 0.52% | 104% | 99% |
| Class A2 | 0.00% | 0.16% | -0.60% | 102% | 99% |
| Class B | 0.00% | 0.58% | -0.30% | 106% | 103% |
| Class C | 0.00% | -0.64% | -0.37% | 103% | 101% |
| Class E | 0.00% | -0.13% | -0.35% | 103% | 102% |
| Overall | 0.00% | 0.07% | -0.24% | 104% | 101% |
| Class D | 0.00% | -0.27% | -0.37% | 108% | 101% |
| Class F | 0.01% | -0.59% | -1.39% | 106% | 103% |

Encoding and decoding times are not reliable.

## Results over NNVC-3.0 without NN loop filtering

Results for RA over NNVC-3.0 without NN loop filtering:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -8.88% | -15.86% | -18.55% | 197% | 34609% |
| Class A2 | -10.23% | -20.20% | -17.43% | 185% | 34302% |
| Class B | -9.08% | -22.83% | -21.79% | 202% | 35297% |
| Class C | -9.93% | -21.98% | -22.38% | 171% | 34312% |
| Class E |  |  |  |  |  |
| Overall | -9.50% | -20.68% | -20.43% | 189% | 34695% |
| Class D | -11.48% | -23.59% | -23.81% | 162% | 30786% |
| Class F | -5.06% | -12.00% | -11.27% | 260% | 14900% |

Encoding and decoding times are not reliable.

Results for LDB over NNVC-3.0 without NN loop filtering:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.00% | 0.00% | #NUM! | #NUM! |
| Class A2 | 0.00% | 0.00% | 0.00% | #NUM! | #NUM! |
| Class B | -7.82% | -16.13% | -13.75% | 183% | 33525% |
| Class C | -9.20% | -15.13% | -14.85% | 167% | 31303% |
| Class E | -8.79% | -16.90% | -17.58% | 339% | 27366% |
| Overall | -8.52% | -15.99% | -15.07% | 207% | 31146% |
| Class D | -10.76% | -18.78% | -18.47% | 163% | 30759% |
| Class F | -4.95% | -9.53% | -6.14% | 248% | 14120% |

Encoding and decoding times are not reliable.

Results for AI over NNVC-3.0 without NN loop filtering:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -6.59% | -17.79% | -19.53% | 186% | 23908% |
| Class A2 | -6.84% | -20.18% | -18.03% | 150% | 19175% |
| Class B | -6.82% | -21.48% | -21.92% | 149% | 19031% |
| Class C | -7.61% | -19.18% | -22.16% | 135% | 13868% |
| Class E | -10.20% | -20.95% | -20.91% | 156% | 22238% |
| Overall | -7.52% | -20.05% | -20.76% | 153% | 18935% |
| Class D | -7.46% | -19.24% | -21.30% | 131% | 11636% |
| Class F | -5.27% | -12.37% | -12.23% | 124% | 15319% |

Encoding and decoding times are not reliable.

# Conclusion

Given that the partitioning input does not seem to bring any benefit for the intra chroma model, and that removing it provides a small reduction in worst case kMACs/pixel and makes the filter more similar to the luma filters, it is proposed to adopt this proposal into NNVC 3.0 software for filter set 1.

# References

[1] J. Ström, D. Liu, M. Damghanian, K. Andersson, Y. Li, P. Wennersten, R. Yu, “EE1-1.2: NN intra model without attention and partitioning strength”, JVET-AB0053, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 28th Meeting, Mainz, DE, 20–28 October 2022

[2] Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A. M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, “EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation”, JVET-AA0111, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 27th Meeting, by teleconference, 13–22 July 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).