| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0145-v1 |
| --- | --- |

| Title: | AHG4: experiments in preparation of dual-layer VVC visual tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document discusses the quality ladder category of the VVC multi-layer visual tests planned in JVET-AB2021. It proposes a recommended quality ladder for VVC, and reports about experiments of dual-layer coding various pairs in the quality ladder. Using one example sequence, an example combination of single-layer and dual-layer coding for the full ladder is said to save about 24% storage space, at the expense of 15 to 22% objective losses for the end user for some rate points (measured as PSNR-Y BD-rate increase).

# Introduction

Visual tests of VVC multi-layer coding are planned, as described in JVET-AB2021. The first two test categories involve visual assessment of dual-layer coding performance compared to single-layer coding. More specifically, the second category tests the potential benefits of dual-layer coding in terms of edge storage savings for streaming applications. The discussion and experiments reported here are intended to help refine the test conditions for this category.

# Experiments on dual-layer coding for quality ladder

### Use case description

In streaming applications, the quality of the video distributed to the end user is typically adapted to network conditions and device capabilities. The same video content is then encoded at several resolutions, bitrates (and potentially different codecs), which is called here a quality ladder. The quality ladder is typically stored in what is called “edge storage” here, used as a cache to adaptively distribute content to several local users: the relevant item is selected and distributed to each end user.

Since the same video content is encoded several times, at different qualities (rate and resolution), multi-layer coding can potentially help reduce storage requirements by combining several quality points, while coding performance of these arrangements would impact the end user.

It is suggested to focus on dual-layer coding, so that the end user device has to decode at most one base layer and one dependent layer. Note that several independent pairs can reuse the same base layer.

### Quality ladder for VVC

HLS authoring specification [1] from Apple contains the following recommended quality ladder for AVC, HEVC SDR and HEVC HDR:

Table 1 – Example quality ladder for AVC and HEVC (bitrates in kbit/s)

| Resolution | AVC | HEVC (SDR) | HEVC (HDR) | Frame rate |
| --- | --- | --- | --- | --- |
| 416 x 234 | 145 | 145 | 160 | ≤ 30 fps |
| 480 x 270 | 365 | 300 | 360 | ≤ 30 fps |
| 640 x 360 | 730 | 600 | 730 | ≤ 30 fps |
| 768 x 432 | 1100 | 900 | 1090 | ≤ 30 fps |
| 960 x 540 | 2000 | 1600 | 1930 | Same as source |
| 1280 x 720 | 3000 | 2400 | 2900 | Same as source |
| 1280 x 720 | 4500 | 3400 | 4080 | Same as source |
| 1920 x 1080 | 6000 | 4500 | 5400 | Same as source |
| 1920 x 1080 | 7800 | 5800 | 7000 | Same as source |
| 2560 x 1440 | - | 8100 | 9700 | Same as source |
| 3840 x 2160 | - | 11600 | 13900 | Same as source |
| 3840 x 2160 | - | 16800 | 20000 | Same as source |

Those bitrates are said to be initial encoding targets for typical streaming content, and it is recommended that they are evaluated against actual encoded content and encoding workflow, then adjusted accordingly.

It is noted that HEVC SDR rates are about 80% to 75% of AVC rates (higher difference for higher rates), and that HEVC HDR rates are 120% of HEVC SDR rates.

Table 1 could be extended to VVC with the following proposed bitrates:

Table 2 – Proposed quality ladder for VVC (bitrates in kbit/s)

| Resolution | HEVC (HDR) | VVC | % of HEVC HDR | % of previous step | % of total |
| --- | --- | --- | --- | --- | --- |
| 416 x 234 | 160 | 130 | 81 | - | 0.4 |
| 480 x 270 | 360 | 260 | 72 | 200 | 0.7 |
| 640 x 360 | 730 | 530 | 73 | 204 | 1.5 |
| 768 x 432 | 1090 | 790 | 72 | 149 | 2.2 |
| 960 x 540 | 1930 | 1300 | 67 | 165 | 3.6 |
| 1280 x 720 | 2900 | 1800 | 62 | 138 | 4.9 |
| 1280 x 720 | 4080 | 2400 | 59 | 133 | 6.6 |
| 1920 x 1080 | 5400 | 3100 | 57 | 129 | 8.5 |
| 1920 x 1080 | 7000 | 4000 | 57 | 129 | 11 |
| 2560 x 1440 | 9700 | 5200 | 54 | 130 | 14.2 |
| 3840 x 2160 | 13900 | 7000 | 50 | 135 | 19.2 |
| 3840 x 2160 | 20000 | 10000 | 50 | 143 | 27.4 |

Figure 1 – Bitrate distribution within proposed quality ladder for VVC

### Sequence used for experiments

One sequence, called “Procession” here, has been used for experiments; it is extracted from the NetFlix “ElFuente” test sequence, from picture 12000 to 12599. It is the same sequence that was used for SHVC verification tests in 8-bit, and re-extracted from the source material to get a (cropped) 10-bit version, as explained in JVET-Y0047, using the following command line:

ffmpeg -y -ss 00:03:20.2 -i NETFLIX_ElFuente_4096x2160_28592frames_5994fps.mov -f rawvideo -pix_fmt yuv420p10le -sws_flags lanczos+accurate_rnd+full_chroma_inp -vsync 0 -vf crop=3840:2160:128:0,scale=out_v_chr_pos=128:out_h_chr_pos=0:3840:2160,setsar=1/1 -frames 600 ElFuente-12000.yuv

This sequence contains 3 scene cuts, but is still nice to assess visually. It has average coding bitrate, with a mix of characteristics: fixed camera with decent detail and moderate motion, face close-up with subtle details (man’s white hat), crowd with high motion with faces and details and floor/background with more subtle details.

The sequence was downscaled to 2560x1440 and 1920x1080 using the TAppDownConvertStatic tool from the SHM repository, then downscaled again from 1920x1080 to 1280x720 and 960x540 resolutions using the same tool. The following example command line has been used:

TAppDownConvertStatic 3840 2160 <input_UHD.yuv> 1920 1080 <output_HD.yuv> 10 10

Table 3 – Test sequence used for experiments

| ElFuente-12000 (Procession) 3840×2160, 60fps, 600 frames |  |
| --- | --- |

### Experiments at 1x scaling ratio (SNR scalability)

Several options are possible to pair ladder items in a dual-layer bitstream. Pairing neighboring items looks more appropriate, so that inter-layer prediction can bring significant benefit. Experiments with 1x scaling ratio evaluate the space savings and coding performance impact of pairing same-resolution streams together, with the lower quality as a base layer and higher quality as enhancement layer (SNR scalability).

#### At 1280x720 resolution

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 1280 x 720 | 1800 | 1 |
| 1280 x 720 | 2400 | 1.33 |

Single-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 30 | 1909.0296 | 1 | 38.6854 | 42.3863 | 43.5439 |
| 29 | 2161.7960 | 1.13 | 39.2251 | 42.7419 | 43.9420 |
| 28 | 2471.8400 | 1.29 | 39.8021 | 43.1575 | 44.4079 |
| 27 | 2852.7360 | 1.49 | 40.4150 | 43.5423 | 44.8678 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 (BL) | 1911.6392 | 1.00 | 38.6830 | 42.3843 | 43.5369 | 1.002 |
| 30+28 | 2303.5208 | 1.21 | 39.1031 | 42.5158 | 43.6639 | 1.096 |
| 30+27 | 2908.5424 | 1.52 | 39.8994 | 42.7693 | 43.9342 | 1.150 |

Figure 2 – Rate/PSNR plot for 1280x720 SNR scalability (text labels = QPs)

QP-3 gives 15% loss (luma, much more for chroma) with a rate ratio 1.5x, meaning a quality comparable to a 1.3x rate ratio using single-layer coding (which best matches the proposed ladder). QP-2 would not give a sufficient quality improvement, though the loss is lower.

Comparing dual-layer with to single-layer, this means a storage cost of 1.15*1.33/(1+1.33) = 0.66 (34% savings), for 15% user-end rate increase for the higher quality stream.

#### At 1920x1080 resolution

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 1920 x 1080 | 3100 | 1 |
| 1920 x 1080 | 4000 | 1.29 |

Single-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 30 | 3169.9112 | 1 | 39.7645 | 43.1579 | 44.6078 |
| 29 | 3582.5840 | 1.13 | 40.2443 | 43.4901 | 44.9855 |
| 28 | 4080.5336 | 1.29 | 40.7519 | 43.8585 | 45.3997 |
| 27 | 4691.3960 | 1.48 | 41.2786 | 44.2318 | 45.8220 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 (BL) | 3172.4144 | 1 | 39.7630 | 43.1556 | 44.6007 | 1.001 |
| 30+28 | 3792.2296 | 1.20 | 40.1480 | 43.2761 | 44.7258 | 1.085 |
| 30+27 | 4723.2728 | 1.49 | 40.8177 | 43.5074 | 44.9660 | 1.138 |

Figure 3 – Rate/PSNR plot for 1920x1080 SNR scalability (text labels = QPs)

Conclusions are very similar to the 1280x720 case, with slightly lower losses.

### Experiments at 1.5x scaling ratio (spatial scalability)

These experiments evaluate the space savings and coding performance impact of pairing streams that have a 1.5x resolution ratio (1280x720 and 1920x1080, 2560x1440 and 3840x2160), with the lower resolution as a base layer and higher resolution as enhancement layer (spatial scalability). Since the rate allocated to enhancement layer is expected to be higher than the optimal coding efficiency case, some coding loss compared to single layer is expected. The result is expected to be a mix of spatial and SNR scalability.

#### Resolutions 1280x720 and 1920x1080

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 1280 x 720 | 2400 | 1 |
| 1920 x 1080 | 3100 | 1.29 |

Single-layer 1280x720 coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 29 | 2161.7960 | 0.87 | 39.2251 | 42.7419 | 43.9420 |
| 28 | 2471.8400 | 1 | 39.8021 | 43.1575 | 44.4079 |
| 27 | 2852.7360 | 1.15 | 40.4150 | 43.5423 | 44.8678 |

Single-layer HD coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 31 | 2693.5480 | 1.09 | 39.1377 | 42.7623 | 44.1272 |
| 30 | 3169.9112 | 1.28 | 39.7645 | 43.1579 | 44.6078 |
| 29 | 3582.5840 | 1.45 | 40.2443 | 43.4901 | 44.9855 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 28 (BL) | 2474.1008 | 1 | 39.8019 | 43.1502 | 44.4102 | 1.001 |
| 28 (upscaled BL) | 2474.1008 | 1 | 37.8785 | 42.2624 | 43.5650 | 1.250 |
| 28+36 | 2556.5056 | 1.03 | 38.2508 | 42.3916 | 43.6572 | 1.179 |
| 28+35 | 2584.4984 | 1.04 | 38.3437 | 42.4161 | 43.6831 | 1.166 |
| 28+34 | 2620.8056 | 1.06 | 38.4390 | 42.4347 | 43.7086 | 1.155 |
| 28+33 | 2670.3336 | 1.08 | 38.5532 | 42.4516 | 43.7417 | 1.145 |
| 28+32 | 2748.7552 | 1.11 | 38.6889 | 42.4706 | 43.7691 | 1.140 |
| 28+31 | 2876.0984 | 1.16 | 38.8543 | 42.5002 | 43.8099 | 1.145 |
| 28+30 | 3252.7216 | 1.32 | 39.2320 | 42.6016 | 43.9163 | 1.178 |
| 28+29 | 3827.7208 | 1.55 | 39.7751 | 42.7888 | 44.1192 | 1.204 |
| 28+28 | 4535.6144 | 1.83 | 40.4084 | 43.0932 | 44.4552 | 1.214 |

Here, space savings are around 1.20*1.29 / (1 + 1.29) = 0.68 = -32%, and user-end rate increase around 20% for the higher quality stream.

Figure 4 – Rate/PSNR plot for 720+HD scalability (text labels = QPs, star = upscaled base layer)

#### Resolutions 2560x1440 and 3840x2160

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 2560 x 1440 | 5200 | 1 |
| 3840 x 2160 | 7000 | 1.35 |

Single-layer 2560x1440 coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 30 | 4316.3936 | 0.89 | 40.4761 | 43.8279 | 45.4388 |
| 29 | 4873.3856 | 1 | 40.9009 | 44.1361 | 45.8033 |
| 28 | 5548.8088 | 1.14 | 41.3488 | 44.4629 | 46.1939 |

Single-layer UHD coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 31 | 5944.3688 | 1.22 | 40.1918 | 44.0460 | 45.8498 |
| 30 | 7002.4224 | 1.44 | 40.6251 | 44.3955 | 46.2763 |
| 29 | 7946.6000 | 1.63 | 40.9396 | 44.6844 | 46.6149 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 29 (BL) | 4863.5440 | 1 | 40.8888 | 44.1398 | 45.8020 | 1.001 |
| 29 (upscaled BL) | 4863.5440 | 1 | 39.5585 | 43.6061 | 45.3673 | 1.016 |
| 29+36 | 4917.3128 | 1.01 | 39.5844 | 43.6529 | 45.4108 | 1.018 |
| 29+35 | 4933.4448 | 1.01 | 39.6039 | 43.6619 | 45.4245 | 1.015 |
| 29+34 | 4959.0320 | 1.02 | 39.6236 | 43.6729 | 45.4350 | 1.014 |
| 29+33 | 4997.0912 | 1.03 | 39.6486 | 43.6865 | 45.4482 | 1.013 |
| 29+32 | 5132.6184 | 1.06 | 39.7036 | 43.7161 | 45.4753 | 1.022 |
| 29+31 | 5606.1096 | 1.15 | 39.8588 | 43.7759 | 45.5292 | 1.059 |
| 29+30 | 7034.9248 | 1.45 | 40.3099 | 43.9659 | 45.7206 | 1.132 |
| 29+29 | 8348.6072 | 1.72 | 40.7067 | 44.2162 | 45.9885 | 1.154 |
| 29+28 | 9868.4360 | 2.03 | 41.1066 | 44.5219 | 46.3359 | 1.157 |

Here, space savings are around 1.13*1.35 / (1 + 1.35) = 0.65 = -35%, and user-end rate increase around 13% for the higher quality stream.

Figure 5 – Rate/PSNR plot for 1440+UHD scalability (text labels = QPs, star = upscaled base layer)

### Experiments at 1.33x scaling ratio (spatial scalability)

These experiments evaluate the space savings and coding performance impact of pairing streams that have a 1.33x resolution ratio (960x540 and 1280x720, 1920x1080 and 2560x1440), with the lower resolution as a base layer and higher resolution as enhancement layer (spatial scalability).

#### Resolutions 960x540 and 1280x720

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 960 x 540 | 1300 | 1 |
| 1280 x 720 | 1800 | 1.38 |

Single-layer 960x540 coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 31 | 1119,5728 | 0,84 | 37,0403 | 41,2630 | 42,1815 |
| 30 | 1328,8664 | 1 | 37,7922 | 41,7346 | 42,7062 |
| 29 | 1507,1800 | 1,13 | 38,3662 | 42,1345 | 43,1336 |

Single-layer 1280x720 coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 31 | 1612,3936 | 1,21 | 37,9862 | 41,9445 | 43,0432 |
| 30 | 1909,0296 | 1,44 | 38,6854 | 42,3863 | 43,5437 |
| 29 | 2161,7960 | 1,63 | 39,2251 | 42,7419 | 43,9420 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 (BL) | 1330,7176 | 1 | 37,7894 | 41,7302 | 42,7080 | 1,002 |
| 30 (upscaled BL) | 1330,7176 | 1 | 36,0687 | 41,1157 | 42,1499 | 1,288 |
| 30+38 | 1363,6408 | 1,03 | 36,4570 | 41,1319 | 42,1692 | 1,205 |
| 30+37 | 1372,7432 | 1,03 | 36,5179 | 41,1357 | 42,1737 | 1,196 |
| 30+36 | 1387,2616 | 1,04 | 36,6077 | 41,2458 | 42,2211 | 1,184 |
| 30+35 | 1407,1232 | 1,06 | 36,7105 | 41,2613 | 42,2506 | 1,172 |
| 30+34 | 1435,8984 | 1,08 | 36,8192 | 41,2851 | 42,2754 | 1,166 |
| 30+33 | 1475,9752 | 1,11 | 36,9586 | 41,3069 | 42,3024 | 1,161 |
| 30+32 | 1551,3576 | 1,17 | 37,1596 | 41,3410 | 42,3415 | 1,164 |
| 30+31 | 1688,7808 | 1,27 | 37,4660 | 41,4082 | 42,4191 | 1,181 |
| 30+30 | 2060,8072 | 1,55 | 38,1626 | 41,6269 | 42,6604 | 1,225 |

Here, space savings are around 1.22*1.38 / (1 + 1.38) = 0.71 = -29%, and user-end rate increase around 22% for the higher quality stream.

Figure 6 – Rate/PSNR plot for 540+720 scalability (text labels = QPs, star = upscaled base layer)

#### Resolutions 1920x1080 and 2560x1440

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 1920 x 1080 | 4000 | 1 |
| 2560 x 1440 | 5200 | 1.30 |

Single-layer HD coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 29 | 3582,5840 | 0,88 | 40,2443 | 43,4901 | 44,9855 |
| 28 | 4080,5336 | 1 | 40,7519 | 43,8585 | 45,3997 |
| 27 | 4691,3960 | 1,15 | 41,2786 | 44,2318 | 45,8220 |

Single-layer 2560x1440 coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 29 | 4873,3856 | 1,19 | 40,9009 | 44,1361 | 45,8033 |
| 28 | 5548,8088 | 1,36 | 41,3488 | 44,4629 | 46,1939 |
| 27 | 6374,0416 | 1,56 | 41,8098 | 44,8069 | 46,5925 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 28 (BL) | 4083,8536 | 1,00 | 40,7503 | 43,8517 | 45,3942 | 1,001 |
| 28 (upscaled BL) | 4083,8536 | 1,00 | 39,9355 | 43,5745 | 45,1462 | 1,103 |
| 28+36 | 4119,6432 | 1,01 | 40,0584 | 43,6183 | 45,1828 | 1,074 |
| 28+35 | 4128,3544 | 1,01 | 40,0758 | 43,6307 | 45,1930 | 1,071 |
| 28+34 | 4141,4952 | 1,01 | 40,0972 | 43,6439 | 45,2036 | 1,068 |
| 28+33 | 4167,2056 | 1,02 | 40,1271 | 43,6545 | 45,2143 | 1,066 |
| 28+32 | 4204,5368 | 1,03 | 40,1673 | 43,6651 | 45,2316 | 1,063 |
| 28+31 | 4272,2360 | 1,05 | 40,2201 | 43,6780 | 45,2494 | 1,064 |
| 28+30 | 4408,6624 | 1,08 | 40,3129 | 43,7022 | 45,2821 | 1,070 |
| 28+29 | 4746,7128 | 1,16 | 40,4865 | 43,7529 | 45,3414 | 1,096 |
| 28+28 | 5616,3384 | 1,38 | 40,9192 | 43,8939 | 45,4844 | 1,146 |

Here, space savings are around 1.15*1.30 / (1 + 1.30) = 0.65 = -35%, and user-end rate increase around 15% for the higher quality stream.

Figure 7 – Rate/PSNR plot for 1080+1440 scalability (text labels = QPs, star = upscaled base layer)

### Experiments at 2x scaling ratio (spatial scalability)

For these experiments, the higher quality HD (1920x1080) is paired with lower quality UHD (3840x2160). Those are not neighbours in the ladder (there is 2560x1440 in between), thus the bitrate gap is higher, so the impact of SNR vs spatial scalability is expected to be higher.

Bitrate target (from proposed VVC ladder):

| Resolution | Bitrate (kbps) | Rate ratio |
| --- | --- | --- |
| 1920 x 1080 | 4000 | 1 |
| 3840 x 2160 | 7000 | 1.75 |

Single-layer HD coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 29 | 3582.5840 | 0.88 | 40.2443 | 43.4901 | 44.9855 |
| 28 | 4080.5336 | 1 | 40.7519 | 43.8585 | 45.3997 |
| 27 | 4691.3960 | 1.15 | 41.2786 | 44.2318 | 45.8220 |

Single-layer UHD coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV |
| --- | --- | --- | --- | --- | --- |
| 31 | 5944.3688 | 1.46 | 40.1918 | 44.0460 | 45.8498 |
| 30 | 7002.4224 | 1.72 | 40.6251 | 44.3955 | 46.2763 |
| 29 | 7946.6000 | 1.95 | 40.9396 | 44.6844 | 46.6149 |

Dual-layer coding results:

| QP | Bitrate | Rate ratio | PSNRY | PSNRU | PSNRV | BDrate(PSNRY) |
| --- | --- | --- | --- | --- | --- | --- |
| 28 (BL) | 4075.2824 | 1.00 | 40.7376 | 43.8530 | 45.3972 | 1.002 |
| 28 (upscaled BL) | 4075.2824 | 1.00 | 38.8764 | 43.0976 | 44.7750 | 1.060 |
| 28+36 | 4184.5656 | 1.03 | 39.0384 | 43.2288 | 44.9016 | 1.034 |
| 28+35 | 4235.8240 | 1.04 | 39.0817 | 43.2481 | 44.9214 | 1.033 |
| 28+34 | 4326.6040 | 1.06 | 39.1397 | 43.2860 | 44.9527 | 1.035 |
| 28+33 | 4521.7808 | 1.11 | 39.2411 | 43.3258 | 44.9916 | 1.047 |
| 28+32 | 5092.2472 | 1.25 | 39.4710 | 43.4236 | 45.0864 | 1.095 |
| 28+31 | 6051.4904 | 1.48 | 39.8425 | 43.5767 | 45.2335 | 1.150 |
| 28+30 | 7571.9608 | 1.86 | 40.3996 | 43.9093 | 45.5969 | 1.178 |
| 28+29 | 8784.6104 | 2.16 | 40.7830 | 44.2324 | 45.9680 | 1.177 |
| 28+28 | 10212.0568 | 2.51 | 41.1623 | 44.5844 | 46.3804 | 1.169 |

Here, space savings are around 1.18*1.75 / (1 + 1.75) = 0.75 = -25%, and user-end rate increase around 18%.

Figure 8 – Rate/PSNR plot for HD+UHD scalability (text labels = QPs, star = upscaled base layer)

### Potential combinations

Some bitrate pairs can be combined, and bring significant additional space savings if reusing the same base layer.

Table 4 – Example quality ladder for multilayer VVC (bitrates in kbit/s)

| Resolution | VVC | VVC Multilayer | Single layer | Base layer | Enhancement layer | % of total | % user-end loss |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 416 x 234 | 130 | 130 | x |  |  | 0.5 |  |
| 480 x 270 | 260 | 260 | x |  |  | 0.9 |  |
| 640 x 360 | 530 | 530 | x |  |  | 1.9 |  |
| 768 x 432 | 790 | 790 | x |  |  | 2.8 |  |
| 960 x 540 | 1300 | 1300 |  | x |  | 4.7 |  |
| 1280 x 720 | 1800 | BL+900 |  |  | x | 3.2 | 22 |
| 1280 x 720 | 2400 | 2400 |  | x |  | 8.6 |  |
| 1920 x 1080 | 3100 | BL+1300 |  |  | x | 4.7 | 20 |
| 1920 x 1080 | 4000 | 4000 |  | x |  | 14.3 |  |
| 2560 x 1440 | 5200 | BL+2000 |  |  | x | 7.2 | 15 |
| 3840 x 2160 | 7000 | BL+4300 |  |  | x | 15.4 | 18 |
| 3840 x 2160 | 10000 | 10000 | x |  |  | 35.8 |  |
| Total | 36510 | 27910 |  |  |  |  |  |

Global space savings with this arrangement is around 24%.

# References

- https://developer.apple.com/documentation/http_live_streaming/http_live_streaming_hls_authoring_specification_for_apple_devices, HTTP Live Streaming (HLS) Authoring Specification for Apple Devices.