| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0149-v2 |
| --- | --- |

| Title: | AHG10/12: Reduced I-frame QP for RA |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Per Wennersten | Email: | {kenneth.r.andersson, per.wennersten} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal shows results for a config file change reducing the I-frame QP by one in the RA configuration. It is reported that BD-rate is improved by -0.28%/-2.62%/-2.76% for Y/Cb/Cr in VTM. Similar numbers are reported for ECM. Subjective benefits are reported for static content, no noticeable subjective problems are reported. It is proposed that the change be adopted into the CTC for both VVC and ECM.

v2: complete results for ECM and correction for class F and aligned class F results for VTM with excel and accurate timing for class B, C and D for VTM.

# Config change

This proposal investigates changing the “IntraQPOffset” config parameter from -3 to -4 in the random-access configurations of VTM and ECM.

# Results

Subjective viewings have not identified any problems with the change, but improvements were seen in some fairly static sequences. Only accurate timing for class B, C and D. Otherwise encoding and decoding times are not accurate.

Objective results for VTM 19.0 [1] are as follows:

|  | Random Access Main 10 Over VTM-19.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,14% | -0,63% | -1,69% | 98% | 97% |
| Class A2 | -0,20% | -2,19% | -1,60% | 98% | 98% |
| Class B | -0,44% | -3,42% | -3,86% | 100% | 101% |
| Class C | -0,47% | -3,43% | -3,05% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0,28% | -2,62% | -2,76% | 99% | 99% |
| Class D | -0,20% | -4,13% | -3,62% | 100% | 105% |
| Class F | -1,01% | -2,00% | -1,85% | 102% | 92% |  |

The results for ECM 7.0 [2] are as follows:

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,15% | -0,61% | -1,52% | 111% | 105% |
| Class A2 | -0,18% | -2,05% |  | -1,91% |  | 109% |  | 103% |  |
| Class B | -0,47% | -3,51% | -3,95% | 111% | 107% |
| Class C | -0,38% | -3,30% | -2,82% | 109% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0,26% |  | -2,58% |  | -2,75% |  | 110% |  | 106% |  |
| Class D | -0,16% | -3,81% | -3,62% | 109% | 105% |
| Class F | -1,32% | -2,08% | -1,90% | 99% | 98% |

# Conclusions

This contribution tests the effects of reducing intra frame QP by one for the VTM and ECM common test conditions RA configuration. It is reported that this leads to BD-rate gains of -0.3%/-2.6%/-2.8% for VTM Y/Cb/Cr, and -0.3/-2.6/-2.8 for ECM. Small subjective benefits are reported, and no subjective issues.

It is proposed that the value of “IntraQPOffset” be changed from -3 to -4 for the RA configuration for both VTM and ECM.

# References

[1] VTM-19.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-19.0

[2] ECM-7.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[3] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).