| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0152-v1 |
| --- | --- |

| Title: | AHG9: Bit-masking based representation of nnpfc_purpose |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang and Yue Li 8910 University Center Lane, San Diego, CA 92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com yue.li@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes bit-masking based representation of nnpfc_purpose. Specifically, each bit or effective bit of nnpfc_purpose denotes a primitive format change, which can be chroma format change, resolution change, frame rate change and other primitive format changes.

# Introduction

In the current semantics of nnpfc_purpose, different purposes are enumerated in a table as shown below:

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5, inclusive, in bitstreams conforming to this edition of this document. Values of 6 to 1023, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 to 1203, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |

Based on the value of nnpfc_purpose, different parameters are signalled to indicate format change, including chroma format, resolution and frame rate. We call each of chroma format change, resolution change and frame rate change a primitive format change. In actual applications, those primitive format changes can be used independently or jointly. It does make sense to represent nnpfc_purpose according to those primitive format changes. It also makes further extension of nnpfc_purpose easier.

# Proposed methods

To represent nnpfc_purpose according to primitive format changes, bit-masking representation methods are used to define each value of nnpfc_purpose. Specifically, two methods are considered:

Method 1: nnpfc_purpose is signalled as u(N) and each bit indicates a primitive format change;

Method 2: nnpfc_purpose is still signalled as ue(v) and each effective bit (when the value is represented as a binary number) indicates a primitive format change.

By using bit-masking based representation, independent or jointly usage of primitive format changes is enabled naturally and further extensions to other primitive format changes, e.g. bitdepth change, become straightforward.

# Proposed text changes

The following proposed text changes are based on the MPEG output document ISO/IEC JTC 1/SC 29 /WG 5 N 158, with additions highlighted in turquoise, and removals highlighted in yellow strikethrough in red fonts.

Method 1:

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) u(10) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) if( nnpfc_purpose & 0x01 != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { if( nnpfc_purpose & 0x02 != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ) { if( nnpfc_purpose & 0x04 != 0 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| … |  |

…

8.28.2 Neural-network post-filter characteristics SEI message semantics

…

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5 7, inclusive, or equal to 1023 in bitstreams conforming to this edition of this document. Values of 6 8 to 1023 1022, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 8 to 1203 1022, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When no format change exists, the post-processing filter may be for the purpose of visual quality improvement.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose & 0x01 = = 0 | No chroma upsampling |
| nnpfc_purpose & 0x01 != 0 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| nnpfc_purpose & 0x02 = = 0 | No resolution upsampling |
| nnpfc_purpose & 0x02 != 0 | Resolution upsampling (increasing the width or height of the cropped decoded output picture) |
| nnpfc_purpose & 0x04 = = 0 | No picture rate upsampling |
| nnpfc_purpose & 0x04 != 0 | Picture rate upsampling |
| nnpfc_purpose = = 1023 | May be used as determined by the application |

...

Method 2:

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( nnpfc_formatting_and_purpose_flag ) { |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) if( nnpfc_purpose & 0x01 != 0 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { if( nnpfc_purpose & 0x02 != 0 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ) { if( nnpfc_purpose & 0x04 != 0 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| … |  |

…

8.28.2 Neural-network post-filter characteristics SEI message semantics

…

nnpfc_purpose indicates the purpose of the post-processing filter as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 5 7, inclusive, or euqal to 1023 in bitstreams conforming to this edition of this document. Values of 6 8 to 1023 1022, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 6 8 to 1203 1022, inclusive. Values of nnpfc_purpose greater than 1023 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

When no format change exists, the post-processing filter may be for the purpose of visual quality improvement.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose & 0x01 = = 0 | No chroma upsampling |
| nnpfc_purpose & 0x01 != 0 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| nnpfc_purpose & 0x02 = = 0 | No resolution upsampling |
| nnpfc_purpose & 0x02 != 0 | Resolution upsampling (increasing the width or height of the cropped decoded output picture) |
| nnpfc_purpose & 0x04 = = 0 | No picture rate upsampling |
| nnpfc_purpose & 0x04 != 0 | Picture rate upsampling |
| nnpfc_purpose = = 1023 | May be used as determined by the application |

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).