| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0153-v1 |
| --- | --- |

| Title: | AHG9: Bitdepth increase indication in the NNPFC SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang 8910 University Center Lane, San Diego, CA 92122, USA | Email: | xujizheng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to add a purpose of bitdepth increase to the neural-network post-filter characteristics (NNPFC) SEI message, to indicate the use of bitdepth increase that can be used for support related applications, e.g. SDR to HDR.

# Introduction

In the current text of NNPFC SEI message, nnpfc_purpose is signalled to indicate the purpose of the filter.

We notice that there are bunch of neural networks designed for SDR to HDR conversion and show superior performance, for example in [1-17]. For such kind of applications, bitdepth increase is usually a must from signal process point of view.

# Proposed method

To support applications that need bitdepth change, we propose to add a purpose of bitdepth increase and potentially its combination with other purposes to the NNPFC SEI message. Moreover, when the purpose indicates a bitdepth change, it is required that the value of nnpfc_out_tensor_bitdepth_minus8 shall be greater than nnpfc_inp_tensor_bitdepth_minus8.

# Text changes

The following text changes are based on the MPEG output document ISO/IEC JTC 1/SC 29 /WG 5 N 158, with additions highlighted in turquoise, and removals highlighted in yellow strikethrough in red fonts.

8.28.2 Neural-network post-filter characteristics SEI message semantics

…

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| 0 | May be used as determined by the application |
| 1 | Visual quality improvement |
| 2 | Chroma upsampling from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format |
| 3 | Increasing the width or height of the cropped decoded output picture without changing the chroma format |
| 4 | Increasing the width or height of the cropped decoded output picture and upsampling the chroma format |
| 5 | Picture rate upsampling |
| 6 | Bitdepth increase |

When nnpf_purpose is equal to 6, the value of  and both nnpfc_inp_tensor_bitdepth_minus8 and nnpfc_out_tensor_bitdepth_minus8 are presented, nnpfc_out_tensor_bitdepth_minus8 shall be greater than nnpfc_inp_tensor_bitdepth_minus8.

...

# Reference

[1] Eilertsen, Gabriel, Joel Kronander, Gyorgy Denes, Rafał K. Mantiuk, and Jonas Unger. "HDR image reconstruction from a single exposure using deep CNNs." ACM transactions on graphics (TOG) 36, no. 6 (2017): 1-15.

[2] Marnerides, Demetris, Thomas Bashford‐Rogers, Jonathan Hatchett, and Kurt Debattista. "Expandnet: A deep convolutional neural network for high dynamic range expansion from low dynamic range content." In Computer Graphics Forum, vol. 37, no. 2, pp. 37-49. 2018.

[3] Lee, Siyeong, Gwon Hwan An, and Suk-Ju Kang. "Deep chain hdri: Reconstructing a high dynamic range image from a single low dynamic range image." IEEE Access 6 (2018): 49913-49924.

[4] Lee, Siyeong, Gwon Hwan An, and Suk-Ju Kang. "Deep recursive hdri: Inverse tone mapping using generative adversarial networks." In proceedings of the European Conference on Computer Vision (ECCV), pp. 596-611. 2018.

[5] Endo, Yuki, Yoshihiro Kanamori, and Jun Mitani. "Deep reverse tone mapping." ACM Trans. Graph. 36, no. 6 (2017): 177-1.

[6] Moriwaki, Kenta, Ryota Yoshihashi, Rei Kawakami, Shaodi You, and Takeshi Naemura. "Hybrid loss for learning single-image-based HDR reconstruction." arXiv preprint arXiv:1812.07134 (2018).

[7] Khan, Zeeshan, Mukul Khanna, and Shanmuganathan Raman. "Fhdr: Hdr image reconstruction from a single ldr image using feedback network." In 2019 IEEE Global Conference on Signal and Information Processing (GlobalSIP), pp. 1-5. IEEE, 2019.

[8] Santos, Marcel Santana, Tsang Ing Ren, and Nima Khademi Kalantari. "Single image HDR reconstruction using a CNN with masked features and perceptual loss." ACM Transactions on GraphicsVolume 39, Issue 4, August 2020.

[9] Liu, Yu-Lun, Wei-Sheng Lai, Yu-Sheng Chen, Yi-Lung Kao, Ming-Hsuan Yang, Yung-Yu Chuang, and Jia-Bin Huang. "Single-image HDR reconstruction by learning to reverse the camera pipeline." In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pp. 1651-1660. 2020.

[10] Kim, Junghee, Siyeong Lee, and Suk-Ju Kang. "End-to-end differentiable learning to hdr image synthesis for multi-exposure images." In Proceedings of the AAAI Conference on Artificial Intelligence, vol. 35, no. 2, pp. 1780-1788. 2021.

[11] Wei, Wei, Li Guan, Yue Liu, Hao Kang, Haoxiang Li, Ying Wu, and Gang Hua. "Beyond Visual Attractiveness: Physically Plausible Single Image HDR Reconstruction for Spherical Panoramas." arXiv preprint arXiv:2103.12926 (2021).

[12] Kalantari, Nima Khademi, and Ravi Ramamoorthi. "Deep hdr video from sequences with alternating exposures." In Computer graphics forum, vol. 38, no. 2, pp. 193-205. 2019.

[13] Eilertsen, Gabriel, Rafal K. Mantiuk, and Jonas Unger. "Single-frame regularization for temporally stable cnns." In Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition, pp. 11176-11185. 2019.

[14] Kim, Soo Ye, Jihyong Oh, and Munchurl Kim. "Deep sr-itm: Joint learning of super-resolution and inverse tone-mapping for 4k uhd hdr applications." In Proceedings of the IEEE/CVF International Conference on Computer Vision, pp. 3116-3125. 2019.

[15] Kim, Soo Ye, Jihyong Oh, and Munchurl Kim. "Jsi-gan: Gan-based joint super-resolution and inverse tone-mapping with pixel-wise task-specific filters for uhd hdr video." In Proceedings of the AAAI Conference on Artificial Intelligence, vol. 34, no. 07, pp. 11287-11295. 2020.

[16] Chen, Guanying, Chaofeng Chen, Shi Guo, Zhetong Liang, Kwan-Yee K. Wong, and Lei Zhang. "HDR video reconstruction: A coarse-to-fine network and a real-world benchmark dataset." In Proceedings of the IEEE/CVF International Conference on Computer Vision, pp. 2502-2511. 2021.

[17] Chen, Xiangyu, Zhengwen Zhang, Jimmy S. Ren, Lynhoo Tian, Yu Qiao, and Chao Dong. "A New Journey From SDRTV to HDRTV." In Proceedings of the IEEE/CVF International Conference on Computer Vision, pp. 4500-4509. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).