| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0156-v3 |
| --- | --- |

| Title: | [EE1-related] RTNN: An In-loop Filter Based on Resblock and Transformer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hao Zhang Cheolkon Jung Yang Liu Ming Li | Email: | 13227706628@163.com zhengzk@xidian.edu.cn serena@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network (CNN)-based in-loop filter. The proposed CNN filter explores the combination of Resblock and Transformer block (TB) to build a new backbone network with efficiency. Moreover, a new attention module is proposed to better refine features by introducing auxiliary information. In addition, the training strategy in JVET-AB0090 [1] is used to train the proposed CNN filter. Compared with EE1-anchor-2.0 (VTM-11.0_NNVC-2.0), the proposed CNN filter achieves average {8.49%, 22.98%, 24.07%} BD-rate reductions for {Y, Cb, Cr} under AI configuration and average {9.23%, 22.47%, 22.31%} BD-rate reductions for {Y, Cb, Cr} under RA configuration.

Introduction

Most existing CNN filters use convolution as the basic component of the network, but when the network reaches a certain depth, its performance improvement becomes very limited. Compared with CNN, Transformer shows better global feature processing capability and has proved its effectiveness in some computer vision tasks. Its biggest disadvantage is that the computational complexity of self-attention is quadratic to an input resolution. However, because the loop filter works in the unit of CTU, and its size is fixed and relatively small, which is very suitable for introducing Transformer into the loop filter. To sum up, this contribution explores the combination of traditional CNN and Transformer to achieve better performance in video coding. Specifically, the proposed network uses residual blocks and attention modules to extract shallow features and then utilizes Transformer blocks to extract and process deep features. Restormer [2] has been selected as our Transformer block. In addition, the more auxiliary information is introduced into the attention module to adaptively select and refine features. Finally, the progressive training strategy proposed in JVET-AB0090 is used to train the proposed model, and the iterative training strategy is also used to train the B-Slice model.

Proposed method

Network architecture

For the feature extraction part and reconstruction part, the general architecture is consistent with JVET-AB0090. On this basis, some changes have been made for different components. Fig. 1 shows the feature extraction and reconstruction of the luminance component, while Fig. 2 shows the feature extraction and reconstruction of the chrominance component. First, the number of feature maps is allocated according to the richness of the information contained in the input to reduce the complexity while ensuring performance. Secondly, for the chroma component, according to different categories of input information, a gradual fusion architecture is used to obtain more accurate fusion features. Note that the luma model does not use partition information for B Slice.

Fig. 1 Network architecture of feature extraction and reconstruction of luma component. (a) Feature extraction part. (b) Reconstruction part.

Fig. 2 Network architecture of feature extraction and reconstruction of chroma component. (a) Feature extraction part. (b) Reconstruction part.

In the backbone, the residual attention network [6] and transformer block are combined to extract and process intermediate features. First, residual attention blocks are used to extract shallow features of input information and capture the correlation between local features. However, it is not enough to only consider the local correlation between features. In a video frame, similar patterns often appear repeatedly. It is very important to calculate the response of a single position by weighting all nonlocal related positions. Hence, the transformer block is employed to capture the long-range correlation between features, thereby aiding the network to acquire more effective residual features. Network architectures of the backbone are shown in Fig. 3. Note that the Transformer block used is from [2]. In addition, for the luma model, the backbone network includes 3 RABs and 6 TBs, while for the chroma model, the backbone network includes 1 RAB and 3 TBs.

Fig. 3 Network architectures of the proposed backbone. (a) Residual block. (b) Residual attention block (RAB). (c) Backbone. TB: Transformer block.

In the attention block, the more auxiliary information is introduced to guide the network to adaptively select and refine important features. Specifically, for the spatial attention block, partition map and qp map are introduced to better locate the region where the blocking effect and distortion are located spatially. For the channel attention block, qp is introduced to fuse the results of intensity attention and contrast attention. This is mainly due to an observation that the network tends to focus on structural features for larger qp inputs, while the network tends to focus on texture features for smaller qp inputs.

Fig. 4 Network architectures of the proposed attention block. (a) Spatial attention block. (b) Channel attention block. (c) Channel-spatial joint attention block.

Training

Table 1. Network information of the proposed CNN filter in the training stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 800 |
|  | Batch size: | 32 |
|  | Training time: | ~70h/luma model, ~30h/chroma model |
|  | Training data information: | BVI-DVC, DIV2K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0, QP {12,17,22, 27, 32, 37, 42} |
|  | Loss function: | L1 and L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

Table 2: Network information for the NN-based video coding tool in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.4M for luma, 0.47M for chroma |
|  | Total Number of Parameters (All Models) | 3.74M (4 models) |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 14.96M |
|  | Multiply Accumulate (MAC)/pixel | 353K for luma, 116K for chroma |
| Optional |  |  |
|  | Total Conv. Layers | 54 Depthwise convolution layers 174 Convolution layers (4 models total) |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144x144 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experimental results

Objective comparison

The proposed CNN in-loop filter is evaluated on the top of VTM-11.0_NNVC-2.0 [4] according to the common test conditions (CTCs) defined in [5]. The evaluation is performed in the AI and RA configurations. Experimental results of the proposed CNN filter are shown in Tables 3 and 4. In the AI configuration, the proposed CNN filter achieves {8.49%, 22.98%, 24.07%} BD-rate reduction on average for {Y, Cb, Cr} channel. In the RA configuration, the proposed filter achieves {9.23%, 22.47%, 22.31%} BD-rate reduction on average for {Y, Cb, Cr} channel.

Table 3 Performance of the proposed method in AI configuration

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -5.65% | -20.20% | -22.99% | -5.40% | -23.53% | -24.66% |
| Class A2 | -8.02% | -25.34% | -28.49% | -8.26% | -27.36% | -27.82% |
| Class B | -8.28% | -24.77% | -22.54% | -8.09% | -26.90% | -24.94% |
| Class C | -8.79% | -19.65% | -22.70% | -8.52% | -22.73% | -24.87% |
| Class E | -11.78% | -24.88% | -25.11% | -11.58% | -26.50% | -28.41% |
| Overall | -8.49% | -22.98% | -24.07% | -8.35% | -25.42% | -25.94% |
| Class D | -8.23% | -20.61% | -22.94% | -7.31% | -23.60% | -25.36% |
| Class F | -5.10% | -15.87% | -14.80% | -5.48% | -18.13% | -18.24% |

Table 4 Performance of the proposed method in RA configuration

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-2.0 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM |
| Class A1 | -8.53% | -14.93% | -16.73% | -8.28% | -20.19% | -18.96% |
| Class A2 | -10.02% | -23.30% | -25.74% | -8.95% | -23.84% | -25.25% |
| Class B | -9.17% | -27.88% | -24.12% | -8.18% | -27.67% | -24.45% |
| Class C | -9.23% | -20.74% | -21.66% | -8.47% | -19.57% | -20.25% |
| Class E |  |  |  |  |  |  |
| Overall | -9.23% | -22.47% | -22.31% | -8.43% | -23.25% | -22.39% |
| Class D | -10.43% | -23.09% | -23.56% | -7.08% | -21.99% | -19.83% |
| Class F | -4.08% | -12.76% | -11.75% | -4.24% | -14.07% | -13.97% |

Conclusions

This contribution explores the combination of residual attention block (RAB) and transformer block (TB) into a loop filter network, and achieves an optimal trade-off between complexity and performance. Experimental results demonstrate that the proposed CNN filter achieves average {8.49%, 22.98%, 24.07%} and {9.23%, 22.47%, 22.31%} BD-rate reductions over VTM-11.0_NNVC-2.0 for {Y, Cb, Cr} channels in AI and RA configurations, respectively.

We recommend further study on the proposed CNN filter by including it in EE1.

Reference

[1]	H. Zhang, C. Jung, D. Zou and M. Li, “EE1-1.3 related: Lightweight and Efficient CNN In-loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 Document JVET-AB0090, October 2022.

[2] Zamir S W, Arora A, Khan S, et al. Restormer: Efficient transformer for high-resolution image restoration[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2022: 5728-5739.

[3]	Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

[4]	https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc

[5]	S. Liu, A. Segall, E. Alshina, and R.-L. Liao, “JVET common test conditions and evaluation procedures for neural network-based video coding technology,” JVET-W2016, April 2021.

[6]	Y. Li, K. Zhang, L. Zhang, H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz, “EE1-1.7: Combined Test of EE1-1.6 and EE1-1.3,” JVET-Z0113, by teleconference, April 2022.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).