| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0159-v1 |
| --- | --- |

| Title: | Non-EE2: IBC MBVD list derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi Zhang, Pavel Nikitin, Han Huang, Vadim Seregin, Marta Karczewicz (Qualcomm) | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to derive the IBC MBVD list from the sample positions along the MBVD directions. The MBVD candidates search starts with checking template SAD costs of offsets added to BVP along each direction with the interval of M-pel. The search process repeats around the K candidates having the lowest TM cost with the half interval. The MBVD list is derived when the interval reaches 1-pel, the K candidates with the lowest TM cost are included into the final list.

The proposed method was implemented on top of ECM-7.0 and the test results are as follows:

Max BVD offset=128-pel, direction=4, start search step=8-pel

AI: 		-0.30% (Y), -0.17% (U), -0.26% (V), 10x% (EncT), 10x% (DecT) for class F

-0.64% (Y), -0.58% (U), -0.59% (V), 10x% (EncT), 10x% (DecT) for class TGM

RA:	-0.28% (Y), -0.29% (U), -0.21% (V), 10x% (EncT), 10x% (DecT) for class F

-0.xx% (Y), -0.xx% (U), -0.xx% (V), 10x% (EncT), 10x% (DecT) for class TGM

Max BVD offset=256-pel, direction=4, start search step=4-pel

AI: 		-0.73% (Y), -0.62% (U), -0.72% (V), 10x% (EncT), 10x% (DecT) for class F

-1.10% (Y), -0.98% (U), -1.03% (V), 10x% (EncT), 10x% (DecT) for class TGM

RA:	-0.45% (Y), -0.47% (U), -0.64% (V), 10x% (EncT), 10x% (DecT) for class F

-0.xx% (Y), -0.xx% (U), -0.xx% (V), 10x% (EncT), 10x% (DecT) for class TGM

# Introduction

In ECM-7.0, IBC merge mode with block vector differences (IBC-MBVD) derives a MBVD list of 8 candidates for each BVP by selecting lowest template SAD costs of all possible positions. Surrounding the BVP position, the possible positions are two horizontal and two vertical directions, with an offset from the distance set {1-pel, 2-pel, 4-pel, 8-pel, 12-pel, 16-pel, 24-pel, 32-pel, 40-pel, 48-pel, 56-pel, 64-pel, 72-pel, 80-pel, 88-pel, 96-pel, 104-pel, 112-pel, 120-pel, 128-pel}. The MBVD index is binarized by the rice code with the parameter equal to 1.

The possible positions are a subset of the BV search area. When a BVD distance to the BVP is larger than 16, the distance set interval is 8-pel, a position in between is not considered as a possible position. When a BVD distance to the BVP is larger than 128-pel, it is also not considered as a possible position.

# Proposal

It proposes to allow adaptive BVD offsets along MBVD directions. A MBVD list of K candidates with lowest template SAD costs are derived as following steps:

- Denote the largest offset as N-pel, e.g. N=128, the number of directions D, e.g. D=4 (left, right, top, and bottom), the starting search interval M-pel, e.g. M=8, the number of candidates in MBVD list is K, e.g. K=8.

- Along each direction, check the TM SAD cost for an offset of every M-th position not exceeding N. The K lowest TM SAD cost candidates are kept in the list.

- For each candidate in the list, check the TM SAD cost of the two candidates with the offset equal to +-M/2 along the direction. The K lowest TM SAD cost candidates are kept in the list.

- Repeat step 3, while reducing the interval M by half until it reaches 1-pel.

The amount of MBVD candidates and MBVD index signaling is kept the same as ECM-7.0.

# Simulation results

Max offset N= 128-pel, direction D= 4, starting search step M = 8-pel (same configuration as in ECM-7.0)

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.30% | -0.17% | -0.26% | 10x% | 10x% | -0.28% | -0.29% | -0.21% | 10x% | 10x% |
| Class TGM | -0.64% | -0.58% | -0.59% |  |  |  |  |  |  |  |
| Overall | -0.47% | -0.38% | -0.42% |  |  |  |  |  |  |  |

Max offset N = 256-pel, direction D = 4, starting search step M = 8-pel

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.53% | -0.31% | -0.39% | 10x% | 10x% | -0.39% | -0.40% | -0.38% | 10x% | 10x% |
| Class TGM | -0.85% | -0.81% | -0.82% |  |  |  |  |  |  |  |
| Overall | -0.69% | -0.56% | -0.60% |  |  |  |  |  |  |  |

Max offset N = 256-pel, direction D = 4, starting search step M = 4-pel

|  | All Intra Main10 |  |  |  |  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.73% | -0.62% | -0.72% | 10x% | 10x% | -0.45% | -0.47% | -0.64% | 10x% | 10x% |
| Class TGM | -1.10% | -0.98% | -1.03% |  |  |  |  |  |  |  |
| Overall | -0.92% | -0.80% | -0.88% |  |  |  |  |  |  |  |

# References

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).