| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0161-v4 |
| --- | --- |

| Title: | Non-EE2: IBC adaptation for coding of natural content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Hongtao Wang, Chun-Chi Chen, Vadim Seregin, Marta Karczewicz | Email: | bray@qti.qualcomm.com hongtaow@qti.qualcomm.com chunchic@qti.qualcomm.com |
| Source: | Qualcomm. |  |  |

_____________________________

# Abstract

This contribution investigates adaptation of IBC tool for the coding of camera captured content.

The experimental results on top of ECM-7.0 are as below:

AI(Y/U/V): -0.90%/-0.91%/-0.99%, EncT 128%, DecT 100%

RA(Y/U/V): -0.34%/-0.35%/-0.40%, EncT 106%, DecT 102%

Further, RR-IBC and TM-IBC is disabled and encoding optimization is done. The results on top of ECM-7.0 are as follows:

AI(Y/U/V): -0.75%/-0.88%/-0.90%, EncT 110%, DecT 101%

Additionally fractional pel motion compensation for IBC is also investigated on top of previous result. The results on top of ECM-7.0 are as follows:

AI(Y/U/V): -0.92%/-1.08%/-1.14%, EncT 113%, DecT 101%

# Introduction

IBC in current ECM-7.0 is currently tailored to screen content coding. In JVET-AA0106, the performance of IBC for camera captured content is also investigated as part of EE2. However, one of the main issue was that the gain of RA was small with some of the classes showing coding loss. In this proposal, some non-normative and normative modifications was done to improve the coding performance of IBC for camera captured content.

# Proposed Methods

In aspect 1, the modification of IBC are as follows:

- In ECM-7.0, some partitioning depth is skipped depending on the POC distance of the current picture and nearest reference picture. When IBC is enabled from SPS level, this POC distance is set to 0. In this proposal, for inter-slices, the true POC distance is used instead of setting to 0..

- At the encoder, IBC BV search range is restricted to 32 pixels.

- IBC is not applied to non-intra slices for natural content. This is indicated by high-level syntax to remove CU level signaling of IBC flag. However, for screen content, IBC is applied to all slices.

- SPS level flag for RR-IBC and TM-IBC is introduced.

In aspect 2, Fractional-pel motion compensation is introduced to IBC to support quarter- and half-pel resolutions in addition to existing ones. The signaling of AMVR follows the signaling as of Inter Prediction. The same interpolation filters as for inter prediction is applied.

# Experimental results

Aspect 1:

The proposed method is implemented on top of ECM-7.0 [1]. For the test, for camera captured  content, IBC is only applied to I slices, and for screen content, IBC is applied for all slices. The results of All Intra and Random Access configuration are summarized in the Table 1.

Table 1:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.02% | -0.16% | 127% | 100% |
| Class A2 | -1.25% | -1.60% | -1.55% | 127% | 100% |
| Class B | -1.03% | -0.99% | -1.27% | 128% | 98% |
| Class C | -0.45% | -0.39% | -0.37% | 125% | 98% |
| Class E | -1.67% | -1.63% | -1.64% | 134% | 107% |
| Overall | -0.90% | -0.91% | -0.99% | 128% | 100% |
| Class D | -0.66% | -1.07% | -0.68% | 120% | 95% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.29% | -0.11% | 102% | 100% |
| Class A2 | -0.60% | -0.27% | -0.44% | 102% | 100% |
| Class B | -0.51% | -0.57% | -0.71% | 108% | 103% |
| Class C | -0.12% | -0.18% | -0.21% | 108% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.35% | -0.40% | 106% | 102% |
| Class D | -0.20% | -0.43% | -0.19% | 108% | 101% |
| Class F | -0.18% | -0.13% | -0.12% | 105% | 99% |

We further disabled RR-IBC and TM-IBC, and performed additional encoder optimization for natural content classes. The result compared to ECM-7.0 are as follows in Table 2.

Table 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.09% | -0.13% | 108% | 101% |
| Class A2 | -1.09% | -1.44% | -1.47% | 108% | 101% |
| Class B | -0.84% | -0.94% | -1.05% | 110% | 100% |
| Class C | -0.36% | -0.40% | -0.33% | 112% | 102% |
| Class E | -1.37% | -1.63% | -1.60% | 111% | 102% |
| Overall | -0.75% | -0.88% | -0.90% | 110% | 101% |
| Class D | -0.56% | -0.98% | -0.81% | 114% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

We also provide a different tradeoff by further speeding up the encoder. The result compared to ECM-7.0 are as follows in Table 3.

Table 3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.02% | -0.06% | 100% | 101% |
| Class A2 | -0.61% | -0.92% | -0.86% | 101% | 101% |
| Class B | -0.48% | -0.60% | -0.69% | 101% | 100% |
| Class C | -0.15% | -0.19% | -0.22% | 102% | 100% |
| Class E | -0.77% | -0.87% | -0.89% | 102% | 100% |
| Overall | -0.40% | -0.51% | -0.54% | 101% | 100% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Aspect 2:

Fractional pixel motion compensation is evaluated on top of modification associated to Table 2 results. The result compared to ECM-7.0 are as follows in Table 4:

Table 4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.11% | -0.14% | 111% | 101% |
| Class A2 | -1.38% | -1.77% | -1.82% | 111% | 101% |
| Class B | -0.98% | -1.16% | -1.27% | 114% | 101% |
| Class C | -0.42% | -0.46% | -0.48% | 116% | 101% |
| Class E | -1.83% | -2.05% | -2.09% | 115% | 101% |
| Overall | -0.92% | -1.08% | -1.14% | 113% | 101% |
| Class D | -0.64% | -1.05% | -0.73% | 116% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).