| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0162-v2 |
| --- | --- |

| Title: | EE2-5.1: Using prediction samples or residual samples for adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Changyue Ma, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Hong-Jheng Jhu, Ning Yan, Xianglin Wang | Email: | machangyue@kuaishou.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test-5.1. In the test, the performance of ALF using prediction samples (5.1a) or residual samples (5.1b) as additional inputs are evaluated. The tested methods are implemented on top of ECM-7.0 and the simulation results are summarized as follows:

Test-5.1a:

AI: -0.07%,	0.00%,	0.00%;	100%, 101%.

RA: -0.08%	, -0.04%, 0.02%	; 101%, 103%.

LB: -0.12%, -0.25%	, -0.05%; 101%, 103%.

Test-5.1b:

AI: -0.07%, 0.00%, 0.00%; 100%, 101%.

RA: -0.11%	, 0.05%	, -0.02%;	101%, 102%.

LB: -0.16%	, -0.17%, -0.20%; 101%, 103%.

# Introduction

In the ALF design of ECM-7.0 [2], online-trained filters consist of 3 kinds of filter taps: spatial taps, reconstruction-before-DBF based taps and fixed-filter-output based taps, as shown in Figure 1-1. The spatial taps (i.e., tap #0 ~ #19) located in a cross shape are followed by 3 reconstruction-before-DBF based taps (i.e., tap #26, #27, #30) and 8 additional offline-filtered taps (i.e., tap #20 ~ tap #25, tap #28, #29), resulting in 31 taps in total. Correspondingly, the specific ALF equation is as illustrated as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2025cigi,0+gi,1+i=2627cihi,0+hi,1+i=2829cigi+i=3030cihi

(1)

where Rx, y indicates the current sample; fi,j indicates the clipped difference between a neighboring sample after SAO and Rx, y; gi,j indicates the clipped difference between a neighboring fixed-filter-output sample and Rx, y; hi,j indicates the clipped difference between a neighboring sample before DBF and Rx, y, gi indicates the clipped difference between a collocated fixed-filter-output sample and Rx, y. hi indicates the clipped difference between a collocated before DBF sample and Rx, y. The filter coefficients ci, i=0,…30, are signalled.

Figure 1-1. The Currently Used Filter Shape in ALF.

At the JVET-AB meeting, ALF using prediction samples or residual samples as additional inputs are proposed [1]. The two methods were included in Exploration Experiment (EE) 2 as Test-5.1a (prediction samples) and Test-5.1b (residual samples) for further evaluation. This contribution reports the corresponding results of two EE tests.

# Proposal

In EE2-5.1, it is proposed to additionally utilize the prediction samples (5.1a) or the residual samples (5.1b) as additional inputs to the ALF.

Specifically, in 5.1a, assuming that the prediction samples are used, a filtered sample in the proposed method is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2025cigi,0+gi,1+i=2627cihi,0+hi,1+i=2829cigi+i=3030cihi+i=3131cipi+i=3232cipFilterdi

where pi is the clipped difference between the collocated prediction sample and current sample Rx, y and pFilterdi is the clipped difference between the collocated prediction sample filtered by the fixed-filter and current sample Rx, y.

Alternatively, in 5.1b, when the residual samples are used, a filtered sample is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2025cigi,0+gi,1+i=2627cihi,0+hi,1+i=2829cigi+i=3030cihi+i=3131ciri+i=3232cirFilterdi

where ri is the clipped neighboring residual sample value and rFilteredi is the clipped residual sample filtered by the fixed-filter. For both prediction and residual samples, the fixed filter reuses the offline fixed filter trained for reconstruction after SAO.

In EE2-5.1, when either prediction samples or residual samples are applied, the complete filter shape of ALF using prediction or residual samples as additional inputs is shown in Figure 2-1.

Figure 2-1 The Newly Introduced Filter Shape of EE2-5.1.

In an adaptation parameter set, a flag is signalled to indicate whether only prediction/residual based taps or both prediction/residual based taps and prediction/residual filtered by fixed filter based taps are used for the ALF. In the proposed method, this flag is always set as true at encoder for AI test, which means only prediction/residual based taps are used for the ALF; this flag is always set as false at encoder for RA/LDB test, which means both prediction/residual based taps and prediction/residual filtered by fixed filter based taps are used for the ALF.

# Simulation results

The proposed techniques were implemented on ECM-7.0 reference software [2] and tested by following the common test condition (CTC) as specified in [3]. The results of 5.1a and 5.1b compared to ECM-7.0 are shown in Table 3-1 and 3-2.

Table 3-1 Performance of EE2-5.1a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  | -0.08% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  | -0.09% |  | -0.18% |  | 0.03% |  | 100% |  | 104% |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  | -0.07% |  | 0.00% |  | 0.00% |  | 100% |  | 100% |  | -0.10% |  | -0.01% |  | -0.02% |  | 100% |  | 104% |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.07% |  | 0.00% |  | 0.00% |  | 101% |  | 102% |  | -0.05% |  | -0.01% |  | 0.01% |  | 101% |  | 103% |  | -0.10% |  | -0.17% |  | -0.21% |  | 101% |  | 104% |  |
| Class C |  | -0.08% |  | 0.00% |  | 0.00% |  | 100% |  | 102% |  | -0.09% |  | 0.01% |  | 0.05% |  | 101% |  | 103% |  | -0.21% |  | -0.05% |  | 0.05% |  | 101% |  | 102% |  |
| Class E |  | -0.07% |  | 0.00% |  | 0.00% |  | 101% |  | 102% |  |  |  |  |  |  |  |  |  |  |  | -0.02% |  | -0.64% |  | 0.09% |  | 102% |  | 103% |  |
| Overall |  | -0.07% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  | -0.08% |  | -0.04% |  | 0.02% |  | 101% |  | 103% |  | -0.12% |  | -0.25% |  | -0.05% |  | 101% |  | 103% |  |
| Class D | -0.04% | 0.03% | 0.03% | 100% | 101% | -0.05% | -0.07% | 0.20% | 103% | 103% | -0.20% | -0.77% | 0.61% | 103% | 104% |
| Class F |  | -0.08% |  | 0.01% |  | 0.01% |  | 100% |  | 101% |  | -0.09% |  | 0.05% |  | 0.24% |  | 101% |  | 103% |  | 0.10% |  | -0.47% |  | -1.06% |  | 101% |  | 103% |  |

Table 3-2 Performance of EE2-5.1b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  | -0.08% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  | -0.12% |  | -0.15% |  | 0.04% |  | 101% |  | 104% |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  | -0.07% |  | 0.00% |  | 0.00% |  | 100% |  | 100% |  | -0.14% |  | 0.16% |  | -0.06% |  | 100% |  | 103% |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.07% |  | 0.00% |  | 0.00% |  | 101% |  | 101% |  | -0.06% |  | 0.12% |  | -0.06% |  | 101% |  | 102% |  | -0.08% |  | 0.05% |  | -0.33% |  | 101% |  | 104% |  |
| Class C |  | -0.08% |  | 0.00% |  | 0.00% |  | 100% |  | 100% |  | -0.13% |  | 0.02% |  | 0.01% |  | 101% |  | 102% |  | -0.19% |  | -0.07% |  | -0.22% |  | 101% |  | 102% |  |
| Class E |  | -0.07% |  | 0.01% |  | 0.01% |  | 101% |  | 101% |  |  |  |  |  |  |  |  |  |  |  | -0.22% |  | -0.68% |  | 0.06% |  | 101% |  | 102% |  |
| Overall |  | -0.07% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  | -0.11% |  | 0.05% |  | -0.02% |  | 101% |  | 102% |  | -0.16% |  | -0.17% |  | -0.20% |  | 101% |  | 103% |  |
| Class D | -0.04% | 0.03% | 0.03% | 100% | 100% | -0.07% | 0.02% | 0.13% | 101% | 102% | -0.16% | 0.72% | 0.14% | 102% | 103% |
| Class F |  | -0.09% |  | 0.01% |  | 0.01% |  | 100% |  | 100% |  | -0.09% |  | 0.01% |  | -0.01% |  | 101% |  | 103% |  | 0.04% |  | -0.31% |  | -0.42% |  | 102% |  | 103% |  |

# References

- C. Ma, X. Xiu, C.-W. Kuo, W. Chen, H.-J. Jhu, N. Yan, X. Wang, “ AHG12: Using prediction samples or residual samples for adaptive loop filter”, JVET-AB0181, Mainz, DE, 20–28 October 2022.

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).