| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0163-v1 |
| --- | --- |

| Title: | Non-EE2: On non-separable primary transform for intra blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ning Yan, Xiaoyu Xiu, Wei Chen, Hong-Jheng Jhu, Che-Wei Kuo, Changyue Ma, Xianglin Wang | Email: | {yanning03, xiaoyuxiu, chenwei06, jhuhong-jheng, cheweikuo, machangyue, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this contribution, non-separable primary transform kernels are proposed for intra coded blocks. Specifically, besides the existing DCT-II + LFNST and adaptive MTS, additional non-separable transforms are applied to intra coded blocks with sizes ranging from 4x4 to 16x16. Furthermore, two simplification methods, namely transform kernel sharing and transform coefficient zeroing-out, are applied to reduce the computational complexity and memory usage of the proposed non-separable transform kernels.

Compared to ECM7.0 anchors, the performance of the proposed scheme is summarized as follows.

- Test #1: non-separable primary transforms

AI: -xx% (Y), -xx% (U), -xx % (V), xx% (EncT), xx% (DecT).

- Test #2: Test #1 + transform kernel sharing

AI: -xx% (Y), -xx% (U), -xx % (V), xx% (EncT), xx% (DecT).

- Test #3: Test #1 + transform coefficient zero-out

AI: -xx% (Y), -xx% (U), -xx % (V), xx% (EncT), xx% (DecT).

- Test #4: Test #1 + transform kernel sharing + transform coefficient zero-out

AI: -xx% (Y), -xx% (U), -xx % (V), xx% (EncT), xx% (DecT).

# Introduction

In ECM7.0, DCT-II + LFNST and adaptive MTS are applied to decorrelate the residual samples of intra coded blocks. In the previous JVET meetings, non-separable primary transforms have shown potentials to further improve the efficiency of intra coding [2][3].

# Proposal

This contribution proposes to utilize extra non-separable transforms for intra blocks with sizes 4x4, 4x8, 4x16, 8x4, 8x8, 8x16, 16x4, 16x8 and 16x16. For each block size, a total of 67 transform kernels are derived based on different intra prediction modes. For signaling, one additional flag is introduced after the existing MTS flag to indicate whether the proposed non-separable primary transform is applied. Specifically, when the flag is true, it indicates the non-separable transform is enabled; otherwise (i.e., the flag is false), the MTS index is further signaled to select the separable MTS transform kernels.

To reduce the computational complexity and memory usage of the proposed non-separable transform kernels, the following two simplifications are applied:

- Transform kernel sharing: In this method, the shape symmetry of rectangular blocks is utilized to reduce the number of the non-separable transform kernels. Specifically, the transform blocks with sizes 4x8, 4x16 and 8x16 shares the same transform kernels with the transform blocks with sizes of 8x4, 16x4 and 16x8.

- Transform coefficient zeroing-out: zero-out operation is applied to the non-separable transform kernel sizes 16x8, 8x16 and 16x16, where only the first 64 transform coefficients are retained while the rest coefficients are zeroed out.

# Simulation results

The proposed methods were implemented on ECM-7.0 reference software [1] and tested by following the common test condition (CTC) as specified in [4].

## Test #1: non-separable primary transfroms

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | -0.31% | -0.46% | 104% | 101% |
| Class C | -0.78% | -0.55% | -0.67% | 104% | 101% |
| Class E | -3.26% | -2.83% | -2.61% | 102% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.15% | -0.30% | -0.15% | 104% | 100% |
| Class F |  |  |  |  |  |

## Test #2: Test1 + transform kernel sharing

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.31% | -0.27% | 103% | 101% |
| Class C | -0.64% | -0.44% | -0.44% | 103% | 100% |
| Class E | -2.59% | -2.25% | -2.14% | 102% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.10% | -0.25% | -0.06% | 103% | 99% |
| Class F |  |  |  |  |  |

## Test #3: Test #1 + transform coefficient zero-out

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.07% | -0.12% | 103% | 101% |
| Class C | -0.30% | -0.16% | -0.17% | 102% | 100% |
| Class E | -2.22% | -1.75% | -1.83% | 102% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.01% | -0.01% | 101% | 99% |
| Class F |  |  |  |  |  |

## Test #4: Test #1 + transform kernel sharing + transform coefficient zero-out

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.14% | 102% | 101% |
| Class C | -0.18% | -0.07% | -0.15% | 101% | 101% |
| Class E | -1.63% | -1.40% | -1.40% | 101% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | -0.14% | 0.04% | 101% | 101% |
| Class F |  |  |  |  |  |

# Conclusion

This contribution proposes a non-separable transform scheme for intra coding. The experimental results have shown potential coding performance. It is suggested to investigate the proposed method in the next EE2.

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- J. Choi, M. Koo, J. Lim, J. Zhao, S. Kim, “AHG12: A study on non-separable primary transform”, JVET-AA064, July 2022.

- P. Garus, M. Coban, B. Ray, V. Seregin, M. Karczewicz, “Non-EE2: Non-Separable Primary Transform for Intra Coding”, JVET-AB0175, Oct. 2022.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).