| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0170 |
| --- | --- |

| Title: | Non-EE2: Fuse intra template matching prediction with intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents to fuse intra template matching prediction (Intra TMP) with intra prediction, in which the prediction block is the weighted sum of two prediction signals generated by Intra TMP and intra prediction. On top of ECM-7.0, simulation results of the proposed method are reported as below:

AI: {%, %, %; %, %};

RA: {%, %, %; %, %}.

Introduction

In ECM, intra template matching prediction (Intra TMP) is a special intra prediction mode that copies the best prediction block from the reconstructed part of the current frame, whose L-shaped template matches the current template. Since the best prediction block is generated by template matching, the coding performance of Intra TMP may be limited when the template is not similar to the current block.

Proposed method

To improve Intra TMP, it is proposed to fuse Intra TMP with intra prediction. Specially, the prediction block is the weighted sum of two prediction signals generated by Intra TMP and intra prediction, in which the intra prediction mode derived by TIMD is used for intra prediction. A CU flag is signalled whether the proposed method is used.

Simulation results

The proposed method is implemented on top of ECM-7.0 [1]. Simulations are performed following the common test conditions [2]. The simulation results compared to ECM-7.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-7.0

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.02% | -0.05% | -0.03% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.01% | -0.26% | 0.05% |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, fusion of Intra TMP and intra prediction is proposed. It is recommended to study the proposed method in EE2.

Reference

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).