| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0172-v3 |
| --- | --- |

| Title: | Non-EE2: IBC with fractional block vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Chen, Xiaoyu Xiu, Changyue Ma, Hong-Jheng Jhu, Che-Wei Kuo, Ning Yan, Xianglin Wang | Email: | {chenwei06,xiaoyuxiu,machangyue,jhuhong-jheng,cheweikuo, yanning03,xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

This contribution investigates the performance of the IBC mode with fractional BVs. Specifically, besides the existing integer BVs, two fractional BV precisions, including 1/2-pel and 1/4-pel, are introduced for the IBC model. For IBC AMVP, following the existing IBC design, additional adaptive block vector resolution (ABVR) signaling is introduced for the proposed fractional BV precisions. When any fractional BV precision is enabled, the corresponding BVPs and BVDs are in the unit of the selected precision. For IBC merge, fractional BVs are inherited through spatial neighbors.

Compared to the IBC mode in ECM-7.0, the average BD-rate performance of the fractional IBC is summarized as:

For camera captured content:

- AI: {-x.xx%, - x.xx %, - x.xx %}

For class F:

- AI: {-x.xx%, - x.xx %, - x.xx %}

For class TGM:

- AI: {-x.xx%, - x.xx %, - x.xx %}

Introduction

IBC is known to be one efficient tool for screen content coding. In ECM-7.0, IBC is limited to use only integer BVs. To further explore the coding benefits, especially for the mixed screen contents (e.g. class F) and camera-capture contents, this contribution investigates the performance of using fractional BVs for IBC mode.

Proposal

For IBC AMVP, by following the existing design of adaptive block vector resolution (ABVR), additional ABVR signaling is introduced for the proposed 1/2-pel and 1/4-pel BV precisions. When any fractional BV precision is enabled, the corresponding BVPs and BVDs are in the unit of the selected precision. At encoder side, the existing integer BV searching methods are performed first and a group of N integer BVs are then selected for further fractional refinement, where 1/2-pel and 1/4-pel BV searching are sequentially applied. The BV with minimum RD cost, either at an integer precision or fractional precision, is signaled to decoder side. In the current design, 12-tap DCT-IF interpolation filters are applied to generate the IBC prediction samples at fractional sample positions. Moreover, in case the interpolation process accesses reconstructed samples beyond the available reference area, padding operations are conducted, which are copied from the nearest integer sample position within the valid reference area.

For IBC merge mode, fractional BVs are inherited through spatial neighbors, as same as the existing IBC inheritance process in the ECM-7.0.

Simulation results

The proposed method was implemented on ECM-7.0 reference software [1] and tested by following the common test condition (CTC) as specified in [2]. The table 1 below shows the BD-rate impact of the fractional IBC mode, where the anchor is the results of ECM7.0 with IBC turned on and the test is the anchor plus the proposed fractional IBC.

Table 1  Performance of IBC with fractional BVs

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 with IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.30% | -0.30% | 108% | 99% |
| Class C | -0.17% | -0.15% | -0.17% | 109% | 100% |
| Class E | -0.99% | -0.88% | -1.02% | 108% | 100% |  |
| Overall |  |  |  |  |  |
| Class D | -0.21% | -0.33% | -0.35% | 109% | 100% |  |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

For reference, the performance of IBC with only integer BVs is shown in table 2.

Table 2  Performance of IBC with integer BVs

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM7.0 with IBC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.10% | -0.15% | 230% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | -1.07% | -1.02% | -1.28% | 169% | 100% |
| Class C | -0.48% | -0.43% | -0.50% | 162% | 102% |
| Class E | -1.76% | -1.83% | -1.81% | 173% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.70% | -0.96% | -0.65% | 146% | 102% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/VVCSoftware_VTM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Apr. 2022.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).