| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0173-v2 |
| --- | --- |

| Title: | Non-EE2: ALF classification based on residual data |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ikram Jumakulyyev, Nan Hu, Zhi Zhang, Vadim Seregin, Marta Karczewicz, Han Huang 5775 Morehouse Drive, San Diego, CA 92121, USA | Tel: Email: | ijumakul@qti.qualcomm.com nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In adaptive loop filter (ALF) process of ECM-7.0, a luma coding tree block (CTB) can reference a filter set in an adaptation parameter set. A Laplacian classifier or a band-based classifier is applied to each 2x2 reconstructed luma block in the CTB. In this proposal, a new classifier that applied to the residual samples is proposed. In another aspect, the filter shape of an online derived ALF is modified. On top of ECM-7.0 with common test condition, simulation results of the proposed methods are reported as follows.

|  | All intra |  |  | Random access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT | BD-rate Y | EncT | DecT |
| Classifier |  |  |  | -0.12% | 100% | 100% |  |  |  |
| Filter shape | -0.01% | 101% | 102% | -0.06% | 100% | 101% | -0.03% | 102% | 102% |

# Introduction

In ALF process of ECM-7.0, a luma CTB can reference an online derived filter set in an adaptive parameter set (APS). For each filter set, a flag is signalled to indicate either a Laplacian-based classifier or a band-based classifier is applied to a 2x2 reconstructed luma block. For each class, a specific filter is applied among the different filters in the filter set.

The filter shape of an online derived ALF contains 3 types of taps: taps applied to neighboring reconstructed samples (i.e., taps #0-#19), taps applied to offline-filtered results (i.e., taps #20-#27) and taps applied to samples before deblocking filter (i.e., taps #28-#30).

Figure 1. Filter shape of an online derived ALF in ECM-7.0.

# Proposal

To extend the two-classifier based ALF in ECM-7.0, in this contribution, a third classifier is added to ALF which is applied to residual samples. The classifier usage is signalled for each luma filter set in APS.

In this classifier, for each 2x2 block, the sum of absolute values of residual samples in a neighboring window is calculated, and the class index is derived as:

classIdx = sum >> (sample bit depth – 2).

The value of classIdx is in the range of 0 to 24, same as in ECM-7.0.

In another aspect, the filter shape of an online derived ALF is modified as shown in Figure 2. The number of taps (i.e., taps #0-#9) that applied to neighboring reconstructed samples is reduced and the number of taps (i.e., taps #10-#29) that applied to offline-filtered results is increased.

Figure 2. Proposed filter shape of an online derived ALF.

# Simulation results

The proposed methods were implemented on top of ECM-7.0 and tested with common test condition described in JVET-Y2017.

Table 1. Results of the proposed classifier

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | -0.01% | -0.01% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.09% | 0.04% | 100% | 101% |
| Class A2 | -0.07% | 0.01% | -0.07% | 101% | 100% |
| Class B | -0.10% | -0.07% | -0.10% | 101% | 101% |
| Class C | -0.21% | 0.03% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.03% | -0.03% | 100% | 100% |
| Class D | -0.17% | 0.03% | -0.06% | 101% | 101% |
| Class F | -0.08% | 0.03% | -0.01% | 101% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.50% | 0.20% | -0.10% | 100% | 100% |
| Class E | -0.23% | -0.63% | -0.45% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.23% | -0.25% | -0.49% | 101% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Results of proposed filter shape

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.00% | 0.00% | 100% | 102% |
| Class C | -0.03% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 102% |
| Overall | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Class D | -0.02% | 0.01% | 0.01% | 100% | 99% |
| Class F | -0.02% | 0.01% | 0.01% | 102% | 103% |
|  | Random access Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.12% | -0.03% | 102% | 102% |
| Class A2 | -0.01% | -0.05% | -0.05% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.10% | 101% | 102% |
| Class C | -0.15% | -0.04% | 0.04% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.04% | -0.04% | 100% | 101% |
| Class D | -0.34% | -0.04% | 0.08% | 99% | 100% |
| Class F | -0.06% | 0.02% | 0.09% | 101% | 101% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.12% | -0.26% | 99% | 101% |
| Class C | -0.13% | 0.48% | 0.06% | 104% | 103% |
| Class E | 0.01% | -0.02% | 0.19% | 102% | 102% |
| Overall | -0.03% | 0.10% | -0.04% | 102% | 102% |
| Class D | -0.25% | 0.19% | 0.16% | 100% | 102% |
| Class F | -0.20% | -0.57% | -0.73% | 103% | 103% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).