| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0174-v1 |
| --- | --- |

| Title: | AHG9: Multiple input pictures for neural-network post-processing filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Ye-Kui Wang, Chaoyi Lin, Junru Li, Jizheng Xu, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com yekui.wang@bytedance.com linchaoyi.cy@bytedance.com lijunru@bytedance.com xujizheng@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

Currently, the neural-network post-filter characteristics (NNPFC) SEI message specifies to use only one input picture when the post filter is for visual quality improvement, chroma upsampling, resolution upsampling, or both chroma upsampling and resolution upsampling. However, it might be beneficial to input multiple pictures for these purposes.

This contribution proposes to provide the capability of feeding multiple pictures into the neural network post-processing filter in case of all purposes.

Introduction

Using multiple input pictures has been shown to improve performance in a broad variety of image/video restoration tasks such as deep-learning based in-loop filtering [1], post filtering [2, 3], and super-resolution (SR) [4, 5, 6]. For example, compared to single-picture-based SR, networks with multiple-pictures input can either reduce the computational cost by 30% whilst maintaining the same quality or provide a 0.2dB gain for a similar computational cost according to [4]. Actually, most of state-of-the-art deep video SR methods assume multiple input pictures as reviewed in [6].

Analysis

In video super-resolution tasks, observations of a same object or scene are highly likely to exist in consecutive pictures of a video sequence. As a result, the neighboring pictures may contain the content missed when down sampling the current picture. Therefore, taking neighboring pictures as additional input may improve the SR performance of the current picture.

In visual quality enhancement tasks, the neighboring pictures might have a higher quality than the current picture since quality fluctuation usually exists across compressed pictures. Consequently, the quality of a video picture may be effectively improved by taking advantage of their adjacent pictures with higher quality.

Proposal

The proposed text changes are as follows (additions are highlighted in turquoise, and removals are highlighted in yellow strikethrough in red fonts):

8.28.1 Neural-network post-filter characteristics SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| nnpfc_purpose | ue(v) |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_purpose = = 2 ¦ ¦ nnpfc_purpose = = 4 ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| else if( nnpfc_purpose = = 3 ¦ ¦ nnpfc_purpose = = 4 ) { |  |
| nnpfc_pic_width_in_luma_samples | ue(v) |
| nnpfc_pic_height_in_luma_samples | ue(v) |
| } |  |
| else if( nnpfc_purpose = = 5 ) { |  |
| nnpfc_num_input_pics_minus2 | ue(v) |
| for( i = 0; i <= nnpfc_num_input_pics_minus2 nnpfc_num_input_pics_minus1 − 1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| } |  |
| ... |  |
| } |  |

8.28.2 Neural-network post-filter characteristics SEI message semantics

...

nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples specify the width and height, respectively, of the luma sample array of the picture resulting from applying the post-processing filter identified by nnpfc_id to a cropped decoded output picture. When nnpfc_pic_width_in_luma_samples and nnpfc_pic_height_in_luma_samples are not present, they are inferred to be equal to CroppedWidth and CroppedHeight, respectively. The value of nnpfc_pic_width_in_luma_samples shall be in the range of CroppedWidth to CroppedWidth * 16 − 1, inclusive. The value of nnpfc_pic_height_in_luma_samples shall be in the range of CroppedHeight to CroppedHeight * 16 − 1, inclusive.

nnpfc_num_input_pics_minus2 nnpfc_num_input_pics_minus1 plus 2 1 specifies the number of decoded output pictures used as input for the post-processing filter. When nnpfc_purpose is equal to 5, nnpfc_num_input_pics_minus1 shall be greater than 0.

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the post-processing filter between the i-th and the ( i + 1 )-th picture used as input for the post-processing filter.

The variables numInputPics, specifying the number of pictures used as input for the post-processing filter, and numOutputPics, specifying the total number of pictures resulting from the post-processing filter, are derived as follows:

numInputPics = nnpfc_num_input_pics_minus1 + 1
if( nnpfc_purpose  = =  5 ) {
	numInputPics = nnpfc_num_input_pics_minus2 + 2
	for( i = 0, numOutputPics = 0; i  <=  numInputPics − 2; i++ ) 	(76)
		numOutputPics  +=  nnpfc_interpolated_pics[ i ]
} else
	numInputPics = 1
	numOutputPics = 1

...

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.4: Deep In-Loop Filter with Additional Input Information, JVET-AB0073, Mainz, DE, October 2022.

- Z. Guan, Q. Xing, M. Xu, R. Yang, T. Liu, Z. Wang (2019). MFQE 2.0: A new approach for multi-frame quality enhancement on compressed video. IEEE Transactions on Pattern Analysis and Machine Intelligence, 43(3), 949-963.

- X. HoangVan, H. H. Nguyen (2020, October). Enhancing quality for VVC compressed videos with multi-frame quality enhancement model. In 2020 International Conference on Advanced Technologies for Communications (ATC) (pp. 172-176). IEEE.

- J. Caballero, C. Ledig, A. Aitken, A. Acosta, J. Totz, Z. Wang, W. Shi (2017). Real-time video super-resolution with spatio-temporal networks and motion compensation. In Proceedings of the IEEE conference on computer vision and pattern recognition (pp. 4778-4787).

- D. Liu, Z. Wang, Y. Fan, X. Liu, Z. Wang, S. Chang, X. Wang, T. S. Huang (2018). Learning temporal dynamics for video super-resolution: A deep learning approach. IEEE Transactions on Image Processing, 27(7), 3432-3445.

- H. Liu, Z. Ruan, P. Zhao, C. Dong, F. Shang, Y. Liu, L. Yang, R. Timofte (2022). Video super-resolution based on deep learning: a comprehensive survey. Artificial Intelligence Review, 1-55.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).