| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0176-v3 |
| --- | --- |

| Title: | Non-EE2: Non-Local Cross-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Zhipin Deng 8910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents methods of non-local cross-component prediction.

Method #1: Non-adjacent cross-component prediction (NA-CCP) is proposed to derive CCCM models with samples in regions non-adjacent to the current block.

Method #2: History-based cross-component prediction (H-CCP) is proposed to inherit CCCM models from stored models of CCP-coded blocks in history.

On top of ECM-7.0, simulation results of the proposed method are reported as below:

Method #1 : AI: { -0.02%, -0.90%, -0.90%, 101%, 95%}; RA:{ -0.02%, -0.65%, -0.80%, 98%, 96%}

Method #2 : AI: { -0.03%, -0.91%, -0.94%, 107%, 96%}; RA:{ }

# Introduction

Cross-component prediction (CCP) including CCLM, CCCM and their variants are adopted in ECM to exploit the cross-component correlation. With CCLM or CCCM, Training samples are always adjacent to the current block. However, the cross-component relationship of the current block may be more correlated to that of a non-local region,

# Proposed methods

Methods of non-local cross-component prediction are proposed to boost CCP by taking more advantage from non-local regions.

Method #1:

Non-adjacent cross-component prediction (NA-CCP) mode is proposed. With NA-CCP mode, Samples in regions non-adjacent to the current block can be used to derive a CCCM model for the current block. A candidate region list with 6 candidates is constructed by checking potential 8×8 regions in order. If a checked region is available, it is put into the candidate region list. The top-left positions of the potential 8×8 regions are predetermined as {(-xStep, 0), (0, -yStep), (xStep, -yStep), (-xStep, yStep), (-xStep, -yStep), (-2*xStep, 0), (0, -2*yStep), (-2 * xStep, 2 * yStep), (2 * xStep, -2 * yStep), (-2 * xStep, yStep), (xStep, -2 * yStep), (-2 * xStep, -yStep), (-xStep, -2 * yStep), (-2 * xStep, -2 * yStep), (-xStep/2, 0), (0, -yStep/2), (xStep/2, -yStep/2), (-xStep/2, yStep/2), (-xStep/2, -yStep/2)}, where xStep = Max(width, 16), yStep = Max(height, 16). Fig. 1 show some possible positions of candidate regions.

A flag is signaled to indicate whether NA-CCP is applied to a chroma block. If NA-CCP is applied, an index is signaled to indicate which candidate in the candidate region list is used to derive the CCCM model.

Fig. 1 Possible positions of candidate regions.

Method #2:

History-based cross-component prediction (H-CCP) mode is proposed. With H-CCP, a H-CCLM table and a H-CCCM table are maintained similar to the HMVP table. After decoding a CCLM or CCCM coded block, the corresponding table is updated. In the implementation of H-CCP, the size of either H-CCLM table or H-CCCM table is 6. If the current block is coded with CCLM or CCCM mode, a flag is signaled to indicate whether H-CCP is applied. If H-CCP is used, an index is further signaled to indicate which candidate model in the H-CCLM table or H-CCCM table is selected.

# Experimental results

The proposed method #1 and method #2 are implemented on top of ECM-7.0 [1] and tested following the common test conditions [2]. Encoding/decoding time is not reliable.

The simulation results of NA-CCP and H-CCP are summarized in Table 1 and Table 2, respectively.

Table 1: Coding performance of method #1

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.79% | -0.77% | 101% | 95% | -0.05% | -0.47% | -0.60% | 99% | 96% |
| Class A2 | -0.03% | -1.80% | -1.86% | 103% | 96% | -0.03% | -0.89% | -0.99% | 98% | 97% |
| Class B | -0.03% | -1.11% | -1.07% | 101% | 96% | 0.00% | -0.97% | -1.22% | 98% | 97% |
| Class C | -0.02% | -0.37% | -0.48% | 103% | 92% | 0.00% | -0.21% | -0.30% | 98% | 95% |
| Class E | -0.02% | -0.46% | -0.35% | 100% | 94% |  |  |  |  |  |
| Overall | -0.02% | -0.90% | -0.90% | 101% | 95% | -0.02% | -0.65% | -0.80% | 98% | 96% |
| Class D | -0.01% | -0.58% | -0.44% | 103% | 91% | -0.01% | -0.76% | -0.51% | 99% | 96% |
| Class F | -0.04% | -0.41% | -0.55% | 101% | 94% | -0.08% | -0.33% | -0.20% | 98% | 95% |

Table 2: Coding performance of method #2

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.89% | -1.05% | 107% | 97% |  |  |  |  |  |
| Class A2 | -0.03% | -1.83% | -1.80% | 111% | 99% |  |  |  |  |  |
| Class B | -0.03% | -1.03% | -1.08% | 105% | 98% | -0.01% | -0.86% | -0.83% | 101% | 99% |
| Class C | -0.03% | -0.46% | -0.56% | 109% | 93% | 0.01% | -0.40% | -0.35% | 99% | 98% |
| Class E | -0.02% | -0.40% | -0.22% | 105% | 95% |  |  |  |  |  |
| Overall | -0.03% | -0.91% | -0.94% | 107% | 96% |  |  |  |  |  |
| Class D | 0.01% | -0.84% | -0.55% | 108% | 92% | -0.06% | -0.86% | -0.49% | 100% | 101% |
| Class F | -0.07% | -0.64% | -0.78% | 104% | 95% | -0.09% | -0.35% | -0.61% | 101% | 99% |

# Conclusions

This contribution presents methods of non-local CCP. The proposed method contributes considerable coding gains. It is recommended to further study non-local CCP in EE2.

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).