| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0177-v1 |
| --- | --- |

| Title: | EE1-1.7: Deep In-Loop Filter with Additional Input Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents the EE test results of JVET-AB0073. The proposed technique includes two aspects:

Aspect #1: Feeding pictures from reference picture lists into the CNN filter as additional information.

Aspect #2: Flipping the input/output samples of the CNN filter.

The proposed CNN model is implemented on top of NNVC-3.0 using SADL int16 precision.

BD-rate changes of {Y, Cb, Cr} on top of NNNC-3.0 with filter set #1 enabled and NNVC-3.0 anchor (NN filters are disabled) are reportedly summarized as below:

EE1-1.7.1 (aspect #1 + aspect #2):

Compared with NNVC-3.0 filter set #1, RA: {-0.67%, 0.03%, 0.05%}, EncT 92%, DecT 90%

Compared with NNVC-3.0 anchor, RA: {-10.09%, -20.68%, -20.36%}, EncT 168%, DecT 34064%

EE1-1.7.2 (aspect #1):

Compared with NNVC-3.0 filter set #1, RA: {-0.58%, -0.02%, -0.04%}, EncT 91%, DecT 89%

Compared with NNVC-3.0 anchor, RA: {-10.01%, -20.72%, -20.43%}, EncT 167%, DecT 33870%

EE1-1.7.3 (aspect #2):

Compared with NNVC-3.0 filter set #1, RA: {-0.12%, 0.03%, 0.07%}, EncT 84%, DecT 92%

Compared with NNVC-3.0 anchor, RA: {-9.59%, -20.68%, -20.34%}, EncT 154%, DecT 34758%

Introduction

In JVET-AB0073 [1], two aspects are proposed to improve the CNN models with additional input information. First, collocated blocks from pictures in reference picture list 0 and list 1can be input to the in-loop filtering network. Second, the input samples of CNN can be flipped, and the output samples of CNN can be flipped back. In this EE test, the deep filter presented in JVET-AB0073 is implemented on top of NNVC common software NNVC-3.0 with filter set #1 enabled [2]. Impact from each aspect is studied. In addition, the training procedure of the proposed filter is crosschecked.

Proposed method

In the proposed method, only three highest temporal layers are filtered by a multiple frames-based CNN model, which takes reconstruction, prediction, collocated block from a picture in reference picture list 0, collocated block from a picture in reference picture list 1, and QP as input. For other temporal layers, the CNN models from NNVC-3.0 filter set #1 are used. To better capture geometric features, horizontal flipping can be applied to input samples of CNN. The reconstruction and all the auxiliary inputs are flipped horizontally and then fed into the CNN. The output of the CNN is then flipped back to get the final filtered samples. Other parts such as parameter selection, residual scaling, combination with deblocking, etc. remain the same as NNVC-3.0 filter set #1.

Section 2.1 - 2.3 present the CNN structure, the parameter selection, and the residue scaling, respectively. In addition, section 2.4 presents the inference and section 2.5 describes the training process.

Architecture

The backbone of the proposed inter CNN filter is shown in Fig. 1. Rec, Pred, Col_0, Col_1, and QP stand for the reconstruction, the prediction, the collocated block from picture in reference picture list 0, the collocated block from picture in reference picture list 1, and the sequence-level input quantization parameter respectively. Fig. 2 gives an illustration of Col_0 and Col_1. The numbers of feature maps and residual blocks in Fig. 1 are set as 96 and 8 respectively

Fig. 1. Architecture of the proposed CNN filtering method. M denotes the number of feature maps. R stands for the number of residual blocks.

Fig. 2. Illustration of Col_0 and Col_1.

Parameter Selection

In the proposed technique, each slice or block could determine whether to apply the CNN-based filter or not. If the CNN-based filter is determined to be applied to a slice/block, then a conditional parameter from a candidate list including three candidates derived from QP could be further decided. Denote the sequence level QP as q, the candidate list includes conditional parameters {q, q - 5, q (hor)}. For the third candidate, the input will be horizontally flipped while quantization parameter is still q. The selection process is based on the rate-distortion cost at the encoder side. Indication of on/off control as well as the conditional parameter index, if needed, are signalled in the bitstream. Other details regarding the parameter selection are provided as below.

- The granularity of the filter determination and the parameter selection is dependent on resolution and QP. In the proposed scheme, given a higher resolution and a larger QP, the determination and selection will be performed in a larger region.

- For all-intra configuration, parameter selection is disabled while on/off control is still preserved

- A shared conditional parameter is used for the two chroma components.

Residue Scaling

When a NN filter is being applied to reconstructed pictures, a scaling factor is signaled for each color component in the picture header. The difference between the input samples and the NN filtered samples (residues) are scaled by the scaling factors before being added to input samples. To enable a combination with deblocking, the input samples used in the residual scaling is the output of deblocking filtering. The residual scaling process is shown below, where RNN and RDB refer to the outputs of NN filtering and deblocking filtering respectively.

RRefine = (RNN-RDB)×w + RDB=w×RNN + (1-w)×RDB

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [3].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.56M/model, 5 models in total |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 3.1MB/model, 5 models in total |
|  | Multiply Accumulate (kMAC/pixel) | 539K/pixel (assuming frame-level input); 682K/pixel (assuming block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | 25+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. The DIV2K [4] and BVI-DVC [5] datasets are adopted to train the CNN filters of I slices and B slices, respectively. The network information in the training stage is provided in Table 2 as suggested by [3].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 50 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is integrated into NNVC-3.0 (filter set #1) and tested according to the common test conditions defined in [3]. SAO is disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering.

EE1-1.4.1 (aspect1 + aspect2):

This is a joint test of aspect #1 (using additional inputs) and aspect #2 (flipping input/output samples). BD-rate changes of {Y, Cb, Cr} compared with NNVC-3.0 with filter set #1 enabled and NNVC-3.0 anchor are summarized in Table 3 and Table 4. It can be observed that the proposed method brings on average {-0.67%, 0.03%, 0.05%} and {-10.09%, -20.68%, -20.36%} BD-rate changes for {Y, Cb, Cr} under RA configurations compared with NNVC-3.0 filter set #1 and NNVC-3.0 anchor.

EE1-1.7.2:

This is to test aspect #1 (using additional inputs). BD-rate changes of {Y, Cb, Cr} compared with NNVC-3.0 with filter set #1 enabled and NNVC-3.0 anchor are summarized in Table 5 and Table 6. It can be observed that the proposed method brings on average {-0.58%, -0.02%, -0.04%} and {-10.01%, -20.72%, -20.43%} BD-rate changes for {Y, Cb, Cr} under RA configurations compared with NNVC-3.0 filter set #1 and NNVC-3.0 anchor.

EE1-1.7.3:

This is to test aspect #2 (flipping input/output samples). BD-rate changes of {Y, Cb, Cr} compared with NNVC-3.0 with filter set #1 enabled and NNVC-3.0 anchor are summarized in Table 7 and Table 8. It can be observed that the proposed method brings on average {-0.12%, 0.03%, 0.07%} and {-9.59%, -20.68%, -20.34%} BD-rate changes for {Y, Cb, Cr} under RA configurations compared with NNVC-3.0 filter set #1 and NNVC-3.0 anchor.

Table 3. RA performance of EE1-1.7.1 (Anchored on NNVC-3.0 filter set #1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | 0.11% | 0.09% | 91% | 88% |
| Class A2 | -0.54% | -0.04% | 0.08% | 90% | 88% |
| Class B | -0.57% | -0.01% | 0.02% | 90% | 88% |
| Class C | -1.04% | 0.06% | 0.02% | 96% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.67% | 0.03% | 0.05% | 92% | 90% |
| Class D | -1.59% | 0.08% | 0.07% | 93% | 91% |
| Class F | -0.38% | 0.11% | 0.02% | 93% | 96% |

Table 4. RA performance of EE1-1.7.1 (Anchored on NNVC-3.0 anchor)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.31% | -15.82% | -18.64% | 172% | 34528% |
| Class A2 | -10.73% | -20.28% | -17.36% | 159% | 31567% |
| Class B | -9.57% | -23.01% | -21.65% | 177% | 35153% |
| Class C | -10.85% | -21.71% | -22.28% | 159% | 34326% |
| Class E |  |  |  |  |  |
| Overall | -10.09% | -20.68% | -20.36% | 168% | 34064% |
| Class D | -12.87% | -23.57% | -23.88% | 155% | 29910% |
| Class F | -5.36% | -11.70% | -10.56% | 239% | 15978% |

Table 5. RA performance of EE1-1.7.2 (Anchored on NNVC-3.0 filter set #1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.01% | 0.05% | 91% | 88% |
| Class A2 | -0.43% | -0.03% | 0.00% | 90% | 89% |
| Class B | -0.47% | 0.02% | -0.07% | 91% | 88% |
| Class C | -0.95% | -0.07% | -0.09% | 93% | 92% |
| Class E |  |  |  |  |  |
| Overall | -0.58% | -0.02% | -0.04% | 91% | 89% |
| Class D | -1.41% | -0.07% | -0.05% | 95% | 90% |
| Class F | -0.30% | 0.01% | 0.00% | 92% | 94% |

Table 6. RA performance of EE1-1.7.2 (Anchored on NNVC-3.0 anchor)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.26% | -15.92% | -18.67% | 172% | 34621% |
| Class A2 | -10.63% | -20.27% | -17.43% | 158% | 31633% |
| Class B | -9.48% | -22.99% | -21.72% | 179% | 35140% |
| Class C | -10.76% | -21.82% | -22.37% | 155% | 33492% |
| Class E |  |  |  |  |  |
| Overall | -10.01% | -20.72% | -20.43% | 167% | 33870% |
| Class D | -12.70% | -23.69% | -23.97% | 158% | 29688% |
| Class F | -5.29% | -11.78% | -10.58% | 235% | 15644% |

Table 7. RA performance of EE1-1.7.3 (Anchored on NNVC-3.0 filter set #1)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.11% | 0.07% | 87% | 94% |
| Class A2 | -0.17% | -0.04% | 0.10% | 87% | 95% |
| Class B | -0.12% | -0.01% | 0.06% | 82% | 91% |
| Class C | -0.14% | 0.09% | 0.06% | 83% | 88% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | 0.03% | 0.07% | 84% | 92% |
| Class D | -0.15% | 0.11% | 0.12% | 81% | 87% |
| Class F | -0.07% | 0.08% | 0.02% | 84% | 90% |

Table 8. RA performance of EE1-1.7.3 (Anchored on NNVC-3.0 anchor)

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -8.92% | -15.83% | -18.66% | 165% | 36932% |
| Class A2 | -10.38% | -20.28% | -17.35% | 154% | 34015% |
| Class B | -9.16% | -23.02% | -21.62% | 162% | 36101% |
| Class C | -10.04% | -21.69% | -22.24% | 137% | 32193% |
| Class E |  |  |  |  |  |
| Overall | -9.59% | -20.68% | -20.34% | 154% | 34758% |
| Class D | -11.61% | -23.55% | -23.86% | 135% | 28587% |
| Class F | -5.08% | -11.72% | -10.57% | 214% | 14923% |

Conclusions

This contribution presents the test result of a multiple frames-based deep in-loop filtering method. The proposed filtering method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt EE1-1.7 into NNVC common software.

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.4: Deep In-Loop Filter with Additional Input Information, JVET-AB0073, Mainz, DE, October 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-3.0

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz, DE, October 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).