| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0178-v1 |
| --- | --- |

| Title: | EE1-related: In-Loop Filter with Wide Activation and Large Receptive Field |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a deep in-loop filter built based on basic residual blocks with wide activation and large receptive field. The proposed filter is obtained by revising the one presented in JVET-AB0179 with two aspects of changes. First, a unified model is used to deal with different types of slices. Second, the chroma model is simplified. It is reported that kMAC/pixel is reduced by 17% and the model storage is reduced by 66% compared with filter set #1. Compared with NNVC-3.0 filter set #1 and NNVC-3.0 anchor, the proposed filter shows BD-rate changes reported as below:

Compared with NNVC-3.0 filter set #1:

RA: {-1.72%, -3.05%, -2.41%}, LB: {-2.72%, -8.64%, -10.17%}, AI: {-0.93%, 1.91%, 2.26%}

Compared with NNVC-3.0 anchor:

RA: {-11.04%, -22.99%, -22.26%}, LB: {-11.00%, -23.30%, -23.66%}, AI: {-8.37%, -18.59%, -18.84%}

Introduction

NNVC common software includes two sets of deep in-loop filter, where filter set #1 is based on a residual block containing two 3x3 convolutional layers as shown in Fig.1 (a) [1, 2]. Studies in [3, 4] have shown benefits to build a network using residual block with wide activation as shown in Fig. 1 (b). However, the receptive filter of wide residual block is restricted compared with the normal one. JVET-AB0179 [5] proposes a new type of residual block with both wide activation and large receptive field as shown in Fig. 2. To obtain a simplified and unified CNN filter, this contribution proposes to revise the method in JVET-AB0179 with two aspects of changes. First, a unified model is designed to deal with both intra slices and inter slices. Second, the chroma model is simplified by using a smaller number of feature maps and layers.

Fig. 1. (a) Vanilla residual block in [2]. (b) Wide residual block, M > K.

Proposed method

Section 2.1 - 2.4 present luma CNN structure, chroma CNN structure, inference and training process, respectively. Note that other designs of the proposed method such as parameter selection, residual scaling, combination with deblocking, etc. remain the same as the NN-based filter set #1 in NNVC-3.0.

Luma CNN structure

Fig. 2. gives the architecture of the proposed CNN filter, which comprises three types of basic blocks known as HeadBlock, BackboneBlock, and TailBlock. The design of these blocks follows the principle of wide activation, large receptive field, and multi-scale feature extraction.

HeadBlock is responsible for extracting features from input. Cin denotes the number of input channels. The proposed model takes reconstruction, prediction, slice type, and qp as input, therefore Cin is equal to 4. C stands for the basic number of feature maps and is set as 64. {C1, C2} represent numbers of output channels in large activation branch and large receptive field branch, and are set as {160, 32}. S means the stride of convolution and is set as 2 to achieve feature down-sampling. Backbone of the proposed network containing a series of BackboneBlocks achieves feature embedding. N, the number of BackboneBlocks, is set as 19 for. In the end, there is a TailBlock mapping the embedded features from backbone to the final output.

Fig. 2. Architecture of the proposed deep in-loop filter.

Chroma CNN structure

The CNN filter for chroma components has similar architecture as that for luma, but includes less BackboneBlocks and layers. In particular, {N, C, C1, C2} is set as {8, 48, 120, 24}

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1 as suggested by [6].

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 2.05M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | 4.17 MB |
|  | Multiply Accumulate (kMAC/pixel) | 446K/pixel (frame-level input) 564K/pixel (block-level input) |
| Optional |  |  |
|  | Total Conv. Layers | compact: 85 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

PyTorch is used as the training platform. DIV2K [7] and BVI-DVC [8] are adopted as the training datasets. The network information in the training stage is provided in Table 2 as suggested by [6].

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | 60 |
|  | Batch size: | 64 |
|  | Training time: | 140h |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

The proposed CNN-based in-loop filtering method is integrated into NNVC-3.0 and tested according to the common test conditions defined in [6]. SAO is disabled while ALF (and CCALF) is placed after the proposed CNN-based filtering. To better evaluate the proposed method, it is compared with both NNVC-3.0 with NN-based filter set #1 enabled and NNVC-3.0 anchor. Comparison results are shown in Table 3 ~ Table 8.

Compared with NNVC-3.0 filter set #1, the proposed method brings on average {-1.72%, -3.05%, -2.41%}, {-2.72%, -8.64%, -10.17%}, and {-0.93%, 1.91%, 2.26%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations. Compared with NNVC-3.0 anchor, the proposed method brings on average {-11.04%, -22.99%, -22.26%}, {-11.00%, -23.30%, -23.66%}, and {-8.37%, -18.59%, -18.84%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

In addition, with the proposed method, kMAC/pixel is reduced by 17% and the model storage is reduced by 66% compared with NNVC-3.0 filter set #1

Table 3. RA performance anchored on NNVC-3.0 filter set #1

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.54% | -4.93% | -4.13% | 92% | 98% |
| Class A2 | -1.57% | -2.40% | -1.01% | 91% | 99% |
| Class B | -1.67% | -2.77% | -2.24% | 90% | 99% |
| Class C | -2.03% | -2.45% | -2.38% | 97% | 102% |
| Class E |  |  |  |  |  |
| Overall | -1.72% | -3.05% | -2.41% | 92% | 100% |
| Class D | -2.05% | -1.52% | -1.62% | 97% | 106% |
| Class F | -0.86% | -3.93% | -3.88% | 98% | 105% |

Table 4. LDB performance anchored on NNVC-3.0 filter set #1

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.49% | -9.11% | -9.51% | 96% | 110% |
| Class C | -2.60% | -10.39% | -11.64% | 94% | 99% |
| Class E | -3.27% | -5.54% | -9.32% | 92% | 120% |
| Overall | -2.72% | -8.64% | -10.17% | 94% | 108% |
| Class D | -2.44% | -7.69% | -11.61% | 98% | 101% |
| Class F | -2.51% | -9.65% | -9.18% | 93% | 105% |

Table 5. AI performance anchored on NNVC-3.0 filter set #1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | 0.19% | 2.85% | 98% | 103% |
| Class A2 | -0.72% | 1.02% | 2.21% | 93% | 107% |
| Class B | -0.74% | 5.96% | 4.47% | 96% | 106% |
| Class C | -1.16% | 0.26% | 1.86% | 100% | 104% |
| Class E | -1.19% | -0.02% | -1.46% | 104% | 108% |
| Overall | -0.93% | 1.91% | 2.26% | 98% | 106% |
| Class D | -1.00% | 0.19% | 0.88% | 105% | 124% |
| Class F | 0.42% | -1.95% | -2.54% | 96% | 91% |

Table 6. RA performance anchored on NNVC-3.0 anchor

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -10.29% | -19.66% | -22.01% | 174% | 38416% |
| Class A2 | -11.64% | -22.01% | -18.25% | 161% | 35339% |
| Class B | -10.56% | -25.03% | -23.34% | 177% | 39453% |
| Class C | -11.74% | -23.68% | -24.12% | 161% | 37051% |
| Class E |  |  |  |  |  |
| Overall | -11.04% | -22.99% | -22.26% | 169% | 37751% |
| Class D | -13.27% | -24.76% | -25.22% | 162% | 34968% |
| Class F | -5.81% | -15.11% | -14.04% | 251% | 17473% |

Table 7. LDB performance anchored on NNVC-3.0 anchor

|  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -10.12% | -23.49% | -22.10% | 170% | 41671% |
| Class C | -11.51% | -24.32% | -24.64% | 144% | 35342% |
| Class E | -11.80% | -21.59% | -24.95% | 282% | 35989% |
| Overall | -11.00% | -23.30% | -23.66% | 183% | 38025% |
| Class D | -12.99% | -25.73% | -28.25% | 153% | 30476% |
| Class F | -7.29% | -17.86% | -14.95% | 231% | 17505% |

Table 8. AI performance anchored on NNVC-3.0 anchor

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -7.39% | -17.73% | -17.63% | 169% | 29192% |
| Class A2 | -7.51% | -19.41% | -15.76% | 138% | 22663% |
| Class B | -7.50% | -17.34% | -18.31% | 137% | 23860% |
| Class C | -8.67% | -18.49% | -20.55% | 131% | 15401% |
| Class E | -11.26% | -20.84% | -21.72% | 157% | 26975% |
| Overall | -8.37% | -18.59% | -18.84% | 144% | 22656% |
| Class D | -8.38% | -18.87% | -20.46% | 124% | 15602% |
| Class F | -4.88% | -13.64% | -13.34% | 119% | 17411% |

Conclusions

This contribution proposes a CNN-based in-loop filtering network. The proposed method shows favorable trade-off in terms of the coding performance and complexity. It is suggested to study the proposed method in EE.

References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-3.0.

- Y. Li, K. Zhang, J. Li, L. Zhang, H. Wang, M. Coban, A.M. Kotra, M. Karczewicz, F. Galpin, K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.6: Deep In-Loop Filter With Fixed Point Implementation, JVET-AA0111, by teleconference, July 2022.

- S. Eadie, H. Wang, M. Coban, M. Karczewicz, EE1-1.7: Capacity Ablation of CNN-based in-loop filtering, JVET-AB0164, Mainz, DE, October 2022.

- J. Yu, Y. Fan, J. Yang, N. Xu, Z. Wang, X. Wang, T. Huang, Wide activation for efficient and accurate image super-resolution, arXiv preprint arXiv:1808.08718, 2018.

- Y. Li, K. Zhang, L. Zhang, EE1-related: Deep In-Loop Filter with Wide Activation and Large Receptive Field, JVET-AB0179, Mainz, DE, October 2022.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz, DE, October 2022.

- https://data.vision.ee.ethz.ch/cvl/DIV2K/ R. Timofte, E. Agustsson, S. Gu, J. Wu, A. Ignatov, L. V. Gool.

- Ma, Di, Fan Zhang, and David R. Bull. "BVI-DVC: A Training Database for Deep Video Compression." arXiv preprint arXiv:2003.13552 (2020).

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).