| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0182-v1 |
| --- | --- |

| Title: | Non-EE2: High-Precision MV Refinement for BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar, Yuwen He, Kai Zhang, Li Zhang 8910 University Center Lane San Diego, CA 92122, USA | Email: | mehdi.salehifar@bytedance.com yuwen.he@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

High precision MV refinement is proposed for BDOF, based on the accurate BDOF parameter calculation.

In test #1, BDOF MV refinement parameters are more accurately calculated, with additional weights.

In test #2, on top of the test #1, BDOF MV refinement subblock size is reduced from 8×8 to 4×4.

Simulation results on top of the ECM-7.0 are reported as:

Test #1: RA: -0.14%, -0.17%, -0.11%, 100%, 101%

Test #2: RA: -0.25%, -0.33%, -0.28%, 106%, 115%

# Introduction

BDOF sample adjustment was studied during VVC development activity and was adopted in the final standard. It derives a motion refinement (vx, vy) for a 4x4 subblock and adjusts its samples individually. A simplified parameter derivation process was proposed in JVET-O0304 (and adopted in VTM), with 0.02% loss on VTM-5.0.

In ECM, there are 2 BDOFs: BDOF as MV refinement (3rd stage of DMVR process), and BDOF as sample adjustment (similar to VVC, but derives motion adjustment (vx, vy) for each sample separately)

# Proposed method

In this contribution, it is proposed to use the high-precision equations to derive the BDOF MV refinement parameters as:

Gx.Gx * vx  + Gx.Gy * vy  = dI . Gx.  s1 * vx + s2 * vy = s3

Gx.Gy * vx  + Gy.Gy * vy  = dI . Gy   s2 * vx + s5 * vy = s6

where Gx/Gy are the summation of the 2 horizontal/vertical gradients derived for each reference block.

Summations () are weighted sums, where weights depend on the position in the target region . The weights can also be applied to derive vx/vy in other cases.

# Experimental results

Two tests were conducted on top of ECM-7.0.

In test #1, BDOF MV refinement parameters are more accurately calculated, with the proposed high precision equations. Also weighted sum is used for all cases.

In test #2, on top of the test #1, BDOF MV refinement subblock size is reduced from 8×8 to 4×4.

The simulations were performed following the common test conditions [2]. Table I and II show the overall results of test #1 and test #2 compared to ECM-7.0.

Table I: Performance of proposed method test 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.21% | -0.03% | 101% | 101% |
| Class A2 | -0.19% | -0.13% | -0.14% | 101% | 101% |
| Class B | -0.12% | -0.13% | -0.13% | 100% | 101% |
| Class C | -0.17% | -0.21% | -0.12% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.17% | -0.11% | 100% | 101% |
| Class D | -0.32% | -0.22% | -0.05% | 99% | 99% |
| Class F | -0.08% | -0.08% | 0.08% | 99% | 101% |

Table II: Performance of Test 2; Reducing BDOF MV refinement subblock size from 8x8 to 4x4 on Test 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.29% | -0.07% | 106% | 113% |
| Class A2 | -0.32% | -0.39% | -0.40% | 106% | 115% |
| Class B | -0.25% | -0.29% | -0.34% | 106% | 116% |
| Class C | -0.32% | -0.37% | -0.29% | 106% | 115% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.33% | -0.28% | 106% | 115% |
| Class D | -0.64% | -0.46% | -0.54% | 106% | 114% |
| Class F | -0.14% | -0.17% | -0.10% | 104% | 113% |

Additional information: Performance of reducing the BDOF MV refinement subblock size from 8x8 to 4x4 in ECM-7.0 is reported in Tab. III. With roughly similar complexity as test 2, it only brings additional -0.05% gain. Meanwhile proposed test #2, brings additional -0.11% gain respect to test #1.

Table III: Performance of Reducing BDOF MV refinement subblock size from 8x8 to 4x4 in ECM-7.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over ECM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.01% | 105% | 111% |
| Class A2 | -0.08% | -0.13% | -0.17% | 105% | 114% |
| Class B | -0.04% | -0.03% | -0.15% | 105% | 114% |
| Class C | -0.07% | -0.13% | -0.21% | 105% | 113% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.07% | -0.14% | 105% | 113% |
| Class D | -0.17% | -0.26% | -0.28% | 106% | 114% |
| Class F | -0.02% | -0.10% | -0.02% | 103% | 111% |

# Conclusions

In this contribution, improvement for BDOF is proposed. With a negligible impact on encoder and decoder complexity, it provides a reasonable coding gain. It is recommended to study this new method in EE2.

# References

[1] ECM-7.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).