| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0184 |
| --- | --- |

| Title: | EE2-5.3: Combination Tests of 5.1 and 5.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenbin Yin1, Kai Zhang2, Li Zhang2 Changyue Ma, Xiaoyu Xiu, Che-Wei Kuo, Wei Chen, Hong-Jheng Jhu, Ning Yan, Xianglin Wang 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com machangyue@kuaishou.com |
| Source: | Bytedance Inc. Kwai Inc. |  |  |

_____________________________

Abstract

In this contribution, combination tests of EE2-5.1 and EE2-5.2 are reported. In EE2-5.1, the prediction samples or residual samples are introduced into ALF online-trained-filters. In EE2-5.2, several diversified extended taps are introduced into ALF online-trained-filters. There are totally 2 combination tests. The prediction samples are jointly used with diversified extended taps in EE2-5.3a while the residual samples are jointly utilized with diversified extended taps in EE2-5.3b.

On top of ECM-7.0, simulation results of EE2-5.3 are reported as below:

5.3a:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

5.3b:

AI: %, %, %, %, %.

RA: %, %, %, %, %.

LB: %, %, %, %, %.

Introduction

In ALF of ECM-7.0, online-trained filters consist of 3 kinds of filter taps: spatial taps, reconstruction-before-DBF based taps and fixed-filter-output based taps, as shown in Figure 1-1. The spatial taps (i.e., tap #0 ~ #19) located in a cross shape are followed by 3 reconstruction-before-DBF based taps (i.e., tap #26, #26, #30) and the offline-filtered taps (i.e., tap #20 ~ #25, #28, #29), resulting in 31 taps in total.

Figure 1-1 The Currently Used Filter Shape in ALF.

EE2-5.1

In EE2-5.1, it is proposed to additionally utilize the prediction samples (5.1A) or the residual samples (5.1B) as the input to the ALF.

Specifically, in 5.1A, assuming that the prediction samples are used, a filtered sample in the proposed method is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2025cigi,0+gi,1+i=2829cigi+i=2627cihi,0+hi,1+i=3030cihi+i=31Ncipi,0+pi,1+i=32NcipFilterdi,0+pFilterdi,1

where pi,j is the clipped difference between a neighboring prediction sample and current sample Rx, y and  pFilterdi,j is the clipped difference between a prediction sample filtered by the fixed-filter and current sample Rx, y.

Alternatively, in 5.1B, when the residual samples are used, a filtered sample is derived as

Rx,y=Rx, y+i=019cifi,0+fi,1+i=2025cigi,0+gi,1+i=2829cigi+i=2627cihi,0+hi,1+i=3030cihi+i=31Nciri,0+ri,1+i=32NcirFilterdi,0+rFilteredi,1

where ri,j is the clipped neighboring residual sample value and rFilteredi,j is the clipped residual sample filtered by the fixed-filter.

In EE2-5.1, when either prediction samples or residual samples are applied, 2 additional taps with 1x1 diamond shape as shown in Figure 2-1, is tested.

Figure 2-1 The Newly Introduced Filter Shape of EE2-5.1.

In an adaptation parameter set, a flag is signalled to indicate whether prediction samples/residual samples are used for the ALF. In the proposed method, this flag is always set as true at encoder.

EE2-5.2

The diversified extended taps for ALF (ALF-DET) in EE2-5.2 is shown in Figure 2-2, where spatial taps (i.e., tap #0 ~ #19),  reconstruction-before-DBF based taps (i.e., tap #26, #27, #38) and fixed-filter-output based taps (i.e., tap #20 ~ #25, #36, #37) are kept the same as the ECM-7.0, and several extended taps (i.e., tap #28 ~ #35, #39, # 40) are introduced into luma online-trained filters. The diversified extended taps can take either reference samples or samples filtered by a pre-defined-filter as the input source. The encoder could select the best input source and signal one flag in each APS to indicate which source is applied. The coefficient signaling mechanism is the same as the current design in ECM. ALF-DET is always enabled without filter shape switching.

Figure 2-2 The Newly Introduced Filter Shape of EE2-5.2.

Simulation Results

The combination tests were implemented on top of the ECM-7.0 [1]. Simulations were performed following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The results of 5.3A and 5.3B compared to ECM-7.0 are shown in Table 3-1 and 3-2.

Table 3-1 Performance of EE2-5.3A

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.01% | 0.01% | 101% | 102% | -0.11% | 0.21% | 0.09% | 101% | 102% |  |  |  |  |  |
| Class C | -0.12% | 0.00% | 0.00% | 100% | 99% | -0.11% | 0.07% | 0.10% | 102% | 101% | -0.08% | -0.07% | 0.00% | 102% | 103% |
| Class E | -0.09% | 0.00% | 0.00% | 102% | 102% |  |  |  |  |  | 0.06% | -0.03% | 0.21% | 106% | 104% |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.06% | 0.03% | 0.03% | 101% | 99% | -0.05% | -0.21% | 0.22% | 108% | 100% | 0.02% | 0.11% | 0.07% | 109% | 100% |
| Class F | -0.12% | 0.04% | 0.04% | 102% | 102% | -0.39% | 0.18% | 0.14% | 105% | 104% |  |  |  |  |  |

Table 3-2 Performance of EE2-5.3B

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.08% | 0.00% | 0.00% |  |  | -0.18% | 0.04% | 0.11% |  |  | -0.26% | -0.20% | -0.24% |  |  |
| Class E | -0.07% | 0.01% | 0.01% |  |  |  |  |  |  |  | -0.10% | -0.68% | -0.27% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.03% | 0.03% |  |  | -0.10% | 0.05% | 0.08% |  |  | -0.15% | -0.09% | 0.04% |  |  |
| Class F |  |  |  |  |  | -0.46% | 0.07% | 0.17% |  |  |  |  |  |  |  |

Conclusions

In this contribution, combination tests of 5.1 and 5.2 are reported. The combination tests can achieve promising coding performance with limited encoding time and decoding time increase. It is recommended to adopt one of the combination tests into the next version of ECM.

Reference

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

Patent Rights Declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).