| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0186-v2 |
| --- | --- |

| Title: | EE2-related: EE2-2.7 with further encoder optimizations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

Abstract

This contribution reports the performance of EE2-2.7a with further encoder optimizations. It is reported that on top of ECM-7.0, simulation results of the proposed method are as below:

Test 1: EE2-2.7a + modified condition for fast algorithms

RA: -0.11 % (Y), -0.21 % (U), -0.23 % (V), 101% (EncT), 101% (DecT)

LB: -0.15 % (Y), -0.35 % (U), -0.15 % (V), 102% (EncT), 102% (DecT)

Test 2: EE2-2.7a + modified condition for fast algorithms + CTU restriction for sbTMVP

RA: -0.10 % (Y), -0.12 % (U), -0.16 % (V), 101% (EncT), 101% (DecT)

LB: -0.11 % (Y), -0.38 % (U), -0.36 % (V), 101% (EncT), 102% (DecT)

- Introduction

In EE2-2.7, several fast algorithms are integrated, and a SPS syntax (sps_fast_sub_tmvp_enabled_flag) is signaled to indicate whether these fast algorithms are enabled or not.

In ECM-7.0, different location restrictions are imposed for TMVP and sbTMVP, where CTU row restriction is used for TMVP, and CTU restriction is used for sbTMVP. To take full advantage of the temporal motion information that is already available to TMVP, an unified CTU row restriction is imposed for both TMVP and sbTMVP in EE2-2.7. In this contribution, the impact of this location restriction modification is also tested.

- Proposed methods

In this contribution, the determination of sps_fast_sub_tmvp_enabled_flag is modified as below:

sps.setUseFastSubTmvp = ((m_sourceWidth * m_sourceHeight) > (m_intraPeriod == -1 ? 0 : 832 * 480)).

- Experimental results

Two experiments are tested on top of ECM-7.0 under the common test conditions [1]. In Test 1, the proposed encoder optimization is integrated on top of EE2-2.7a, and Test 2 provides the results of preserving the CTU restriction for sbTMVP on top of Test 1, as listed below:

Test 1: EE2-2.7a + modified condition for fast algorithms

Test 2: EE2-2.7a + modified condition for fast algorithms + CTU restriction for sbTMVP

Table 1: Coding performance of Test 1.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.28% | -0.21% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.08% | -0.14% | -0.20% | 101% | 101% |  |  |  |  |  |
| Class B | -0.09% | -0.12% | -0.25% | 101% | 101% | -0.12% | -0.46% | -0.64% | 103% | 103% |
| Class C | -0.16% | -0.34% | -0.22% | 102% | 103% | -0.27% | -0.33% | -0.06% | 103% | 103% |
| Class E |  |  |  |  |  | -0.02% | -0.20% | 0.57% | 101% | 101% |
| Overall | -0.11% | -0.21% | -0.23% | 101% | 101% | -0.15% | -0.35% | -0.15% | 102% | 102% |
| Class D | -0.13% | -0.24% | -0.20% | 104% | 103% | -0.05% | 0.49% | 0.37% | 102% | 101% |
| Class F | -0.02% | 0.03% | 0.13% | 100% | 100% | -0.13% | 0.29% | 0.18% | 102% | 102% |

Table 2: Coding performance of Test 2.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.28% | -0.11% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.06% | -0.09% | -0.10% | 101% | 101% |  |  |  |  |  |
| Class B | -0.08% | -0.04% | -0.24% | 101% | 101% | -0.12% | -0.58% | -0.45% | 102% | 102% |
| Class C | -0.15% | -0.11% | -0.13% | 103% | 102% | -0.17% | -0.09% | -0.32% | 100% | 103% |
| Class E |  |  |  |  |  | -0.02% | -0.43% | -0.26% | 101% | 102% |
| Overall | -0.10% | -0.12% | -0.16% | 101% | 101% | -0.11% | -0.38% | -0.36% | 101% | 102% |
| Class D | -0.12% | -0.29% | -0.13% | 104% | 103% | 0.05% | -0.08% | -0.02% | 102% | 101% |
| Class F | -0.03% | -0.07% | 0.11% | 100% | 100% | -0.18% | -0.13% | -0.82% | 101% | 101% |

- Conclusions

This contribution reports the performance of EE2-2.7a with further encoder optimizations. It is recommended to adopt this contribution in ECM.

- References

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).