| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0187-v2 |
| --- | --- |

| Title: | Non-EE2: Template matching-based subblock motion refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Zhipin Deng1, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents methods to refine subblock motion based on template matching (TM). Firstly, TM-based control points refinement is proposed for uni-directional affine candidates. Secondly, the motion shift to locate SbTMVP is also refined by TM. It is reported that on top of ECM-7.0, simulation results of the proposed method are as below:

RA: -0.09 % (Y), -0.12 % (U), -0.14 % (V), 107% (EncT), 102% (DecT)

LB: -0.13 % (Y), -0.12 % (U), -0.33 % (V), 111% (EncT), 108% (DecT)

- Introduction

In ECM, affine and SbTMVP modes are used to provide subblock-level motion information. In particular, affine mode is capable of representing non-translational subblock motion based on control point motion vectors (CPMVs). Regarding SbTMVP mode, the motion information from the left neighboring position is used as a motion shift, which is employed to obtain TMVPs at subblock level. In this contribution, TM is utilized to refine subblock motion.

- Proposed methods

After the candidate list is constructed for affine merge and affine AMVP modes, uni-directional affine candidates will perform CPMV offset refinement based on TM. In particular, a unified MV offset is assigned to all the CPMVs of an affine candidate, and the TM cost of the affine candidate is calculated accordingly. The optimal CPMV offset with the minimum TM cost can be used to refine the corresponding affine candidate if  CRefinedCOri<T, where CRefined  denotes the TM cost after CPMVs are refined by the optimal offset,  COri is that calculated by the original CPMVs, and T is a constant threshold.

It is also proposed to refine the motion shift of SbTMVP based on TM. When the initial motion shift is determined, it will be further refined with TM, then the refined motion shift will be utilized to derive subblock motion information. Here, CTU row restriction is imposed to get SbTMVP motion.

- Experimental results

The proposed method has been implemented on top of the ECM-7.0 [1]. Results under RA/LDB configurations are presented in Table 1. The simulations were performed in accordance with the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.24% | -0.04% | 106% | 102% |  |  |  |  |  |
| Class A2 | -0.11% | -0.04% | -0.15% | 105% | 102% |  |  |  |  |  |
| Class B |  | -0.05% |  | -0.02% |  | -0.21% |  | 107% |  | 102% |  | -0.10% |  | -0.19% |  | -0.46% |  | 109% |  | 107% |  |
| Class C |  | -0.10% |  | -0.20% |  | -0.11% |  | 108% |  | 104% |  | -0.17% |  | 0.01% |  | -0.04% |  | 113% |  | 109% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  | -0.12% |  | -0.16% |  | -0.50% |  | 112% |  | 109% |  |
| Overall |  | -0.09% |  | -0.12% |  | -0.14% |  | 107% |  | 102% |  | -0.13% |  | -0.12% |  | -0.33% |  | 111% |  | 108% |  |
| Class D |  | -0.09% |  | -0.16% |  | 0.00% |  | 108% |  | 104% |  | -0.22% |  | 0.70% |  | 0.51% |  | 114% |  | 118% |  |
| Class F |  | -0.05% |  | 0.08% |  | 0.23% |  | 105% |  | 103% |  | -0.14% |  | -0.03% |  | -0.32% |  | 108% |  | 108% |  |

- Conclusions

TM-based subblock motion information refinement is presented in this contribution. Simulation results indicate 0.09%/0.13% coding gain in RA/LDB configurations with 107%/102% and 111%/108% complexity increase. It is recommended to include the proposed methods in the next round of EE2.

- References

[1] ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).