| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0188 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0162 (EE2-5.1: Using prediction or residual samples for adaptive loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Wenbin Yin Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | yinwenbin.hit@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-AC0162[1], which proposes to use prediction/residual samples in adaptive loop filter. A final ALF sample is derived by weighting the regular ALF and the filter applied to the prediction/residual samples. In EE2-5.1a, the prediction samples are applied for ALF. In EE2-5.1b, the residual samples are introduced into ALF. The available simulation results match with those provided in JVET-AC0162.

Experimental results

The modifications on using prediction/residual samples for adaptive loop filter were implemented on top of ECM-7.0. The simulations were conducted under the common test conditions of EE2 [2]. The crosscheck results of EE2-5.1a are shown in Table 1 and the crosscheck results of EE2-5.1b are shown in Table 2.

Table 1. Results of EE2-5.1a

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.00% | 0.00% | 102% | 101% | -0.09% | -0.18% | 0.03% | 102% | 107% |  |  |  |  |  |
| Class A2 | -0.07% | 0.00% | 0.00% | 101% | 102% | -0.10% | -0.01% | -0.02% | 101% | 105% |  |  |  |  |  |
| Class B | -0.07% | 0.00% | 0.00% | 102% | 102% | -0.05% | -0.01% | 0.01% | 102% | 104% | -0.10% | -0.17% | -0.21% | 102% | 106% |
| Class C | -0.08% | 0.00% | 0.00% | 102% | 100% | -0.09% | 0.01% | 0.05% | 102% | 103% | -0.21% | -0.05% | 0.05% | 101% | 105% |
| Class E | -0.07% | 0.00% | 0.00% | 102% | 103% |  |  |  |  |  | -0.02% | -0.64% | 0.09% | 103% | 105% |
| Overall | -0.07% | 0.00% | 0.00% | 102% | 102% | -0.08% | -0.04% | 0.02% | 101% | 105% | -0.12% | -0.25% | -0.05% | 102% | 105% |
| Class D | -0.04% | 0.03% | 0.03% | 100% | 101% | -0.05% | -0.07% | 0.20% | 103% | 103% | -0.20% | -0.77% | 0.61% | 103% | 105% |
| Class F | -0.08% | 0.01% | 0.01% | 103% | 104% | -0.09% | 0.05% | 0.24% | 101% | 104% | 0.10% | -0.47% | -1.06% | 103% | 105% |

Table 2. Results of EE2-5.1b

|  | AI |  |  |  |  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.00% | 0.00% | 102% | 100% | -0.12% | -0.15% | 0.04% | 101% | 105% |  |  |  |  |  |
| Class A2 | -0.07% | 0.00% | 0.00% | 100% | 100% | -0.14% | 0.16% | -0.06% | 100% | 104% |  |  |  |  |  |
| Class B | -0.07% | 0.00% | 0.00% | 101% | 99% | -0.06% | 0.12% | -0.06% | 100% | 104% | -0.08% | 0.05% | -0.33% | 101% | 106% |
| Class C | -0.08% | 0.00% | 0.00% | 102% | 100% | -0.13% | 0.02% | 0.01% | 102% | 103% | -0.19% | -0.07% | -0.22% | 101% | 105% |
| Class E | -0.07% | 0.01% | 0.01% | 102% | 100% |  |  |  |  |  | -0.22% | -0.68% | 0.06% | 103% | 104% |
| Overall | -0.07% | 0.00% | 0.00% | 101% | 100% | -0.11% | 0.05% | -0.02% | 101% | 104% | -0.16% | -0.17% | -0.20% | 102% | 105% |
| Class D | -0.04% | 0.03% | 0.03% | 103% | 101% | -0.07% | 0.02% | 0.13% | 105% | 104% | -0.16% | 0.72% | 0.14% | 103% | 103% |
| Class F | -0.09% | 0.01% | 0.01% | 101% | 99% | -0.09% | 0.01% | -0.01% | 101% | 103% | 0.04% | -0.31% | -0.42% | 101% | 103% |

References

C. Ma, X. Xiu, C.-W. Kwo, W. Chen, H.-J. Jhu, N. Yan, X. Wang, “EE2-5.1: Using prediction or residual samples for adaptive loop filter,” Document of Joint Video Experts Team, JVET-AC0162, by teleconference, January 2023.

M. Karczewicz, and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.