| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0190-v2 |
| --- | --- |

| Title: | Non-EE2: On OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm }@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM, OBMC is always used on inter merge coded blocks if the CU is not LIC coded and W×H >=32, without encoder selection. However, OBMC may not be efficient on SCC. In this contribution, three methods are proposed to harmonize OBMC and SCC.

- Method #1 (normative change):

OBMC is adaptively enabled or disabled on inter merge coded CUs, based on the block characteristics.

- Method #2 (configuration change):

OBMC is disabled at sequence level on screen contents sequences.

- Method #3 (encoder change):

At encode, the inter AMVP mode without OBMC is preferred by shrinking its RD cost.

Simulation results based on ECM-7.0 are reported as below:

Method #1:

RA configuration: Class F 0.00%, 100%, 98%; Class TGM -1.29%, 100%, 102%

LB configuration: Class F -0.02%, 102%, 103%; Class TGM -2.09%, 100%, 102%

Method #2:

RA configuration: Class F 0.45%, 98%, 87%; Class TGM -1.32%, 97%, 97%

LB configuration: Class F 0.84%, 98%, 91%; Class TGM -2.11%, 97%, 96%

Method #3:

RA configuration: Class F -0.01%, 100%, 100%; Class TGM -0.40%, 100%, 99%

LB configuration: Class F -0.05%, 100%, 100%; Class TGM -0.70%, 100%, 99%

# Introduction

In ECM, overlapped subblock based motion compensation (OBMC) mode is allowed in inter merge and inter AMVP. For an inter AMVP coded CU, an OBMC flag is signalled at CU level, indicating whether OBMC is used on the CU. However, for an inter merge coded block, the OBMC is always applied, if the CU is not LIC coded and W×H >=32, without encoder selection and block level signalling.

# Proposed Methods

OBMC may not be efficient for text and graphics with motion content, such as TGM content. In this contribution, three methods are proposed to improve the OBMC mode.

- Method #1:

It is proposed to adaptively determine the application of OBMC for inter merge coded CUs, based on the block characteristics. The number of main gradients of an inter merge coded CU is calculated through the histogram of gradients based on prediction samples before OBMC. CUs with limited main gradients are treated as typical TGM content and no OBMC is applied on such CUs. Moreover, if there is a neighbor block coded with SCC mode, it is also treated as typical TGM content and no OBMC is applied.

- Method #2:

In this test, OBMC tool is disabled at sequence level.

- Method #3:

In this test, the encoder cost of inter AMVP without OBMC is multiplied by a factor such that inter AMVP without OBMC is more preferred at the encoder.

# Experiments

The proposed methods were implemented on top of ECM-7.0 and test results are summarized in the below tables.

From the experimental results in Table 2, turning off OBMC for all types of SCC sequences (both class F and class TGM) is not optimal, i.e., 1.32% and 2.11% bitrate reductions are observed for class TGM in RA and LB, respectively, but a noticeable loss is reported for class F. A fallback solution is to turn off OBMC only for class TGM.

Table 1. Block level adaptive OBMC (Method #1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.05% | 100% | 101% |
| Class A2 | 0.01% | 0.06% | -0.02% | 100% | 101% |
| Class B | 0.00% | 0.05% | -0.08% | 101% | 102% |
| Class C | 0.00% | -0.05% | 0.06% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.01% | 101% | 102% |
| Class D | 0.01% | -0.17% | -0.08% | 102% | 104% |
| Class F | 0.00% | 0.16% | 0.28% | 100% | 98% |
| Class TGM | -1.29% | -0.86% | -0.75% | 100% | 102% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.05% | -0.64% | 101% | 103% |
| Class C | -0.04% | -0.04% | 0.03% | 102% | 106% |
| Class E | -0.03% | -0.14% | -0.33% | 100% | 104% |
| Overall | -0.02% | -0.07% | -0.34% | 101% | 104% |
| Class D | 0.03% | 0.00% | 0.47% | 101% | 104% |
| Class F | -0.02% | 0.18% | -0.74% | 102% | 103% |
| Class TGM | -2.09% | -1.53% | -1.45% | 100% | 102% |

Table 2. Totally turn off the OBMC tool (Method #2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.22% | 0.40% | 0.68% | 94% | 80% |
| Class A2 | 0.50% | 1.38% | 1.36% | 94% | 75% |
| Class B | 0.45% | 1.15% | 0.87% | 96% | 79% |
| Class C | 0.77% | 1.34% | 1.60% | 97% | 84% |
| Class E |  |  |  |  |  |
| Overall | 0.50% | 1.10% | 1.13% | 95% | 80% |
| Class D | 0.83% | 1.35% | 1.57% | 97% | 84% |
| Class F | 0.45% | 0.93% | 1.07% | 98% | 87% |
| Class TGM | -1.32% | -0.80% | -0.70% | 97% | 97% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.55% | 0.95% | 0.74% | 95% | 85% |
| Class C | 0.85% | 1.53% | 1.36% | 96% | 89% |
| Class E | 1.02% | 0.48% | 1.53% | 96% | 89% |
| Overall | 0.77% | 1.03% | 1.14% | 95% | 87% |
| Class D | 1.56% | 2.66% | 1.89% | 97% | 86% |
| Class F | 0.84% | 1.09% | 1.01% | 98% | 91% |
| Class TGM | -2.11% | -1.44% | -1.42% | 97% | 96% |

Table 3. OBMC encoder optimization (Method #3)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.02% | -0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.02% | 0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.01% | 100% | 101% |
| Class F | -0.01% | -0.02% | -0.08% | 100% | 100% |
| Class TGM | -0.40% | -0.35% | -0.29% | 100% | 99% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.06% | -0.06% | 100% | 100% |
| Class C | 0.00% | 0.08% | -0.03% | 100% | 99% |
| Class E | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Overall | 0.01% | 0.05% | -0.03% | 100% | 99% |
| Class D | 0.03% | 0.10% | -0.06% | 100% | 100% |
| Class F | -0.05% | 0.37% | -0.28% | 100% | 100% |
| Class TGM | -0.70% | -0.56% | -0.53% | 100% | 99% |

# Conclusions

The contribution reports the results of three methods to improve OBMC for SCC. It is asserted that the proposed methods show noticeable bitrate reduction for TGM content. It is recommended to adopt one of the proposed methods in ECM.

# References

- ECM-7.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-7.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).