| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0192 |
| --- | --- |

| Title: | Non-EE2: Temporal block vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM, temporal block vector (BV) prediction is not utilized for either IBC merge or IBC AMVP mode. To further improve the efficiency of BV prediction, temporal BV prediction is proposed to mimic TMVP.

On top of the ECM7.0, simulation results of the proposed method are reported as below:

RA: Class F -0.07%, 100%, 99%; Class TGM -0.46%, 100%, 100%

LB: Class F -0.23%, 99%, 99%; Class TGM -0.51%, 100%, 100%

# Introduction

In ECM, IBC merge/AMVP candidates are derived from spatial candidates, HMVP candidates, pairwise average candidates and some predefined BV candidates. However, temporal BV candidates are not utilized.

# Proposed methods

In this contribution, temporal BV prediction (TBVP) is proposed to mimic TMVP. Temporal BV candidates are introduced to IBC merge/AMVP candidate lists. The IBC merge candidate list includes the regular IBC merge candidate list, the IBC-TM merge candidate list and IBC-MBVD base candidate list.

TBVP candidates are derived with full pruning from the same temporal positions as TMVP. They are put before the HMVP candidates.

# Experimental Results

The proposed method has been implemented on top of the ECM-7.0 [1]. Testing results of the proposed method under RA/LB configurations are summarized in Table 1. The simulations were performed following the common test conditions [2]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.07% | -0.15% | 0.02% | 100% | 99% |
| Class TGM | -0.46% | -0.39% | -0.39% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.23% | -0.16% | -0.50% | 99% | 99% |
| Class TGM | -0.51% | -0.34% | -0.46% | 100% | 100% |

- Conclusions

In this contribution, to further improve the efficiency of BV prediction, TBVP is introduced. The proposed method can bring some coding gain with almost no coding time change. It is recommended to further study of this method in the EE2.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-7.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).