| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0196-v2 |
| --- | --- |

| Title: | EE1-2.2: GOP Level Adaptive Resampling with CNN-based Super Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Tel: Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-AB0098. The proposed method in JVET-AB0098 is a GOP level adaptive resampling method with CNN-based super resolution. At the GOP level, the proposed method can adaptively select a scale factor from ×1.0 (original size) and ×2.0 (half size) to determine the encoding resolution, and the designed CNN-based super-resolution will be used for the half size. Compared with NNVC-3.0-NnlfOption=0, the experimental results show {-5.54%, -2.38%, -2.08%} and {-3.35%, -8.02%, -4.98%} BD-rate savings for {Y, Cb, Cr} under RA and AI configurations, respectively.

Introduction

Recently, many convolutional neural network (CNN)-based super resolution strategies have been studied for video coding and achieved promising average coding gains. However, it is still difficult for CNNs to achieve significant coding gains on all sequences, especially for chroma components. Based on CNN-based super resolution, this contribution proposes a GOP level adaptive resampling method to obtain a more balanced coding performance.

Proposed method

GOP level encoding resolution decision

In this proposal, one of resampling scale factors {×1.0 (original size) and ×2.0 (half size)} is selected at GOP level and the same scale factor is applied on all frames in a common one GOP. To determine the resampling factor, the PSNR value and initial QP for the first frame of GOP are exploited. Similar to JVET-Y0068 [1], the first frame of GOP is downscaled to quarter resolution and then resampled to the original resolution by using the existing RPR technology. The PSNR is calculated between the original frame and the down-up scaled frame. Considering the different characteristics between luma and chroma components, PSNRs from different components are calculated in the scale factor decision. The decision process is shown in Fig. 1.

(a) Scale factor decision for RA			   (b) Scale factor decision for AI

Fig. 1: The scale factor decision process

Specifically, the calculation formulas used is Fig. 1 are shown as follows.

D1=DnUpPSNRuv-BasePSNRuv

D2=DnUpPSNRy-BasePSNRy

D3=DnUpPSNRy-(PredefinedPSNRy-QPi-BaseQP*0.5)

where DnUpPSNRy and DnUpPSNRuv represent down-up scaled luma and chroma PSNR for the first frame of GOP, respectively. BasePSNRy and BasePSNRuv represent predefined threshold for luma and chroma PSNR, respectively. PredefinedPSNRy represents another predefined criteria for luma PSNR and BaseQP represents predefined criteria for QP. QPi is the initial QP for the first frame of GOP and SF in Fig. 1 is the determined resampling scale factor.

CNN-based super-resolution

The proposed CNN-based super-resolution method is the same as JVET-AA0084 [2]. The designed network structure for luma is shown in Fig. 2. Along with the reconstruction Ylrrec, the prediction Ylrpred, slice QP and base QP are also fed into the proposed network. Several ordinary convolutions and N residual blocks are used to obtain effective features from the input. Then, a pixel shuffle layer is utilized to output the high-resolution residuals. Finally, the RPR reconstruction Yhrrpr upsampled by RPR mechanism is adopted to generate the high-resolution reconstruction Yhrrec. Besides, it is noted that the base QP is removed for the I slices.

Fig. 2 The super resolution network architecture for luma

The designed network structure for chroma shown in Fig. 3 is similar to the network structure for luma, and the main difference is in the input part. Since the cross-component correlation is beneficial to the super resolution of chroma components, the luma reconstruction is fed into the network. Besides, the slice type is also taken as one input for better generalization.

Fig. 3 The super resolution network architecture for chroma

Network information in training stage

The network information in the training stage is shown in Table 1.

Table 1 Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla V100 32GB |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~925 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~10 days |
|  | Training data information: | TVD_4K, BVI-DVC_4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
|  | Number of iterations | 1000 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Other information: |  |

Network information in inference stage

The network information in the inference stage is shown in Table 2.

Table 2 Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma model for I slice: 1.49 M luma model for B slice: 1.49M chroma model: 1.50M |
|  | Total Number of Parameters (All Models) | 4.48M |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 3 models in total: 17.14MB |
|  | Multiplay Accumulate (MAC)/pixel | 469K |
| Optional |  |  |
|  | Total Conv. Layers | 76 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | whole frame |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Results

In the experiments, NNVC-3.0 [3] is used as the anchor and the QP values are set as {22, 27, 32, 37, 42}, according to the common test conditions [4]. The proposed GOP level adaptive resampling method is evaluated without turning on any NN filter tools. The test results with rate matching are shown as follows.

Table 3. Performance of the proposed method (RA)

|  |  |  |  | Random Access Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over NNVC-3.0 NnlfOption=0 (QP 22, 27, 32, 37, 42) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Cass A1 |  | Tango2 |  | -11.64% |  | -21.62% |  | -14.87% |  | 113% |  | 9700% |  |
|  |  | FoodMarket4 |  | -7.79% |  | -2.66% |  | -6.11% |  |  |  |  |  |
|  |  | Campfire |  | -2.93% |  | 0.25% |  | -2.74% |  |  |  |  |  |
| Class A2 |  | CatRobot1 |  | -3.14% |  | -1.73% |  | 3.88% |  | 123% |  | 1613% |  |
|  |  | DaylightRoad2 |  | -1.63% |  | -1.72% |  | 0.61% |  |  |  |  |  |
|  |  | ParkRunning3 |  | -4.87% |  | 13.18% |  | 6.72% |  |  |  |  |  |
| Average on A1 |  |  |  | -7.46% |  | -8.01% |  | -7.90% |  | 113% |  | 9700% |  |
| Average on A2 |  |  |  | -3.22% |  | 3.24% |  | 3.73% |  | 123% |  | 1613% |  |
| overall |  |  |  | -5.34% |  | -2.38% |  | -2.08% |  | 118% |  | 5657% |  |

Table 4. Performance of the proposed method (AI)

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over NNVC-3.0 NnlfOption=0 (QP 22, 27, 32, 37, 42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 | Tango2 | -9.44% | -19.91% | -17.14% | 181% | 4951% |
|  | FoodMarket4 | -6.83% | -0.87% | -3.82% |  |  |
|  | Campfire | -0.08% | 0.11% | -0.06% |  |  |
| Class A2 | CatRobot1 | -2.94% | -8.43% | 2.04% | 121% | 9663% |
|  | DaylightRoad2 | 6.68% | -29.52% | -15.84% |  |  |
|  | ParkRunning3 | -7.50% | 10.48% | 4.93% |  |  |
| Average on A1 |  | -5.45% | -6.89% | -7.01% | 181% | 4951% |
| Average on A2 |  | -1.25% | -9.16% | -2.96% | 121% | 9663% |
| overall |  | -3.35% | -8.02% | -4.98% | 151% | 7307% |

Conclusions

In this contribution, a GOP level adaptive resampling method with CNN-based super resolution is proposed. By considering the difference between luma and chroma components, the proposed resampling factor decision strategy can better adaptively select the encoding resolution between the original size and the half size. The experimental results verify that the proposed method can bring considerable coding performance.

Reference

- J. Nam, S. Yoo, J. Lim, S.  Kim, E1-2.1-related: RPR encoder with multiple scale factors, JVET-Y0068, Jan. 2022.

- R. Chang, L. Wang, X. Xu, S. Liu, “AHG11: Neural Network based Super Resolution for Video Coding Using Multiple Side Information”, JVET-AA0084, Jul. 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/NNVC-3.0.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, Oct. 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).