| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0197-v1 |
| --- | --- |

| Title: | EE1-1.1-related: More refinements on NN based in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports more refinements on NN based in-loop filter for single model and dual models. The potential performance for single model based on real-iterative training is reported in this contribution. Besides, a dual-model filter based on new training scripts is proposed to achieve better performance. Based on NNVC-3.0, the test results are shown in order of RA and AI configurations as follows.

Test 1 (single model with real-iterative training similar to EE1-1.1.3):

RA: -10.11%		-21.26%	-21.14%	EncT: 153%	DecT: 55477%

AI : -7.70%		-16.71%	-18.00%	EncT: 148%	DecT: 31664%

Test 2 (dual models retrained based on new training scripts):

RA: -x.xx%	     	-xx.xx%	-xx.xx%	EncT: xxx%	DecT: xxxxx%

AI : -x.xx%		-xx.xx%	-xx.xx%	EncT: xxx%	DecT: xxxxx%

Introduction

In this contribution, more refinements for single model and dual models are implemented on NNVC-3.0 (filter set #0) [1]. In order to further improve the performance, the single model based on real-iterative training as in EE1-1.1.3 is trained longer by using the new training scripts. The dual models are also trained based on the new training scripts. There are totally 2 models for dual-model filter, where one is for luma component and another one is for chroma components.

In brief, there are two tests as follows:

Test 1: Test the same real-iterative single model as in EE1-1.1.3 but trained longer.

Test 2: Test the dual-model filter by training model on new training scripts.

Proposed method

The network architecture is firstly presented in Section 2.1, then the network information in both training stage and inference stage is provided in Section 2.2 and Section 2.3, respectively. Section 2.4 presents the related implementation details.

Network architecture

The network structure of the proposed NN filter is shown in Fig. 1. Along with the reconstructed image (rec_yuv), additional side information is also fed into the network, such as the prediction image (pred_yuv) [2], slice QP, base QP and slice type. As for the structure of Resblock, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. Specifically, K and M are set to 64 and 160, respectively. For the network structure of single-model filter, the number of Resblock N is set to 32. For the network structure of dual-model filter, the chroma network has a similar structure to the luma network, and the number of Resblock N is set to 28 and 16 for luma network and chroma network respectively.

Fig. 1: Network architecture

Network information in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Pytorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~100 |
|  | Batch size: | 64 |
|  | Loss function: | Weighted L1 and L2 |
|  | Training time (for 1 model): | ~10 days |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Training data update strategy: |  |
|  | Other information: |  |

Network information in inference stage

In the inference stage, SADL [3] is adopted as the inference library. In addition, all the models in this contribution are quantized into int16 precision.

| Network Information in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | Filters | Test 1 | Test 2 |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL |  |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 1.9M | 1.66M for luma, 0.96M for chroma |
|  | Total Number of Parameters (All Models) | 1.9M | 2.62M |
|  | Parameter Precision (Bits) | 16 |  |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all | 3.3MB/model for luma, 1.9MB/model for chroma, 2 models in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) | 488K (w/o block extension) 618K (w/ block extension) |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers | 101 | 89 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- Multiple results are inferred by the common one model but with the different inputs {BaseQP, BaseQP – 5, BaseQP + 5} into the BaseQP interface of the network, where the index is decided at the slice-level.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in [4]. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [5].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the experiments are performed based on the common test conditions in [6]. These two test results are provided in Section 3.1 and Section 3.2 respectively. For the convenient comparison, the test results of EE1-1.1.3 and NNVC-3.0 (filter set #0) are provided in Section 3.3 and Section 3.4. The average results can also be summarized as follows.

Test 1 (single model):

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -10.11% | -21.26% | -21.14% | 153% | 55477% |
| AI | -7.70% | -16.71% | -18.00% | 148% | 31664% |

Test 2 (dual models):

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -x.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |
| AI | -x.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |

EE1-1.1.3:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -9.98% | -21.42% | -20.92% | 150% | 55833% |
| AI | -7.61% | -16.65% | -17.45% | 146% | 31664% |

NNVC-3.0 (filter set #0):

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -8.91% | -18.47% | -19.19% | 150% | 55422% |
| AI | -6.51% | -15.52% | -16.61% | 148% | 31806% |

Results of test1

Table 1. Performance of test1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.57% | -17.70% | -20.60% | 167% | 58933% |
| Class A2 | -10.51% | -20.92% | -15.87% | 159% | 57328% |
| Class B | -9.50% | -21.49% | -22.65% | 159% | 58579% |
| Class C | -10.22% | -23.89% | -23.60% | 135% | 48329% |
| Class E |  |  |  |  |  |
| Overall | -10.11% | -21.26% | -21.14% | 153% | 55477% |
| Class D | -11.47% | -22.97% | -24.27% | 133% | 42967% |
| Class F | -4.79% | -14.92% | -13.75% | 190% | 25165% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.19% | -15.38% | -17.96% | 197% | 41889% |
| Class A2 | -6.76% | -16.87% | -14.55% | 151% | 34151% |
| Class B | -6.86% | -16.59% | -17.94% | 142% | 31566% |
| Class C | -7.91% | -17.41% | -19.91% | 121% | 21819% |
| Class E | -10.28% | -17.13% | -19.05% | 150% | 36653% |
| Overall | -7.70% | -16.71% | -18.00% | 148% | 31664% |
| Class D | -7.83% | -16.01% | -19.19% | 116% | 19931% |
| Class F | -5.04% | -13.54% | -14.03% | 119% | 25533% |

Results of test2

Table 2. Performance of test2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -9.78% | -20.39% | -18.94% | 145% | 47451% |
| Class C | -10.65% | -17.24% | -16.97% | 128% | 39216% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -11.35% | -16.50% | -16.96% | 127% | 35687% |
| Class F | -5.11% | -10.66% | -10.22% | 174% | 18378% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.91% | -17.29% | -19.13% | 183% | 34841% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -7.21% | -18.84% | -19.75% | 135% | 26084% |
| Class C | -8.35% | -17.28% | -19.40% | 119% | 18182% |
| Class E | -10.61% | -20.33% | -20.88% | 143% | 30143% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -8.14% | -16.58% | -19.12% | 114% | 16660% |
| Class F | -4.84% | -13.45% | -13.04% | 116% | 20460% |

Results of EE1-1.1.3

Table 3. Performance of EE1-1.1.3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.24% | -17.67% | -20.18% | 167% | 59645% |
| Class A2 | -10.30% | -20.79% | -15.67% | 160% | 57170% |
| Class B | -9.49% | -21.85% | -22.41% | 160% | 59373% |
| Class C | -10.14% | -24.16% | -23.55% | 135% | 48339% |
| Class E |  |  |  |  |  |
| Overall | -9.98% | -21.42% | -20.92% | 154% | 55833% |
| Class D | -11.51% | -23.86% | -24.19% | 133% | 43016% |
| Class F | -4.79% | -14.78% | -14.04% | 189% | 26804% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.01% | -14.91% | -17.98% | 197% | 41491% |
| Class A2 | -6.63% | -16.95% | -14.44% | 151% | 34110% |
| Class B | -6.76% | -16.82% | -17.39% | 141% | 31584% |
| Class C | -7.88% | -16.87% | -19.30% | 121% | 22064% |
| Class E | -10.22% | -17.52% | -17.60% | 150% | 36464% |
| Overall | -7.61% | -16.65% | -17.45% | 147% | 31664% |
| Class D | -7.80% | -15.95% | -18.58% | 116% | 20016% |
| Class F | -4.83% | -12.82% | -13.29% | 120% | 25484% |

Results of NNVC-3.0 (filter set #0)

Table 4. Performance of NNVC-3.0 (filter set #0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.26% | -13.15% | -18.91% | 158% | 60002% |
| Class A2 | -9.50% | -19.53% | -14.74% | 153% | 57315% |
| Class B | -8.59% | -19.05% | -20.78% | 155% | 58726% |
| Class C | -8.62% | -20.95% | -20.76% | 135% | 47363% |
| Class E |  |  |  |  |  |
| Overall | -8.91% | -18.47% | -19.19% | 150% | 55422% |
| Class D | -9.74% | -20.01% | -21.14% | 134% | 41976% |
| Class F | -3.79% | -12.18% | -11.09% | 189% | 22973% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NnlfOption=0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.12% | -14.26% | -17.26% | 197% | 42252% |
| Class A2 | -5.73% | -15.81% | -12.98% | 151% | 34195% |
| Class B | -5.93% | -15.53% | -17.44% | 142% | 31665% |
| Class C | -6.40% | -16.42% | -18.16% | 121% | 21963% |
| Class E | -8.83% | -15.25% | -16.12% | 151% | 36759% |
| Overall | -6.51% | -15.52% | -16.61% | 148% | 31806% |
| Class D | -6.41% | -15.30% | -18.56% | 117% | 20036% |
| Class F | -4.02% | -11.81% | -11.71% | 119% | 26418% |

Conclusions

In this contribution, more refinements for single-model filter and dual-model filter are proposed to further improve the performance of NNVC-3.0 (filter set #0). The single-model filter based on real-iterative training is trained longer than EE1-1.1.3. Additional 1.20% (8.91% to 10.11%) and 1.19% (6.51% to 7.70%) BD-rate luma gains are observed under RA and AI configuration without any increase in both MAC result and memory size. The dual-model filter is designed and trained based on the new training scripts. Additional x.xx% (8.91% to x.xx%) and x.xx% (6.51% to x.xx%) BD-rate luma gains are observed under RA and AI configuration.

References

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tags/NNVC-3.0.

- H. Zhu, X. Xu, S. Liu. Deep learning based in-loop filter with QP and residual distribution information. AVS-M5654, Aug. 2020.

- F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AB0086, 28th Meeting, Mainz, DE, 20–28 October 2022.

- H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

- K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, Mainz, DE, 20–28 October 2022.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).