| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0200-v1 |
| --- | --- |

| Title: | AhG12 – Dynamic CABAC models |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Federico Lo Bianco Franck Galpin Edouard François | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

A dynamic CABAC model is presented where a set of CABAC parameters modifications can be optimally selected at encoder in a set of N choices. The method allows to better adapt the CABAC model of each context to the data content without additional complexity at the decoder. It also allows to decrease the dependencies between the CABAC performance and the fixed CABAC parameters. The following results are reported:

Test 1: AI RA:

Test 2: AI RA

without additional encoding or decoding runtime.

# Introduction

In Enhanced Compression Model (ECM), several improvements have been done on the CABAC model. These improvements allow a better adaptation of the CABAC to the content. Some adaptations were done by increasing the number of parameters of the model, while other propose to dynamically adapt the model to the data content.

It is proposed to add more dynamic adaptation to the data content without increasing the decoder complexity.

# Current CABAC model

Figure 1 Current CABAC model overview

In current ECM, the CABAC model uses 9 parameters. For a given P or B slice, these parameters can be chosen among 3 sets. The Figure 1 shows an overview of the current CABAC model process in ECM-7.0:

- Grey boxes denote computation

- Black text denotes computed parameters

  - P0 and P1 denotes the internal states

  - P0’ P’1 denotes the states used for final probability computation

  - w’0 and w’1 denote the updated window size for each state

- Red text denotes model parameters (fixed during a slice encoding/decoding):

  - P0i and P1i are the initial states for the CABAC

  - w0 and w1 are the windows size for each state

  - a is the weight between the 2 states

  - dw00, dw01, dw10, dw11 are the rate offset modifying the windows size depending on the bin

In ECM the following changes were added to the CABAC design of VTM:

- The model parameters can be reloaded from a previous slice at same TiD level.

- The weight between the 2 states (a in the figure) can be chosen per context.

- The 2 states window sizes depend on the previous bin to compute the current final state.

The first change does not increase the number of computations. The second change slightly increases the computation by adding 2 multiplications for each probability update, while the third change doubles the number of computations.

The switch to decide which CABAC parameters set is deduced as follows:

- First, for non intra slice, it is signaled by the slice header flag cabac_init_flag which allows to switch between the B and P CABAC parameters set.

- Second, when a previously decoded non intra slice of same TiD and base QP is available, the parameters of the model are carried over to decode the current slice.

# CABAC model changes

Figure 2 CABAC process changes

The Figure 2 illustrates in green the additional changes brought to the ECM CABAC.

It is proposed to add offsets to “default” CABAC parameters at the beginning of each frame. More precisely, at decoder side:

- Default CABAC parameters are selected (using current ECM-7.0 method)

- An index i_cfg signaled by the encoder is read. If no index is transmitted, the default model is ised.

- For each CABAC context and each CABAC parameter (initial states p0 and p1, windows w0 and w1), an offset is derived from i_cfg and the context index through a LUT

- Offsets are added to these default parameters and used to decode the slice.

At decoder side, the new CABAC parameters are updated once per slice and thus do not change the current CABAC process. The proposed tests do not increase the number of necessary computations in the CABAC process at decoder side, as only the initialization of CABAC models is affected.

## Encoder changes

At encoder side, the default configuration (no offset) as well as the configuration using the ith offset are tested after the encoding has been done. This way, no additional complexity is added at encoder, except for the computation of the cost using each model. The best cost is selected, and the index is transmitted in the slice header.

## Example

| i_cfg | Ctx 0 | Ctx 1 | Ctx 2 | Ctx 3 | Ctx 4 | Ctx 5 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | +1 | 0 | -1 | -1 | 0 | 0 |
| 1 | +1 | 0 | +2 | -1 | -2 | +1 |

For example, the above table depicts a possible LUT for the offset of the first window w0 in a test with 2 possible configurations (signaled through i_cfg) and 6 CABAC contexts. The first configuration corresponds to an offset of +1 (resp, 0, -1, -1, 0, 0) to be added to the default w0 of the 0th (resp., 1st, 2nd, 3rd, 4th, 5th) context; similarly, the second configuration corresponds to an offset of +1 (resp, 0, +2, -1, -2, +1) to be added to the default w0 of the 0th (resp., 1st, 2nd, 3rd, 4th, 5th) context.

At encoder side, the best model between no offset, i_cfg equals 0 and i_cfg equals 1 is selected and signaled.

# Training

The LUTs which allow to associate to each configuration and each CABAC context the corresponding offsets of CABAC parameters are obtained using a training on the BVI dataset.

More precisely, for each frame and each CABAC context, the total (fractional) bitrate for every possible combination of parameters corrections is dumped. Each configuration is initialized to random offsets for each context. The training itself is performed by alternating the following two steps:

- Given the current offsets LUTs, derive the optimal configuration for each frame

- Given the optimal configuration for each frame, derive the optimal offsets LUTs

In step 1, the “optimal” configuration is the one yielding the lowest bitrate for the given frame; while in step 2, the “optimal” offsets LUTs are the ones yielding the best mean percentage gain on the whole dataset.

After training for a few cycles using randomly initialized configurations, the default configuration is also added to the possibilities in step 1. At this point, the difference in signaling is also taken into account.

# Simulation results

Two tests were performed: test1 uses 4 choices for the offset (+ the default no offset choice), and test 2 uses 8 offsets.

The encoding and decoding time are not reliable but partial tests on a stable machine show no increase in runtime for both encoder and decoder.

It should be noted that the observed BD-rates variations are of same range as the ones obtained from the current ECM7.0 CABAC changes described in section 2.

## Test 1 - 4 offsets model

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.04% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 102% | 101% |
| Class E | -0.02% | -0.02% | -0.02% | 103% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.03% | -0.03% | 102% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.42% | 103% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.03% | 0.30% | 0.30% | 100% | 101% |
| Class C | -0.02% | 0.00% | 0.15% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.13% | -0.19% | 0.10% | 101% | 101% |
| Class F | 0.02% | 0.01% | 0.14% | 101% | 104% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Test 2 - 8 offsets model

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.04% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | -0.01% | -0.01% | 101% | 101% |
| Class C | -0.01% | -0.01% | -0.01% | 103% | 100% |
| Class E | -0.02% | -0.02% | -0.02% | 102% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.03% | -0.03% | 101% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-7.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.03% | 0.41% | 98% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.04% | 0.25% | 0.27% | 102% | 99% |
| Class C | -0.04% | 0.10% | 0.18% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.14% | -0.05% | 0.25% | 101% | 101% |
| Class F | -0.04% | -0.02% | 0.09% | 102% | 100% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

The proposed method gives additional gains similar to the already proposed modification on CABAC in ECM, without additional complexity during the CABAC process. It is proposed to study this tool in EE2.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).