| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0203-v2 |
| --- | --- |

| Title: | AHG12: On LMCS luma mapping in template processing for IBC TM-based tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Filippov, V. Rufitskiy | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

This contribution presents a technique of harmonizing the luma mapping process with the mechanism of computing template matching (TM) costs for IBC TM-based tools that use templates of a current block and candidate reference blocks in different domains, i.e. before and after LMCS inverse luma mapping. This proposal aims at aligning domains where TM costs are calculated for templates of the current block and the candidate reference blocks in P- and B-slices. We investigate 2 methods of how to perform this alignment:

- Method 1: TM costs are computed in spatial domain (i.e. after LMCS inverse luma mapping);

- Method 2: TM costs are computed in LMCS (mapped) domain (i.e. before LMCS inverse luma mapping).

Both methods were implemented and tested on top of ECM-7.0. Reportedly, the following BD-rate gain results are obtained for both methods:

- Method 1: RA configuration: {-0.00%, 0.01%, 0.08%} for class F and {-0.02%, -0.05%, -0.02%} for class TGM; LDB configuration: {-0.03%, 0.03%, -0.25%} for class F and {-0.07%, 0.06%, 0.01%} for class TGM;

- Method 2: RA configuration: {0.01%, 0.05%, 0.06%} for class F and {-0.03%, -0.05%, -0.03%} for class TGM; LDB configuration: {0.07%, 0.06%, -0.19%} for class F and {0.00%, 0.22%, 0.22%} for class TGM.

# Introduction

ECM-7.0 includes IBC TM-based tools (e.g., described in [1]) that require TM cost calculation using the templates of a current block and candidate reference blocks. However, it turns out that the template of the current block and the template of the candidate reference block can be in different domains, i.e. before and after LMCS inverse luma mapping. For example, luma samples belonging to the template of the current block can be in spatial domain (i.e. after LMCS inverse luma mapping) whereas luma samples belonging to the template of the candidate reference blocks can be in LMCS (mapped) domain (i.e. before LMCS inverse luma mapping). Note that this inconsistency happens just for P- and B-slices whereas the templates used by IBC TM-based tools are always in LMCS (mapped) domain. This proposal aims at aligning the domains where TM costs are calculated for the templates of the current block and the candidate reference blocks in P- and B-slices.

# Proposed methods

We investigate 2 methods of how to address the inconsistency in the mechanisms of computing TM costs between the templates of the current block and the candidate reference blocks in P- and B-slices are computed:

–	Method 1: TM costs are computed in spatial domain (i.e. after LMCS inverse luma mapping);

–	Method 2: TM costs are computed in LMCS (mapped) domain (i.e. before LMCS inverse luma mapping).

In Method 1, we apply LMCS inverse luma mapping to the templates in P- and B-slices before computing TM costs for all the IBC-related tools that use TM. Hence, calculations of TM costs between the templates are carried out in spatial domain.

On contrary, in Method 2, calculations of TM costs are carried out before applying LMCS inverse luma mapping to the templates of both the current block and the candidate reference blocks in P- and B-slices. Thus, TM costs are computed in LMCS domain for all the IBC-related tools that use TM.

# Simulation results

Both methods were implemented on top of ECM-7.0 and are tested under CTC for enhanced compression tool testing [2] for Random-Access and Low-Delay B configurations as the modifications are only applicable to P- and B-slices. The simulation results for Method 1 and Method 2 are shown in Table 1 and Table 2, respectively.

Table 1. The simulation results of Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.01% | 0.08% | 100% | 100% |
| Class TGM | -0.02% | -0.05% | -0.02% | 100% | 100% |
| Overall | -0.01% | -0.02% | 0.03% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.03% | 0.03% | -0.25% | 100% | 100% |
| Class TGM | -0.07% | 0.06% | 0.01% | 100% | 100% |
| Overall | -0.05% | 0.04% | -0.12% | 100% | 100% |

Table 2. The simulation results of Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.01% | 0.05% | 0.06% | 100% | 100% |
| Class TGM | -0.03% | -0.05% | -0.03% | 100% | 100% |
| Overall | -0.01% | 0.00% | 0.02% | 100% | 100% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.07% | 0.06% | -0.19% | 100% | 100% |
| Class TGM | 0.00% | 0.22% | 0.22% | 100% | 100% |
| Overall | 0.04% | 0.14% | 0.01% | 100% | 100% |

# Conclusion

This contribution proposes mechanism of aligning domains where TM costs are calculated for IBC TM-based tools in P- and B-slices. We recommend adopting one of the proposed methods into the next version of ECM.

# References

- A. Robert, K. Naser, T. Poirier, Y. Chen, and F. Galpin, “EE2-3.4: IBC with Template Matching”, JVET-Z0084, April 2022.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).