| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0234-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AC0143 (EE1-1.6: NN chroma model without partitioning input) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Maria Santamaria Francesco Cricri Nokia Technologies Tampere, Finland | Email: | {maria.santamaria_gomez, Francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the cross-check (training and inference) results of JVET-AC0143 (EE1-1.6) related to a NN-based intra chroma loop filter. The training was cross-checked by means of using the models trained by the cross-checker during inference. It is reported that, compared to the proponents’ results, the overall BD-rate deviates up to 0.05% for luma and up to 0.20% for chroma.

# Introduction

JVET-AC0143 [1] studies an intra chroma loop filter, which follows the simplifications defined in JVET-AB0053 [2] for an intra luma filter. Specifically, the partitioning data as input and the spatial attention are removed from the neural network.

Both the training and inference codes are available at [3].

The training environment has Ubuntu 20.04.3 LTS, an NVIDIA GPU A100-SXM-80GB, CUDA Toolkit 11.3.1 and PyTorch 1.12.1.

After the training was done, both models were converted to SADL format (int16) for inference.

# Cross-check results

The inference simulation ran on a cluster with CentOS Linux (Core), Intel Xeon Gold 6154 CPUs and GCC 9.3.1.

## Performance on top of NNVC 3.0

For the anchor, the NN filters in NNVC 3.0 were not enabled.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -8,88 % | -15,86 % | -18,54 % | -9,16 % | -18,39 % | -20,82 % | 196 % | 39368 % | 1 % |
| Class A2 | -10,23 % | -20,29 % | -17,41 % | -9,91 % | -20,26 % | -16,42 % | 187 % | 37177 % | 1 % |
| Class B | -9,07 % | -22,96 % | -21,77 % | -8,76 % | -22,76 % | -22,05 % | 190 % | 38265 % | 1 % |
| Class C | -9,92 % | -21,93 % | -22,44 % | -9,45 % | -19,47 % | -21,49 % | 160 % | 35841 % | 1 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -9,49 % | -20,73 % | -20,43 % | -9,26 % | -20,51 % | -20,53 % | 182 % | 37600 % | 1 % |
| Class D | -11,46 % | -23,77 % | -24,22 % | -8,81 % | -23,54 % | -21,25 % | 153 % | 32226 % | 1 % |
| Class F | -5,02 % | -12,09 % | -11,26 % | -5,85 % | -13,78 % | -13,26 % | 248 % | 16538 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -7,83 % | -15,94 % | -13,72 % | -7,82 % | -16,33 % | -12,90 % | 188 % | 34984 % | 2 % |
| Class C | -9,13 % | -15,35 % | -14,70 % | -9,54 % | -10,63 % | -10,25 % | 160 % | 34197 % | 3 % |
| Class E | -8,68 % | -17,33 % | -17,15 % | -9,02 % | -18,08 % | -18,69 % | 317 % | 30075 % | 2 % |
| Overall | -8,48 % | -15,68 % | -14,15 % | -8,69 % | -13,80 % | -11,73 % | 203 % | 33432 % | 2 % |
| Class D | -10,86 % | -19,25 % | -18,32 % | -9,75 % | -19,46 % | -14,62 % | 153 % | 28433 % | 3 % |
| Class F | -4,93 % | -9,88 % | -6,40 % | -6,69 % | -7,93 % | -7,45 % | 251 % | 16878 % | 2 % |
| Class H |  |  |  |  |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | bit DIFF |
| Class A1 | -6,58 % | -18,00 % | -19,35 % | -6,36 % | -20,74 % | -22,21 % | 176 % | 25304 % | 1 % |
| Class A2 | -6,85 % | -20,24 % | -17,98 % | -7,30 % | -21,76 % | -17,38 % | 148 % | 20739 % | 1 % |
| Class B | -6,82 % | -21,76 % | -21,95 % | -6,75 % | -23,31 % | -23,68 % | 141 % | 19407 % | 1 % |
| Class C | -7,61 % | -19,02 % | -22,25 % | -7,56 % | -21,87 % | -24,71 % | 125 % | 13341 % | 0 % |
| Class E | -10,20 % | -21,26 % | -21,04 % | -10,51 % | -21,11 % | -23,34 % | 144 % | 21320 % | 0 % |
| Overall | -7,52 % | -20,19 % | -20,77 % | -7,58 % | -21,94 % | -22,56 % | 144 % | 19169 % | 1 % |
| Class D | -7,46 % | -19,37 % | -21,48 % | -6,66 % | -23,71 % | -24,28 % | 121 % | 11913 % | 0 % |
| Class F | -5,28 % | -12,32 % | -12,06 % | -5,23 % | -15,94 % | -17,35 % | 121 % | 16347 % | 0 % |
| Class H |  |  |  |  |  |  |  |  |  |

## Comparison against proponent’s results

While there are differences between the results reported by the proponent and the cross-checker, on average the BD-rate deviates up to 0.05% for luma and up to 0.20% for chroma.

Notice the anchor simulations ran in a different computing environment.

|  | Random access Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over JVET-AC0143 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | -0,01 % | 0,01 % |
| Class A2 | -0,01 % | -0,11 % | 0,04 % |
| Class B | 0,01 % | -0,21 % | 0,03 % |
| Class C | 0,01 % | 0,06 % | -0,07 % |
| Class E |  |  |  |
| Overall | 0,00 % | -0,08 % | 0,00 % |
| Class D | 0,02 % | -0,24 % | -0,55 % |
| Class F | 0,05 % | -0,09 % | 0,04 % |
| Class H |  |  |  |

|  | Low delay B Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over JVET-AC0143 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | -0,01 % | 0,21 % | 0,05 % |
| Class C | 0,07 % | -0,23 % | 0,16 % |
| Class E | 0,12 % | -0,55 % | 0,41 % |
| Overall | 0,05 % | -0,12 % | 0,17 % |
| Class D | -0,11 % | -0,52 % | 0,21 % |
| Class F | 0,03 % | -0,33 % | -0,19 % |
| Class H |  |  |  |

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over JVET-AC0143 |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | 0,00 % | -0,28 % | 0,25 % |
| Class A2 | -0,01 % | -0,09 % | 0,09 % |
| Class B | 0,00 % | -0,39 % | -0,03 % |
| Class C | 0,00 % | 0,20 % | -0,13 % |
| Class E | 0,00 % | -0,43 % | -0,18 % |
| Overall | 0,00 % | -0,20 % | -0,01 % |
| Class D | 0,00 % | -0,16 % | -0,25 % |
| Class F | 0,00 % | 0,03 % | 0,20 % |
| Class H |  |  |  |

# References

- J. Ström, D. Liu, K. Andersson, P. Wennersten, M. Damghanian, R. Yu. EE1-1.6: NN chroma model without partitioning input. JVET-AC0143. January 2023.

- J. Ström, D. Liu, K. Andersson, P. Wennersten, M. Damghanian, R. Yu. EE1-1.2: NN intra model without attention and partitioning strength. JVET-AB0053. October 2022.

- https://vcgit.hhi.fraunhofer.de/jvet-ab-ee1/VVCSoftware_VTM/-/tree/EE1-1.6

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.