| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0242-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0059 (AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | chunchic@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reported the crosscheck results of JVET-AC0059 (AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)). The cross-checker confirms that the software provided by the proponents matches the algorithm description of JVET-AC0059. It is also reported that all the testing results available in this crosscheck report match those provided by the proponent.

# Simulation Results

The crosscheck has been performed for All Intra and Random Access configurations with the EE2 software (i.e., ECM-7.0) and EE2 common test conditions (JVET-AB2024) of screen content coding. The corresponding simulations were conducted on an Intel Xeon cluster with Linux OS and a GCC 7.3.0 compiler. The test, proposed by the proponent of JVET-AC0059, has been crosschecked.

According to the software package provided by the proponents, it extends the notion of reconstruction reordered IBC (RRIBC) to Intra Template Matching (IntraTMP) and thus the IntraTMP in JVET-AC0059 can now switch adaptively among None, Horizontal and Vertical modes. An index for mode selection is optimized during the search process of the three IntraTMP mode, and thus no signaling overhead is required at CU level to support the flipping feature. Besides, a 1-D search process is applied to IntraTMP for flipping modes. The resulting block vector and flip mode are stored in the motion field of current frame and may be inferred in subsequent CU’s to perform block vector prediction.

It is confirmed that the software implementation perfectly matches the algorithm description of JVET-AC0059. The simulation results produced by the cross-checker in the following table match those provided by the proponents.

- Performance results of the proposed method relative to ECM-7.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.19% | 0.05% | -0.17% | 101% | 101% |
| Class TGM | -0.31% | -0.28% | -0.28% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
| Class F | -0.10% | -0.20% | -0.25% | 101% | 100% |
| Class TGM | -0.11% | -0.22% | -0.10% | 100% | 101% |

# Conclusion

The cross-checker have verified the description and results reported in JVET-AC0059. It is confirmed that the software provided by the proponent matches the description of the proposal, and the testing results produced by the cross-checker match the reported results. From cross-checker’s point of view, JVET-AC0059 provide a simple yet effective extension to the current IntraTMP design to make its prediction signals flip-able, just like the way RRIBC does. Since most of the process could be borrowed from RRIBC (such as signal flipping, BV adjustment), hardware decoder-wise impact could be minimized.

Accordingly, it is encouraged to further study the method proposed in JVET-AC0059 in an EE.