| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0254-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0155 (EE1-1.9: Reduced complexity CNN-based in-loop filtering) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li | Tel: Email: | yue.li@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of EE1-1.9. In EE1-1.9, the floating- and fixed- point, SADL-based inference results are reported for the simplified N=24 backbone block model.

The crosschecked results slightly deviate from match those provided in JVET-AC0155. It is asserted by the proponents of JVET-AC0155 that the deviation is due to the difference about EncDbOp configuration parameter. Proponents are expected to use the same EncDbOpt configuration parameter as crosschecker to obtain matched results.

Experimental results

JVET-AC0155 [1] was evaluated using NNVC CTC [2]. The crosscheck results are summarized in Table 1 ~ Table 4.

Table 1. Performance under RA configuration (SADL fixed-point)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |  |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -11.79% | -9.63% | -16.31% | -18.78% | 233% | 57128% |
| Class A2 | -12.15% | -10.63% | -19.99% | -15.24% | 219% | 53593% |
| Class B | -12.36% | -9.72% | -21.90% | -22.51% | 231% | 58599% |
| Class C | -13.33% | -10.87% | -22.85% | -23.09% | 207% | 55248% |
| Class E |  |  |  |  |  |  |
| Overall | -12.46% | -10.19% | -20.65% | -20.47% | 222% | 56378% |
| Class D | -14.92% | -12.37% | -24.06% | -25.85% | 199% | 51295% |
| Class F | -8.01% | -6.00% | -15.57% | -15.13% | 323% | 29868% |

Table 2. Performance under AI configuration (SADL fixed-point)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |  |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.65% | -6.83% | -16.42% | -18.02% | 214% | 42347% |
| Class A2 | -8.57% | -6.81% | -17.64% | -13.98% | 164% | 33884% |
| Class B | -8.87% | -6.84% | -16.51% | -19.20% | 159% | 32154% |
| Class C | -9.56% | -7.45% | -16.50% | -19.43% | 135% | 21850% |
| Class E | -11.76% | -9.85% | -18.32% | -19.35% | 162% | 37005% |
| Overall | -9.59% | -7.47% | -16.98% | -18.21% | 163% | 31904% |
| Class D | -9.11% | -7.14% | -15.72% | -19.30% | 128% | 18979% |
| Class F | -7.52% | -5.38% | -16.37% | -15.47% | 135% | 28555% |

Table 3. Performance under RA configuration (SADL floating-point)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |  |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -11.85% | -9.69% | -16.66% | -18.65% | 269% | 79131% |
| Class A2 | -12.22% | -10.69% | -20.16% | -15.26% | 247% | 74246% |
| Class B | -12.44% | -9.80% | -21.91% | -22.58% | 283% | 82965% |
| Class C | -13.35% | -10.93% | -22.77% | -22.97% | 239% | 78316% |
| Class E |  |  |  |  |  |  |
| Overall | -12.52% | -10.26% | -20.74% | -20.43% | 261% | 79152% |
| Class D | -15.01% | -12.49% | -23.99% | -25.90% | 230% | 72468% |
| Class F | -8.12% | -6.11% | -15.73% | -15.18% | 408% | 42047% |

Table 4. Performance under AI configuration (SADL floating-point)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over NNVC-2.0 |  |  |  |  |  |
|  | YUV | Y | U | V | EncT | DecT |
| Class A1 | -9.76% | -6.91% | -16.80% | -18.10% | 251% | 59637% |
| Class A2 | -8.74% | -6.99% | -17.83% | -13.95% | 178% | 48959% |
| Class B | -9.17% | -7.23% | -16.37% | -19.24% | 174% | 46621% |
| Class C | -10.08% | -8.11% | -16.50% | -19.44% | 148% | 30950% |
| Class E | -12.36% | -10.52% | -18.55% | -19.92% | 184% | 51307% |
| Overall | -9.93% | -7.88% | -17.08% | -18.32% | 181% | 45430% |
| Class D | -9.72% | -7.90% | -15.72% | -19.30% | 146% | 29036% |
| Class F | -8.25% | -6.29% | -16.45% | -15.57% | 140% | 39434% |

References

- S. Eadie, M. Coban, M. Karczewicz, EE1-1.9: Reduced complexity CNN-based in-loop filtering, JVET-AC0155, by teleconference, January 2023.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AB2016, Mainz, DE, October 2022.