| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 |  |  | Document: JVET-AC0270_v3 |  |
| --- | --- | --- | --- | --- |
| Title: | Crosscheck of EE1-3.2 (JVET-AC0116 : neural network-based intra prediction with learned mapping to VVC intra prediction modes) |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Crosscheck |  |  |  |
| Author(s) or Contact(s): | Mitra Damghanian Jacob Ström | Tel: Email: |  | {mitra.damghanian, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |  |

_____________________________

# Abstract

This contribution reports on the training and inference crosscheck of EE1-3.2 JVET-AC0116, proposed at the last meeting as JVET-AB0149. The neural network-based intra prediction mode comprises 7 different fully connected neural networks. The training procedure of the proponents consists of four cycles of data generation and training. In this crosscheck, this has been broken into three steps, each with an evaluation point at the end. At each evaluation point, the performance was measured using both the proponent’s models and the models trained from the crosscheck. The first step started from a random network initialization and the two other steps were trained starting from existing networks provided by the proponent. All the training steps were done using training data extracted using the supplied software. The three evaluation points resulted in the following BDR figures:

Evaluation point 1 proponent’s models:		AI BDR -4.09% (Y) -3.55% (U) -3.74% (V)  
Evaluation point 1 crosschecker’s models:	AI BDR -4.00% (Y) -3.47% (U) -3.60% (V)

For evaluation point 2, there are only partial results (BCDE, not A). Calculating the average of BCE we have:

Evaluation point 2 proponent’s models:	AI BDR -3.46% (Y) -2.94% (U) -2.76% (V)

Evaluation point 2 crosschecker’s models: 	AI BDR -3.42% (Y) -2.87% (U) -2.66% (V)

At evaluation point 2, one of the crosschecker’s models was trained only 6M steps instead of 6.4M.

Evaluation point 3 proponent’s models:		AI BDR -3.65% (Y) -3.21% (U) -3.29% (V)
Evaluation point 3 crosschecker’s models:	AI BDR -3.64% (Y) -3.20% (U) -3.29% (V)

# Introduction

The training process consists of repeated data generation steps followed by trainings, as shown in the Figure 1 below.

Figure 1 Initial chain of the training stages

The proponents have been extremely helpful in helping us sort out all the many snags and difficulties that we have encountered in the process. Still, this has taken longer than expected, and therefore we decided to move from a pure Method 1 crosscheck to a mix of Method 1 and Method 3.

In the Figure 2 below, you can see that we are using Method 1 to evaluate the performance at Evaluation point 1. However, cycle 2 and 3 both start with models from the proponent and are therefore of the Method 3 kind.

Figure 2 Timings for each part of data extraction and network training chain for the 7 networks are illustrated in bubbles in the top right corner of each process.

The code was taken from https://vcgit.hhi.fraunhofer.de/jvet-ab-ee1/VVCSoftware_VTM/, branch EE1-3.2. Table 1 shows how the different steps were initialized and trained.

Table 1 Method for this crosscheck

|  | Network Initialization | Intermediate network | Network used for generating intermediate training data | Reporting |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Evaluation of trained final network vs. final network provided by proponent | Evaluation of trained intermediate network vs. intermediate network provided by proponent |
| Step 1 | Random | Trained by crosschecker | Trained by crosschecker | Is performed in the crosscheck |  |
| Step 2 Step 3 | Existing network | Trained by crosschecker and also provided by proponent | Provided by proponent | Is performed in the crosscheck | Is performed in the crosscheck |

Each step is finished with an evaluation step. Each evaluation is done for the AI configuration with neural-network loop filters (NNLFs) turned off, and compared to the NNVC3.0 anchor with the NNLF off (which is equivalent to NNVC2.0 with NNLF off).

Full results are in the results section, but the short summary is the following:

Evaluation point 1 proponent’s models:		AI BDR -4.09% (Y) -3.55% (U) -3.74% (V)  
Evaluation point 1 crosschecker’s models:	AI BDR -4.00% (Y) -3.47% (U) -3.60% (V)

Here, the difference of +0.09% in AI is likely attributable to the fact that one of the training sets (UVG) was excluded from the training. This data set, being high resolution, was treated differently than the other training sets in the data generation software, and caused problems. It was agreed with the proponents that this training set would be excluded in order to be able to finish training in time. Given that the UVG training set is high resolution and contains quite a few frames per sequence (300-600) it is reasonable that it can affect the performance even though the number of sequences in the UVG data set is rather small. It is therefore the crosschecker’s opinion that the small difference has a valid explanation.

Evaluation point 2 proponent’s models:		AI BDR -3.30% (Y) -2.79% (U) -2.84% (V)
Evaluation point 2 crosschecker’s models:	AI BDR -x.xx% (Y) -x.xx% (U) -x.xx% (V)

At first it may be surprising that the performance goes down from Evaluation point 1, but this is due to the fact that cycle 2 comprises a complexity reduction step which lowers the complexity at the cost of BDR.

Evaluation point 3 proponent’s models:		AI BDR -3.65% (Y) -3.21% (U) -3.29% (V)
Evaluation point 3 crosschecker’s models:	AI BDR -3.64% (Y) -3.20% (U) -3.29% (V)

The results at evaluation point 3 are very close.

# Tests outlined in EE description

In JVET-AB2023, two tests were requested where this method is tested on top of NNLF0 and NNLF1. These results have been evaluated at evaluation point 3. Anchor is still NNVC 3.0 without NNLF.

## Test 3.2.1

From JVET-AB2023: "Running the provided modified version of NNVC -1.0, Filter-Set-0 being activated, Filter-Set-1 being deactivated, and the neural network-based intra prediction mode being activated”

Proponent’s model + NNLF0: 		AI BDR -9.66% (Y) -18.67% (U) -19.73% (V)
RA BDR -10.36% (Y) -19.56% (U) -20.07% (V)

Crosschecker’s model + NNLF0:	AI BDR -9.66% (Y) -18.62% (U) -19.72% (V)
RA BDR -10.34% (Y) -19.58% (U) -20.02% (V)

## Test 3.2.2

From JVET-AB2023: " running the provided modified version of NNVC -1.0, Filter-Set-1 being activated, Filter-Set-0 being deactivated, and the neural network-based intra prediction mode being activated”

Proponent’s model + NNLF1: 		AI BDR -10.51% (Y) -22.93% (U) -23.30% (V)
RA BDR -10.89% (Y) -21.49% (U) -21.41% (V)

Crosschecker’s model + NNLF1:	AI BDR -10.51% (Y) -22.93% (U) -23.30% (V)
RA BDR -10.88% (Y) -21.63% (U) -21.48% (V)

# Results

Decoder and encoder times are not reliable.

## Evaluation point 1

Proponent’s models, AI Results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.66% | -3.78% | -4.11% | 508% | 2798% |
| Class A2 | -3.16% | -2.91% | -3.42% | 528% | 3337% |
| Class B | -3.69% | -3.16% | -3.48% | 566% | 4157% |
| Class C | -3.68% | -3.09% | -3.42% | 463% | 4387% |
| Class E | -5.63% | -5.22% | -4.52% | 585% | 4977% |
| Overall | -4.09% | -3.55% | -3.74% | 528% | 3913% |
| Class D | -3.45% | -3.00% | -3.27% | 501% | 5094% |
| Class F | -2.67% | -2.09% | -2.17% | 333% | 2569% |

Crosschecker’s models, AI Results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.63% | -3.73% | -4.05% | 506% | 2671% |
| Class A2 | -3.10% | -2.84% | -3.36% | 522% | 3189% |
| Class B | -3.62% | -3.15% | -3.35% | 579% | 4164% |
| Class C | -3.54% | -2.89% | -3.18% | 502% | 4512% |
| Class E | -5.51% | -5.15% | -4.37% | 565% | 4526% |
| Overall | -4.00% | -3.47% | -3.60% | 537% | 3818% |
| Class D | -3.34% | -2.89% | -3.14% | 446% | 4458% |
| Class F | -2.55% | -1.99% | -2.12% | 320% | 2386% |

## Evaluation point 2

Proponent’s models, AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.00% | -3.17% | -3.44% | 268% | 897% |
| Class A2 | -2.48% | -2.37% | -2.74% | 301% | 1158% |
| Class B | -2.95% | -2.46% | -2.67% | 303% | 1232% |
| Class C | -2.95% | -2.29% | -2.38% | 296% | 1438% |
| Class E | -4.49% | -4.05% | -3.25% | 309% | 1367% |
| Overall | -3.30% | -2.79% | -2.84% | 296% | 1218% |
| Class D | -2.80% | -2.39% | -2.52% | 281% | 1536% |
| Class F | -2.09% | -1.44% | -1.52% | 214% | 742% |

Crosschecker’s models, at almost evaluation point 2 (one model trained with 6 M iterations, the remaining 6 with the full 6.4 M iterations in the second step of cycle 2), AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.91% | -2.42% | -2.59% | 301% | 1172% |
| Class C | -2.88% | -2.27% | -2.37% | 284% | 1381% |
| Class E | -4.48% | -3.92% | -3.03% | 303% | 1295% |
| Overall |  |  |  |  |  |
| Class D | -2.77% | -2.28% | -2.50% | 277% | 1440% |

## Evaluation point 2, first frame per sequence only

Crosschecker’s models with the proponents model as the reference. Crosschecker’s models almost at evaluation point 2 (one model trained with 6 M iterations, the remaining 6 with the full 6.4 M iterations in the second step of cycle 2), proponent’s models all trained with the full 6.4 M iterations, AI results, only first frame in each sequence:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.06% | 0.55% | 0.25% | 103% | 104% |
| Class A2 | 0.09% | 0.54% | 0.22% | 97% | 94% |
| Class B | -0.01% | 0.60% | 0.33% | 99% | 101% |
| Class C | 0.07% | 0.06% | -0.07% | 98% | 95% |
| Class E | 0.22% | 0.08% | 0.90% | 97% | 97% |
| Overall | 0.05% | 0.37% | 0.30% | 99% | 98% |
| Class D | -0.04% | -0.68% | -0.30% | 99% | 96% |
| Class F | -0.04% | -0.18% | 0.32% | 97% | 101% |

## Evaluation point 3

Proponent’s models, AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.37% | -3.80% | -4.09% | 241% | 897% |
| Class A2 | -2.85% | -2.50% | -2.89% | 251% | 1086% |
| Class B | -3.26% | -2.78% | -2.97% | 254% | 1174% |
| Class C | -3.21% | -2.83% | -2.99% | 250% | 1478% |
| Class E | -4.95% | -4.56% | -3.85% | 262% | 1400% |
| Overall | -3.65% | -3.21% | -3.29% | 252% | 1201% |
| Class D | -3.00% | -2.53% | -2.93% | 228% | 1503% |
| Class F | -2.22% | -1.63% | -1.80% | 192% | 738% |

Crosschecker’s models, AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -4.36% | -3.73% | -4.12% | 239% | 853% |
| Class A2 | -2.86% | -2.53% | -2.86% | 248% | 1079% |
| Class B | -3.25% | -2.75% | -2.97% | 253% | 1161% |
| Class C | -3.21% | -2.82% | -2.94% | 248% | 1412% |
| Class E | -4.94% | -4.58% | -3.88% | 257% | 1350% |
| Overall | -3.64% | -3.20% | -3.29% | 249% | 1167% |
| Class D | -3.01% | -2.52% | -2.83% | 227% | 1415% |
| Class F | -2.20% | -1.73% | -1.84% | 189% | 721% |

## Test 3.2.1

Proponent’s models plus NNLF0 over NNVC3.0 without NNLF:

RA results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -11.12% | -15.23% | -19.81% | 227% | 82255% |
| Class A2 | -10.66% | -20.33% | -15.57% | 208% | 81023% |
| Class B | -10.04% | -19.99% | -21.61% | 232% | 84315% |
| Class C | -9.98% | -21.69% | -21.71% | 197% | 71454% |
| Class E |  |  |  |  |  |
| Overall | -10.36% | -19.56% | -20.07% | 216% | 79640% |
| Class D | -10.86% | -21.19% | -22.07% | 189% | 63141% |
| Class F | -4.78% | -12.86% | -11.89% | 288% | 36369% |

AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -9.91% | -17.84% | -20.73% | 436% | 64541% |
| Class A2 | -8.32% | -18.45% | -15.60% | 356% | 52884% |
| Class B | -8.77% | -18.30% | -20.32% | 343% | 50924% |
| Class C | -9.12% | -19.26% | -20.94% | 304% | 38139% |
| Class E | -12.92% | -19.54% | -20.27% | 359% | 61400% |
| Overall | -9.66% | -18.67% | -19.73% | 352% | 51576% |
| Class D | -8.94% | -18.13% | -21.16% | 271% | 33886% |
| Class F | -6.03% | -13.50% | -13.52% | 236% | 43171% |

Crosschecker’s models plus NNLF0 over NNVC3.0 without NNLF:

RA results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -11.11% | -15.16% | -19.79% | 231% | 86610% |
| Class A2 | -10.63% | -20.31% | -15.51% | 214% | 81745% |
| Class B | -10.04% | -19.95% | -21.56% | 231% | 85438% |
| Class C | -9.93% | -21.86% | -21.63% | 201% | 74581% |
| Class E |  |  |  |  |  |
| Overall | -10.34% | -19.58% | -20.02% | 219% | 81895% |
| Class D | -10.86% | -21.34% | -22.22% | 193% | 65613% |
| Class F | -4.78% | -12.83% | -11.77% | 289% | 35618% |

AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -9.91% | -17.78% | -20.71% | 443% | 64446% |
| Class A2 | -8.32% | -18.44% | -15.58% | 361% | 53404% |
| Class B | -8.78% | -18.23% | -20.36% | 347% | 50853% |
| Class C | -9.14% | -19.19% | -20.91% | 307% | 38508% |
| Class E | -12.91% | -19.52% | -20.19% | 368% | 62130% |
| Overall | -9.66% | -18.62% | -19.72% | 357% | 51840% |
| Class D | -8.94% | -18.27% | -21.07% | 275% | 34137% |
| Class F | -6.02% | -13.55% | -13.54% | 237% | 43394% |

## Test 3.2.2

Proponent’s models plus NNLF1 over NNVC3.0 without NNLF:

RA results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -10.66% | -17.15% | -20.09% | 312% | 70430% |
| Class A2 | -11.32% | -21.00% | -18.10% | 294% | 69922% |
| Class B | -10.48% | -23.65% | -22.54% | 304% | 73347% |
| Class C | -11.25% | -22.43% | -23.46% | 261% | 68029% |
| Class E |  |  |  |  |  |
| Overall | -10.89% | -21.49% | -21.41% | 291% | 70630% |
| Class D | -12.60% | -24.62% | -24.88% | 246% | 62671% |
| Class F | -5.94% | -12.57% | -11.42% | 409% | 29064% |

AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -10.19% | -21.39% | -23.17% | 394% | 47798% |
| Class A2 | -9.30% | -22.45% | -19.43% | 341% | 39515% |
| Class B | -9.52% | -24.40% | -24.13% | 322% | 37484% |
| Class C | -10.22% | -21.36% | -24.48% | 291% | 28158% |
| Class E | -14.08% | -24.62% | -24.33% | 352% | 46489% |
| Overall | -10.51% | -22.93% | -23.30% | 334% | 38303% |
| Class D | -9.95% | -21.45% | -23.53% | 261% | 24313% |
| Class F | -7.21% | -13.47% | -12.91% | 222% | 30689% |

Crosschecker’s models plus NNLF1 over NNVC3.0 without NNLF:

RA results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -10.66% | -17.33% | -20.06% | 313% | 70452% |
| Class A2 | -11.33% | -21.02% | -18.23% | 297% | 70075% |
| Class B | -10.46% | -23.75% | -22.72% | 306% | 72757% |
| Class C | -11.22% | -22.68% | -23.44% | 253% | 64934% |
| Class E |  |  |  |  |  |
| Overall | -10.88% | -21.63% | -21.48% | 290% | 69605% |
| Class D | -12.58% | -24.32% | -24.89% | 243% | 59403% |
| Class F | -5.97% | -12.41% | -11.38% | 412% | 29776% |

AI results:

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -10.19% | -21.27% | -23.21% | 405% | 49585% |
| Class A2 | -9.30% | -22.41% | -19.35% | 335% | 39210% |
| Class B | -9.51% | -24.47% | -24.17% | 329% | 38718% |
| Class C | -10.23% | -21.31% | -24.55% | 291% | 27913% |
| Class E | -14.07% | -24.69% | -24.20% | 347% | 45435% |
| Overall | -10.51% | -22.93% | -23.30% | 335% | 38613% |
| Class D | -9.93% | -21.36% | -23.61% | 264% | 24391% |
| Class F | -7.22% | -13.55% | -12.90% | 216% | 29536% |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).