| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 29th Meeting, by teleconference, 11–20 January 2023 | Document: JVET-AC0271 |
| --- | --- |

| Title: | Crosscheck of JVET-AC0063 (EE1-1.3: On chroma order adjustment in NNLF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Renjie Chang | Email: | renjiechang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the inference crosscheck results for JVET-AC0063 (JVET-AB-EE1-1.3), which proposes a chroma order adjustment method in NNLF. In this report, two tests based on integer inference were crosschecked including test the proposed method implemented on top of Filter Set #0 and test the proposed method implemented on top of Filter Set #1. The proposed method in JVET-AC0063 is only used for B slices, so only crosscheck tests is LDB and RA configurations are performed. It is reported that, the full crosscheck results for RA configuration and the existing partial results for LDB configuration can exactly match.

Introduction

The JVET-AB-EE1-1.3 described in JVET-AC0063 [] study a chroma order adjustment method in NNLF, where NNVC-3.0 anchor (NnlfOption=0), NNVC-3.0 (NnlfOption=1) and NNVC-3.0 (NnlfOption=2) are used as anchors for comparison. In this crosscheck report, the performance of proposed method in [1] was tested compared to above three anchors.

Results

All the experiments are performed following the common test conditions in []. The inference framework is SADL with integer precision. The crosscheck results are shown in the following tables.

Table 1. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.08% | -3.19% | -0.71% | 105% | 99% |
| Class A2 | -0.02% | -1.14% | -0.38% | 104% | 98% |
| Class B | -0.02% | -1.23% | -0.57% | 105% | 99% |
| Class C | -0.04% | -0.84% | -0.80% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -1.50% | -0.62% | 104% | 99% |
| Class D | -0.03% | -0.11% | -1.74% | 103% | 99% |
| Class F | -0.05% | -0.54% | -0.98% | 106% | 109% |

Table 2. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.09% | -2.38% | -0.78% | 104% | 100% |
| Class C | -0.19% | -2.53% | -1.90% | 103% | 99% |
| Class E | -0.10% | -1.58% | -1.04% | 109% | 103% |
| Overall | -0.13% | -2.45% | -1.28% | 105% | 100% |
| Class D | -0.04% | -2.12% | -4.30% | 103% | 99% |
| Class F | -0.20% | -1.37% | -3.12% | 107% | 108% |

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.19% | -15.84% | -19.46% | 166% | 59350% |
| Class A2 | -9.52% | -20.37% | -15.04% | 160% | 56352% |
| Class B | -8.60% | -20.04% | -21.20% | 162% | 58281% |
| Class C | -8.66% | -21.62% | -21.38% | 139% | 47086% |
| Class E |  |  |  |  |  |
| Overall | -8.92% | -19.69% | -19.67% | 156% | 54888% |
| Class D | -9.77% | -20.09% | -22.44% | 138% | 41589% |
| Class F | -3.84% | -12.63% | -11.93% | 201% | 25108% |

Table 4. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -7.87% | -20.15% | -20.93% | 157% | 54864% |
| Class C | -8.43% | -22.17% | -21.96% | 137% | 45101% |
| Class E | -8.99% | -16.97% | -17.52% | 249% | 50915% |
| Overall | -8.34% | -21.05% | -21.39% | 168% | 50444% |
| Class D | -9.65% | -20.55% | -24.01% | 136% | 40817% |
| Class F | -4.38% | -11.85% | -11.65% | 202% | 26107% |

Table 5. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.02% | -1.53% | -0.86% | 104% | 101% |
| Class A2 | 0.00% | -0.80% | -0.20% | 103% | 101% |
| Class B | 0.00% | -1.02% | -0.62% | 103% | 102% |
| Class C | -0.01% | -0.63% | -0.94% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.97% | -0.67% | 103% | 101% |
| Class D | 0.03% | -0.70% | -0.70% | 102% | 101% |
| Class F | 0.02% | -0.36% | -1.00% | 104% | 104% |

Table 6. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=2) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.05% | -2.73% | -3.99% | 102% | 101% |
| Class E | 0.08% | -1.72% | -2.24% | 105% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -1.90% | -1.92% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 7. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.90% | -17.05% | -19.41% | 198% | 39878% |
| Class A2 | -10.24% | -20.82% | -17.58% | 189% | 38509% |
| Class B | -9.05% | -23.75% | -22.13% | 193% | 39350% |
| Class C | -9.92% | -22.26% | -23.01% | 164% | 34599% |
| Class E |  |  |  |  |  |
| Overall | -9.49% | -21.43% | -20.91% | 185% | 37960% |
| Class D | -11.45% | -24.14% | -24.46% | 158% | 31860% |
| Class F | -5.00% | -12.07% | -11.48% | 252% | 16131% |

Table 8. Performance of the proposed method (LDB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC3.0 (NnlfOption=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -9.19% | -17.65% | -18.04% | 163% | 33715% |
| Class E | -8.72% | -18.55% | -19.04% | 322% | 30149% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.82% | -20.99% | -20.19% | 155% | 30215% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Reference

- Z. Dai, Y. Yu, H. Yu, D. Wang. EE1-1.3: On chroma order adjustment in NNLF. JVET-AC0063, 29th Meeting, by teleconference, 11–20 January 2023

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AB2016, 28th Meeting, Mainz, DE, 20–28 October 2022.